| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | ARQ feedback IE modification (section 15.2.13.1.1) |  |
| Date Submitted | 2009-11-18 |  |
| Source(s) | Youngbin Chang, Hyeunmok Jung, Anil Agiwal, Rakesh Taori Samsung Electronics Aran Bergman Intel Corporation | yb.chang@samsung.com, anilag@samsung.com aran.bergman@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0057, IEEE 802.16 Working Group Letter Ballot Recirc #30a / Topic: ARQ (Section 15.2.13) |  |
| Abstract | This contribution proposes ARQ feedback IE modification |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

ARQ feedback IE modification

Youngbin Chang, Hyeunmok Jung, Anil Agiwal, Rakesh Taori

Samsung Electronics

Aran Bergman

Intel Corporation

# Introduction

In current P802.16m/D2, two ARQ feedback IEs are defined. One is for ARQ block, another is for ARQ sub-block. In this contribution, we propose to merge these ARQ feedback IEs into one ARQ feedback IE format and SSN is defined as 8bits.

## Cumulative ACK

In current ARQ feedback for ARQ feedback IE, cumulative ACK is 3 byte (2byte + 1bit). In most case, ARQ feedback is transmitted as cumulative ACK because ARQ block error is only happen when HARQ residual error occurred. If 1 bit is removed in the cumulative ACK case, it is a meaningful overhead savaging.

## Merging two ARQ feedback IEs

When receiver indicates error of ARQ block and ARQ sub-block, some information fields are duplicated (TYPE, LAST, Flow ID, SN, FLAG). To remove this overhead, two ARQ feedback IEs are merged into one ARQ feedback IE.

## SSN bits

To support 8byte LRU size, minimum payload size can be 2 byte (GMH(2byte)+FPEH(2byte)+payload(2byte) +CRC(2byte), Therefore SSN should be 10bits (GMH length = 11bits)

# Proposed solution

1. In cumulative ACK case, “TYPE” field is removed from ARQ feedback IE, which saves 1 byte overhead in cumulative ACK case. In cumulative ACK case, it is not necessary to indicate ARQ sub-block errors.

2. In selective ACK case, if there are ARQ sub-blocks to indicate error, “Ext” bit is introduced on behalf of “TYPE” field, which is not used in cumulative ACK case. “Ext” means that “Extension for indication ARQ sub-blocks error”.

3. In selective ACK case, to reduce overhead, some fields (TYPE, LAST, Flow ID, SN, FLAG) are removed from ARQ feedback IE for ARQ sub-block.

4. SEM(Sub-block Existence Map) field added on behalf of “SN” in ARQ feedback IE for ARQ-sub-block.

Figure 1. ARQ feedback IE for both ARQ block and ARQ sub-block

Table 1. ARQ feedback IE format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| ARQ_feedback_IE(LAST){ | variable |  |
| Last | 1 | 0 = More ARQ Feedback IE in the list 1 = Last ARQ Feedback IE in the list |
| Flow ID | 4 | The ID of the flow being referenced |
| FLAG1 | 1 | 0 = Cumulative ACK 1 = Selective ACK MAP existence |
| SN | 10 | FLAG1 = 0, ARQ block up to and including SN has been received successfully. FLAG1 = 1, ARQ block less than SN has been received successfully. |
| if(FLAG1==1){ |  |  |
| Selective ACK MAP1 | 6 | Each bit represents ACK or NAK of corresponding ARQ block. '0' is NAK and '1' is ACK. First MSB of first Selective ACK MAP1 represents ACK or NAK information of SN. Contiguous bits after first MSB of first ACK MAP are corresponding to contiguous SN. |
| ExT | 1 | Extension for ARQ sub-block indication 0= No ARQ sub-block ACK/NACK indication 1= ARQ sub-block ACK/NACK indication follows |
| FLAG1 | 1 | 0 = No more selective ACK MAP1 and FLAG1 1 = Another set of selective ACK MAP1 and FLAG1 follows |
| while(FLAG1==1){ |  |  |
| Selective ACK MAP1 | 7 | Each bit represents ACK or NAK of corresponding ARQ block. '0' is NAK and '1' is ACK. Selective ACK MAP1 represents ACK or NAK information of ARQ blocks after first selective ACK MAP1. |
| FLAG1 | 1 | 0 = No more selective ACK MAP1 and FLAG1 1 = Another set of selective ACK MAP1 and FLAG1 follows |
| } |  |  |
| if(ExT==1){ |  |  |
| SEM | variable | Sub-block-Existence Map The numbers of SEM bits are same as NACKed ARQ blocks in the Selective ACK MAP1.Each bit represents the existence of partially nacked ARQ blocks or completely nacked ARQ blocks. 1 = Partially nacked ARQ block (the ARQ block has some sub-blocks which have received correctly) 0 = Completely nacked ARQ blocks( the ARQ block has no sub-blocks which have received correctly) |
| for(i=0;i<Num_Sub-block<i++){ |  | Num_Sub-block is the number of bits equal to 1 in the SEM field |
| FLAG2 | 1 | 0 = Cumulative ACK 1 = Selective ACK MAP existence |
| SSN | 8 | FLAG2 = 0, ARQ sub-block up to and including SSN has been received successfully. FLAG2 = 1, ARQ sub-block less than SSN has been received successfully. |
| while(FLAG2==1){ |  |  |
| Selective ACK MAP2 | 7 | Each bit represents ACK or NAK of corresponding ARQ sub-block '0' is NAK and '1' is ACK. First MSB of first Selective ACK MAP2 represents ACK or NAK information of SSN. Contiguous bits after first MSB of first selective ACK MAP2 are corresponding to contiguous SSN. |
| FLAG2 | 1 | 0 = No more selective ACK MAP2 and FLAG2 1 = Another set of selective ACK MAP2 and FLAG2 follows |
| } |  |  |
| } |  |  |
| Padding | variable | For Byte alignment |
| } |  |  |
| } |  |  |
| } |  |  |

# Overhead saving

1) In cumulative ACK case, it saves 1byte (50% saving).

2) If selective ACK MAP exists and there is no ARQ sub-blocks, overhead saving is 1 byte due to removal of ‘TYPE’ field.

3) If selective ACK MAP exists and there are ARQ sub-blocks to indicate, overhead saving calculated as:

- Remove these fields in ARQ feedback IE for ARQ sub-blocks: ‘TYPE(1bit)’, ‘LAST(1bit)’, flow ID(4bit), Num_SN(4bits), and ‘SN(variable, minimum = 10bits and maximum is 160bits when Num_SN=16)

- Add these fields: ‘ExT(1bit), SEM(variable, minimum = 1 bit and maximum = 16bits). The overhead saving depends on Number of ARQ blocks which have partially received ARQ sub-blocks.

- Minimum overhead saving is 18 bits(3 bytes), Maximum saving is 153bits(20bytes)

- If Num_SN are more than 16, then it needs another ARQ feedback IE for ARQ sub-blocks which is huge overhead compared with the modified ARQ feedback IE.

# Text Proposal

[Modify the text and tablet in section 15.2.3.37 on page 89 as follows:]

15.2.3.37 AAI_ARQ-Feedback message

An AAI_ARQ-Feedback message is used for receiver to inform the reception status of a number of ARQ blocks or ARQ sub-blocks. ARQ feedback IE or ARQ feedback IE2 (see Table 740 and Table 741) is included in the AAI_ARQ-Feedback message.

Table 712—AAI_ARQ-Feedback message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_ARQ-Feedback_Message_Format() { |  |  |
| Mangement Message type = xx | 8 |  |
| ARQ_feedback_IE or ARQ_feedback_IE2 | variable | See table 740 and 741 |
| } |  |  |

[Modify the text from 43-47 on page 188 as follows:]

15.2.13.1.1 ARQ feedback IE

Table 740 defines the ARQ Feedback IE for ARQ block. Table 741 defines the ARQ feedback IE for ARQ sub-blocks. TheyIt areis used by the receiver to signal positive or negative acknowledgments for ARQ block and sub-blocks. ARQ feedback IEs in Table 740 and Table 741 may be transported either as a part of extended header ("piggybacked") within a MAC PDU or a standalone MAC control message.

[[Delete the table 741 on page 189 and replace the table 740 on page 188 as follows:]

Table 740 – ARQ feedback IE format

| Syntax | Size(bit) | Notes |
| --- | --- | --- |
| ARQ_feedback_IE(LAST){ | variable |  |
| Last | 1 | 0 = More ARQ Feedback IE in the list 1 = Last ARQ Feedback IE in the list |
| Flow ID | 4 | The ID of the flow being referenced |
| FLAG1 | 1 | 0 = Cumulative ACK 1 = Selective ACK MAP existence |
| SN | 10 | FLAG1 = 0, ARQ block up to and including SN has been received successfully. FLAG1 = 1, ARQ block less than SN has been received successfully. |
| if(FLAG1==1){ |  |  |
| Selective ACK MAP1 | 6 | Each bit represents ACK or NAK of corresponding ARQ block. '0' is NAK and '1' is ACK. First MSB of first Selective ACK MAP1 represents ACK or NAK information of SN. Contiguous bits after first MSB of first ACK MAP are corresponding to contiguous SN. |
| EXT | 1 | Extension for ARQ sub-block indication 0= No ARQ sub-block ACK/NACK indication 1= ARQ sub-block ACK/NACK indication follows |
| FLAG1 | 1 | 0 = No more selective ACK MAP1 and FLAG1 1 = Another set of selective ACK MAP1 and FLAG1 follows |
| while(FLAG1==1){ |  |  |
| Selective ACK MAP1 | 7 | Each bit represents ACK or NAK of corresponding ARQ block. '0' is NAK and '1' is ACK. Selective ACK MAP1 represents ACK or NAK information of ARQ blocks after first selective ACK MAP1. |
| FLAG1 | 1 | 0 = No more selective ACK MAP1 and FLAG1 1 = Another set of selective ACK MAP1 and FLAG1 follows |
| } |  |  |
| if(EXT==1){ |  |  |
| SEM | variable | Sub-block-Existence Map The numbers of SEM bits are same as NACKed ARQ blocks in the Selective ACK MAP1.Each bit represents the existence of partially nacked ARQ blocks or completely nacked ARQ blocks. 1 = Partially nacked ARQ block (the ARQ block has some sub-blocks which have received correctly) 0 = Completely nacked ARQ blocks( the ARQ block has no sub-blocks which have received correctly) |
| For(i=0;Num_SN;i++) { |  |  |
| do{ |  | All ARQ sub-blocks that are not listed the Start_SSN and Num_SSN in ‘do loop’ are considered as NACK. |
| START_SSN | 11 | Start of ARQ sub-block SN which was received correctly. |
| Num_SSN | 11 | Number of consecutive ARQ sub-blocks which were received correctly from START_SSN onwards |
| End | 1 | 0 = One more set of START_SSN, Length and End fields 1 = There are no more START_SSN, NUM_SSN and End fields |
| }while(End ==0) |  |  |
| } |  |  |
| Padding | variable | For Byte alignment |
| } |  |  |
| } |  |  |
| } |  |  |