| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Relay Frame structure clarification (section 15.6.3.2) |  |
| Date Submitted | 2009-11-18 |  |
| Source(s) | Youngbin Chang, Jaeweon Cho, Mihyun Lee, Hyunjeong Kang, Rakesh Taori Samsung Electronics Jinyoung Chun, Dungguk Lim, and Jin Sam Kwak LG Electronics Alexander Maltsev, Alexey Khoryaev, Alexei Davydov, Andrey Chervyakov Intel Corporation | yb.chang@samsung.com, anilag@samsung.com jychun03@lge.com alexander.maltsev@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0057, IEEE 802.16 Working Group Letter Ballot Recirc #30a / Topic: Relay (Section 15.6) |  |
| Abstract | This contribution proposes Relay Frame structure clarification text |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D3. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Relay Frame structure clarification

Youngbin Chang, Jaeweon Cho, Mihyun Lee, Hyunjeong Kang, Rakesh Taori

Samsung Electronics

Jinyoung Chun, Dungguk Lim and Jin Sam Kwak

LG Electronics

Alexander Maltsev, Alexey Khoryaev, Alexei Davydov, Andrey Chervyakov

Intel Corporation

# Introduction

In Relay section, Gap definition need to be clarified. The contribution proposes the text regarding gap definition.

# Proposed text

[Replace from line 43, page 573 to line 53, page 575 in the section 15.6.3.2 as following text:]

---------------------------------------------- Text Start ----------------------------------------------

15.6.3.2.1 FDD frame structure

The FDD frame shall be constructed on the basis of the basic frame structure defined in 15.3.3.

An ARS supporting FDD mode shall communicate with the ABS using full duplex FDD mode and it shall be able to simultaneously support half duplex and full duplex AMSs operating on the same RF carrier.

An ARS in FDD systems shall use DL carrier frequency (FDL) for receiving from the ABS in the 16m DL Receive zone and shall use UL carrier frequency (FUL) for transmission to the ABS in the 16m UL Transmit zone.

In ARS radio frame, the ARS idle state time interval (R_IdleTime) shall be inserted between two ARS frames. The duration of the R_IdleTime is signaled by the ARS to its AMSs through the AAI_System Configuration Descriptor. In ARS DL frame, R_IdleTime shall be same as IdleTime of ABS. In ARS UL frame, the duration of R_IdleTime shall be less or equal to the duration of IdleTime of ABS. The ARS UL frame may be time-advanced for a Tadv interval referring to the start of the ABS UL frame. The duration of Tadv is calculated according to the following equation:

Tadv= IdleTime−R_IdleTime

When an ARS switches transceiver states from transmit to receive or from receive to transmit, a DL ARS radio frame in FDD system may have a R-TTI between 16m Access and 16m Relay zone and a R-RTI between 16m Relay zone and 16m Access zone in the next DL ARS frame. The location of the R-TTI is the last OFDM symbol of the last AAI subframe of 16m Access zone and the location of the R-RTI is the last OFDM symbol of the last AAI subframe of 16m Relay zone. The duration of R-TTI and R-RTI in DL shall be calculated by following equations:

R-TTI =0 if RTD2≥ARSTTGTs if RTD2<ARSTTG

R-RTI = 0 if IdleTime−RTD2≥ARSRTGTs if IdleTime−RTD2<ARSRTG

Where the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in Table 762.

When an ARS switches transceiver states from receive to transmit or from transmit to receive, an UL ARS radio frame in FDD system may have a R-RTI between 16m Access and 16m Relay zone and a R-TTI between 16m Relay zone and 16m Access zone in the next UL ARS frame. The location of the R-RTI is the first OFDM symbol of the first AAI subframe of 16m Relay zone and the location of R-TTI is the last OFDM symbol of the last sub-frame of 16m Relay zone. The duration of R-RTI and R-TTI in UL shall be calculated by following equations:

R−RTI=0 if Tadv−RTD2≥ARSRTGTs if Tadv−RTD2<ARSRTG

R−TTI=0 if RIdleTime+RTD2≥ARSTTGTs if RIdleTime+RTD2<ARSTTG

Where the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in Table 762.

Figure 578 illustrates an example frame structure with ARS support for FDD mode, which is applicable to the nominal channel bandwidth of 5, 10, 20 MHz with G = 1/8.

Figure 578—Example of ARS FDD frame structure with G=1/8 in 5/10/20MHz

15.6.3.2.2 TDD frame structure

The TDD frame shall be constructed on the basis of the basic frame structure defined in 15.3.3.

In ARS radio frame, the ARS idle state time interval (R_IdleTime) shall be inserted before the switching point from DL to UL. The duration of the R_IdleTime is signaled by the ARS to its AMSs through the AAI_System Configuration Descriptor. The ARS UL frame may be advanced (Tadv) referring to the start of ABS UL frame. The duration of Tadv is calculated according to the following equation:

Tadv=TTG−R_IdleTime

The duration of R_IdleTime shall be less or equal to the TTG. In each ARS radio frame, the RTG interval shall be inserted before the switching point from UL to DL.

An ARS radio frame in TDD system shall have a R-TTI transition interval in DL between 16m Access and 16m Relay zones. The duration of the R-TTI shall be calculated by following equation:

R-TTI =0 if RTD2≥ARSTTGTs if RTD2<ARSTTG

where the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in Table 762.

An ARS radio frame in TDD system shall have a R-RTI transition interval in UL between 16m Access and 16m Relay zones. The duration of the R-RTI shall be calculated by the following equation:

R-RTI=0 if Tadv−RTD2≥ARSRTGTs if Tadv−RTD2<ARSRTG

where the RTD is the round trip delay between the ARS and its superordinate station and Ts is defined in Table 762.

Figure 579 illustrates an example frame structure with ARS support for TDD mode D:U = 5:3, which is applicable to the nominal channel bandwidth of 5, 10, 20 MHz with G = 1/8. The number of AAI subframes allocated to the Relay zone in DL direction is two and in UL direction is one. The duration of R-TTI and R-RTI is equal to the duration of one OFDMA symbol.

Figure 579. Example of ARS TDD frame structure with G=1/8 in 5/10/20MHz

---------------------------------------------- Text End ----------------------------------------------