| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Interference measurements with Relays (15.3.6.5.2.11) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Alexander Maltsev Alexei Davydov Andrey Pudeyev Vadim Sergeyev Jerry Sydir Alexey Khoryaev Intel Corporation | alexander.maltsev@intel.com jerry.sydir@intel.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Comments on IEEE P802.16m/D2 for IEEE 802.16 Working Group Letter Ballot Recirc #30a |  |
| Abstract | This contribution proposes modifications to the P802.16m/D2 text that allow AMSs to perform separate CQI measurements in Access and Relay zone of the frame for system supporting relay stations. |  |
| Purpose | To be discussed and adopted by TGm for P802.16m/D3 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Interference measurements with Relays

Alexander Maltsev, Alexei Davydov, Andrey Pudeyev, Vadim Sergeyev, Jerry Sydir, Alexey Khoryaev

Intel Corporation

# Motivation

This contribution proposes modifications to the P802.16m/D2 [1] text that allow AMSs to perform separate measurements of CQI in Access and Relay zone of the frame.

Operation of the system with Relay stations in accordance with the Relay frame format defined in [2] creates different interference levels in the Access and the Relay zones of frames. Therefore to perform accurate CQI measurements AMSs should treat the Access and Relay zones separately. Separate CQI reports for Access and Relay zones may also be used by the ABS scheduler to increase the cell throughput through adaptive scheduling of AMSs to Access or Relay zones. However, currently specified messages requesting CQI repots do not incorporate indication of the zone where the CQI should be measured.

Figure 1. Example of DL interference environment seen by an AMS in the Access and the Relay zones.

To remove this limitation we propose to introduce Measurement Zone bit in the Feedback Allocation A-MAP IE to indicate in which zone (Access or Rely) the CQI should be measured. An AMS reading the Measurement Zone bit should measure the CQI only in the zone indicated by the bit. If periodic feedback is requested, the AMS should use the zone indicated by the Measurement Zone bit for generating each feedback report in the series.

If the ABS further wants the AMS to send feedback for another zone it should send another Feedback Allocation A-MAP IE with the Measurement Zone bit set or cleared in accordance with the desired zone.

# References

- IEEE P802.16m/D2, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- IEEE 802.16m-09/0034, “IEEE 802.16m System Description Document”

-------------------------------  Text Start  ---------------------------------------------------

[Modify the Table 818 in the subclause 15.3.6.5.2.11 as follows (page 392):]

Table 818—Feedback Allocation A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Feedback-Allocation-MAP_IE() { |  |  |
| A-MAP IE Type | [4] | Feedback Allocation A-MAP IE = 0b0010 |
| Channel Index | Variable | Feedback channel index within the UL fast feedback control resource region (Dependent on LFB,FPi defined in 15.3.8.3.3.2) |
| Measurement Zone | [1] | 0b0: Feedback shall be measured in the Access zone 0b1: Feedback shall be measured in the Relay zone |
| Short-term feedback period (p) | [3] | A feedback is transmitted on the FBCH every 2p frames |
| Long-term feedback Period (q) | [2] | A long-term feedback is transmitted on the FBCH every 2q short-term feedback opportunities. If q = 0b00, long-term feedback is not used. |
| Frame offset | [3] | The AMS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame is specified, the AMS should start reporting in eight frames. |
| Allocation Duration(d) | [3] | An FBCH is transmitted on the FBCH channels indexed by Channel Index for 8×2d frames. If d = 0b000, the FBCH is deallocated. If d = 0b111, the AMS should report until the ABS command for the AMS to stop. |
| ACK allocation flag | [1] | Indicated if one ACK channel is allocated |
| If ( ACK allocation flag == 0b1 ) { |  |  |
| HFA | [3] | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } |  |  |
| MaxMt | Variable [1-2] | Variable number of bits - depends on number of transmit antenna Nt If Nt=2: (SU-MIMO and MU-MIMO) 0b0: 1 0b1: 2 If Nt=4: (SU-MIMO and MU-MIMO) 0b00: 1 0b01: 2 0b10: 3 0b11: 4 If Nt=8 (SU-MIMO) 0b000: 1 0b001: 2 0b011: 4 0b111: 8 If Nt=8: (MU-MIMO) 0b00: 1 0b01: 2 0b10: 3 0b11: 4 |
| MFM | [3] | MIMO Feedback Mode as defined in Table 671 |
| If (MFM = 2,3,5,6) { |  |  |
| Feedback Format | [2] |  |
| } |  |  |
| If (MFM = 0,1,4,7) { |  |  |
| FPI | [2] | Frequency partition indication: ABS indicate AMS to send wideband CQI and STC rate of the frequency partition and reuse factor in the future: 0b00: Frequency partition index 0 0b01: Frequency partition index 1 0b10: Frequency partition index 2 0b11: Frequency partition index 3 |
| } |  |  |
| If (MFM = 0,1 & long term feedback period != 0b00{ |  |  |
| Long term FPI | [2] | Frequency partition indication: ABS indicate AMS to send wideband CQI and STC rate for the second frequency partition using long term feedback: 0b00: Frequency partition index 0 0b01: Frequency partition index 1 0b10: Frequency partition index 2 0b11: Frequency partition index 3 |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | CL SU and MU MIMO |
| CM | [2] | Codebook Feedback Mode and Codebook Coordination Enable 0b00: standard with CCE disabled 0b01: transformation with CCE disabled 0b10: differential with CCE disabled 0b11: standard with CCE enabled |
| CS | [1] | Codebook subset |
| } |  |  |
| If(MFM==0,1,2,5) { |  |  |
| Measurement Method Indication | [1] | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| Padding | Variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } |  |  |

[Insert the following text after the Table 818:]

Measurement Zone: a one-bit field indicating the zone where the feedback information shall be measured. The value of 0 requests the feedback report for the Access Zone of a frame; the value of 1 requests the feedback report for the Relay Zone of a frame.

-------------------------------  Text End  ---------------------------------------------------