| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Changes to ranging code start indexing in IEEE P802.16m/D2 (15.3.9.2.4.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Xinrong Wang, Yang-Seok Choi Intel Corporation | {xinrong.wang; yang-seok.choi}@intel.com |
| Re: | IEEE 802.16-09/0057, IEEE 802.16 Working Group Letter Ballot #30a P802.16m/D2 : 15.3.9.2.4.1 |  |
| Abstract | The contribution proposes change to the ranging code start indexing in IEEE P802.16m/D2 in Section 15.3.9.2.4.1. |  |
| Purpose | To be discussed and adopted by TGm. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Change to the ranging code start indexing in IEEE P802.16m/D2 (15.3.9.2.4.1)

Xinrong Wang, Yang-Seok Choi

Intel Corporation

- 1. Introduction

This contribution provides change to the equation of ranging code start indexing which starts at line 62 on Page 486 in Section 15.3.9.2.4.1 of IEEE P802.16m/D2 document [1].

M is changed to Mns to match that in the Equations 262 and 263.

In order to utilize the root codes with large index number in Format 1, equation “rns0=4·k+1” is changed to “rns0 = 4 × kns + 1 and rns0 = 16 × kns + 1 for ranging Format 0 and Format 1, respectively”. The range of k was from 0 to 16. In order to save one bit in the signaling, the range of kns is changed to 0 to 15.

- 2. References

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems”

- 3. Text proposal

------------------------------------  Text Start  ---------------------------------------------------

Adopt the following modification in section 15.3.9.2.4.1.

The number of cyclic shifted codes per root index (MMns), the start root index of ZC code (rns0), and the ranging preamble code partition information are broadcasted by S-SFH. The number of cyclic shifted codes per root index is defined in Table 876. The start root index of ZC code is defined by rns0=4· k +1 (=1, 5, 9, …, or 65). rns0 = 4 × kns + 1 and rns0 = 16 × kns + 1 for ranging Format 0 and Format 1, respectively, where kns (kns = 0, 1, 2, …, 15) is a cell specific value broadcasts through S-SFH. The ranging preamble code partition information indicate indicates the number of initial and handover ranging preamble codes and is defined in Table 877.

----------------------------------------------  Text End  ---------------------------------------------------