| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on AAI_NBR-ADV (15.2.3.11) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Xiangying Yang Kamran Etemad Intel | E-mail: xiangying.yang@intel.com |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.3.11” |  |
| Abstract | This contribution proposes text for AAI_NBR-ADV format |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal on AAI_NBR-ADV (15.2.3.11)

Xiangying Yang, Kamran Etemad

Intel

- Introduction

The text change propose to introduce frame offset in a mixed-mode deployment scenario to facilitate MS scanning and handover

- Proposed Test

Text proposal

--------------------------------text start-----------------------------------------

15.2.3.11 AAI_NBR-ADV

AAI_NBR-ADV message may sort neighbor ABSs (RSs) according to their deployment types, which is categorized by the following parameters:

1)ABS type (macro, micro, femto, relay, TBD)

2)carrier frequency

3)MAC version

4)TDD/FDD and related definitions (expected to be the same given carrier frequency)

5)BW, CP info: may not be required if carried in P/S-SCH channel

6)Multicarrier capability and configuration

ABS determines and indicates the system configuration information included for each deployment type and their corresponding broadcast frequency.

To allow AAI_NBR-ADV fragmentation while providing flexibility for AMS HO operation without requiring acquisition of the whole AAI_NBR-ADV message, ABS always provides the total number of deployment types and total number of recommended T-ABS for each type. Each AAI_NBR-ADV fragment has corresponding indexes for each deployment type and each neighbor ABS. ABSs with identical type are listed in the AAI_NBR-ADV message in descending order of their cell coverage.

Each AAI_NBR-ADV message carries

•AAI_NBR-ADV change count

•number of total cell types

•segment information for this AAI_NBR-ADV message

•system information of ABSs from one or more cell types, which is of variable size.

For each cell type, shared configuration parameters are carried as follows

•FA information

•BW information

•Duplex mode

•MAC protocol version

•Zone starting offset in the number of OFDMA symbols

For each fragment of the AAI_NBR-ADV, the fragment shall include the following parameters - Starting BS Index (i.e. this is the index offset from the last BS of the previous fragment. If this is first fragment, the Starting Index will be 0. Hence, each fragment has one Index which corresponds to the first BS in that fragment.)

Within each cell type, a list of ABSs are provided and the following parameters are carried for each ABS

•48bit BS-ID

•ABS A-Preamble index

•indication whether full system information or partial information is carried for this ABS, which includes

—SFH information

—ESI information

—Index to MC configuration,

where for macrocell type, all the necessary system information shall be included, and the format may only carry delta information fields with respect to the reference ABS(e.g., the serving BS or the first BS/ABS in this cell type).

•SFH_encoding_format: Based on the present system information, the list of ABSs shall be categorized into three groups. The categorization is indicated by SFH_encoding_format parameter

—0b00: Reserved

—0b01: full Subpacket information

—0b10: delta encoding with TBD Format

—0b11: no SFH included

--------------------------------text end-----------------------------------------