| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal on 16m security PKMv3 state machines (15.2.5) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Avishay Shraga Chang Hong Shan Xiangying Yang | E-mail: xiangying.yang@intel.com Phone : *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.5” |  |
| Abstract | This contribution proposes the PKMv3 authentication/TEK state machines. |  |
| Purpose | Accept the proposed specification changes on IEEE 802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal on 16m security PKMv3 state machines (15.2.5)

Avishay Shraga, Chang Hong Shan, Xiangying Yang

Intel

# Introduction

In IEEE 802.16m/D2 [1], the authentication and TEK state machines are missing. This contribution proposes the PKMv3 authentication and TEK state machines.

# References

[1] IEEE P802.16m/D2, “Part 16: Air Interface for Broadband Wireless Access Systems(Advanced Air Interface)”, July. 2009.

[2] IEEE Std 802.16-2009, “Part 16: Air Interface for Broadband Wireless Access Systems”, May. 2009

# Proposed Text

Add all the following proposed text into Section 15.2.5 as a new sections 15.2.5.6 and 15.2.5.7.

----------------------------------------------------- Start of Proposed Text ---------------------------------------------------

15.2.5.6 Authentication state machine

As in legacy system, the PKMv3 authentication state machine for single EAP authentication consists of six states and sixteen events (including receipt of messages and events from other FSMs) that may trigger state transitions. However, some states, messages and events are modified as newly defined in [1]. The authentication state machine is presented in both a state flow diagram (Figure 15.2.5.6.1) and a state transition matrix (Table 15.2.5.6.1). The state transition matrix shall be used as the definitive specification of protocol actions associated with each state transition.

In PKMv3 [1], the Authentication process has 2 phases: EAP phase and key agreement phase.

The EAP phase is controlled by the EAP_FSM as defined in RFC3748 and RFC4173 and it is out of scope in this standard.

The Auth_FSM is responsible for all PKM phase but the actual EAP exchange and communicates with other FSMs in the system using events.

Through operation of an Authentication state machine, the AMS attempts to get authenticated with the NW, maintain this authentication and support Authentication context switching for HO and Idle situations. The state machine takes care of getting authenticated with the NW, ensuring re-authentication will occur before authentication expires and supports key derivations according to support optimized re-entry for HO and idle.

The optimized re-entry support is done in a special state in which the NW connection is suspended and therefore re-authentication can’t occur, the triggers for re-authentication continue to work in this state but the initiation is done only after returning to an authenticated state.

Figure 15.2.5.6.1 Authentication State Machine for PKMv3 single EAP

Table 15.2.5.6.1. Authentication FSM state transition matrix for PKMv3

| State Event or receive message | (A) Stopped | (B) Not Authenticated | (C) Key Agreement MSG #3 Wait | (D) Authenticated | (E) Reauth Key Agreement MSG #3 Wait | (F) Reentry Auth Wait |
| --- | --- | --- | --- | --- | --- | --- |
| (1) Start Auth | Not Authenticated |  |  |  |  | Not Authenticated |
| (2) PKMv3 Key Agreement MSG #1 |  | Key Agreement MSG #3 Wait | Key Agreement MSG #3 Wait | Reauth Key Agreement MSG #3 Wait, | Reauth Key Agreement MSG #3 Wait |  |
| (3) PKMv3 Key Agreement MSG #3 |  |  | Authenticated |  | Authenticated |  |
| (4) EAP Success |  | Not Authenticated |  | Authenticated |  |  |
| (5) Key Agreement Timeout |  |  | Key Agreement MSG #3 Wait |  | Reauth Key Agreement MSG #3 Wait |  |
| (6) Key Agreement MSG #2 max resend elapsed |  |  | Stopped |  | Stopped |  |
| (7) ReAuth needed |  |  |  | Authenticated |  |  |
| (8) Start Reentry |  |  |  | Reentry Auth Wait | Reentry Auth Wait |  |
| (9) EAPStart timeout |  |  |  | Authenticated |  |  |
| (10) HO cancelled |  |  |  |  |  | Authenticated |
| (11) TBS change |  |  |  |  |  | Reentry Auth Wait |
| (12) Reentry Completed |  |  |  |  |  | Authenticated |
| (13) Auth Expired |  |  |  | Stopped | Stopped | Stopped |
| (14) EAP Fail |  |  |  | Stopped |  |  |
| (15) External Stop |  | Stopped | Stopped | Stopped | Stopped | Stopped |

15.2.5.6.1 States

Stopped: This is the initial state of the FSM. Nothing is done in this state.

Not Authenticated: The Authorization FSM is not authenticated and waiting for an MSK from the EAP FSM to perform key agreement. The FSM also waits for a PKMv3 Key Agreement MSG #1 in this state. Upon receiving a PKMv3 Key Agreement MSG #1,

Key Agreement MSG #3 Wait: The Authorization FSM has sent a PKMv3 Key Agreement MSG #2 protected by CMAC using the calculated CMAC key and waits for a PKMv3 Key Agreement MSG #3 in this state. If it does not receive a PKMv3 Key Agreement MSG #3 within Key Agreement Timer, the AMS may resend the message up to Key Agreement MSG #2 Max Resends times. Upon receiving a PKMv3 Key Agreement MSG #1 including same BS_Nonce as received in the original one , the AMS resends a PKMv3 Key Agreement MSG #2. Any  PKMv3 Key Agreement MSG #3 with invalid CMAC Digest or without CMAC Digest is discarded.

Authenticated: The AMS has successfully completed EAP-based authentication and key agreement and has valid Key context derived from the MSK received from the EAP FSM. Transition from Key Agreement MSG #3 Wait into this state triggers the creation of TEK keys and FSMs.

once  the AMS has a valid AK context, all the management messages should be protected as defined per message type. a received message must be discarded if the message is not protected as defined in this spec. In this state the AMS may hold two PMK contexts: the old PMK context and the new PMK context which is created during re-authentication. The old PMK context is deleted in the frame number specified in the PKMv3 Key Agreement MSG #3. In addition, the authentication FSM also waits for a PKMv3 Key Agreement MSG #1 in this state before the AK expires, this message must be protected with CMAC using the current authentication key context.

Fresh Key Agreement MSG #3 Wait: The authentication FSM has sent a PKMv3 Key Agreement MSG #2 protected with CMAC derived from the new key context for re-authentication and waits for a PKMv3 Key Agreement MSG #3 protected with same CMAC key. If it does not receive a PKMv3 Key Agreement MSG #3 within Key Agreement Timer, the AMS should resend the message up to Key Agreement MAX Resends times.

In this state there are two PMK contexts: the old PMK and sub keys context for the management messages including key agreement MSG#1 which need CMAC and new PMK and sub keys context for Key Agreement #2 and #3 messages during key agreement refresh. The new Key context is created as soon as key agreement MSG#1 received. After the completion of the Key Agreement and TEK update based on new authentication, the old Key context should be discarded.

Reentry Authentication Wait: In this state the Authorization FSM has the PMK context of the target ABS. The AMS should have the PMK context of the target ABS in this state before it sends a RNG-REQ message with CMAC Digest during (ZS) HO or reentry. During (ZS) HO or reentry, the Authorization FSM is in this state when the AMS sends a RNG-REQ message. The state of Authorization FSM changes when the AMS receives a RNG-RSP message. The next state depends on the value of HO Process Optimization TLV included in the received RNG-RSP message. If HO Process Optimization Bit #1 set to zero, meaning the PKM Authentication phase is not omitted, the Authorization FSM receives Start Authentication event which triggers to stop all the TEK FSMs, re-initialize the Authorization FSM and change the state to Not Authenticated. The TLVs included in the RNG-RSP message also affects the next state. If HO Process Optimization Bit #1 and Bit #2 set to one and zero respectively and the SA-TEK-Update TLV is included in the RNG-RSP, the FSM receives Reentry Completed event. The state of the Authorization FSM changes to Authenticated when the Reentry Completed event is issued.

15.2.5.6.2 Messages

PKMv3 Key Agreement MSG #1: The first message of Key Agreement. It is sent from the ABS to the AMS after EAP-based authentication has finished or once the BS decides to renew the KEY context for PMK and derived keys and it is protected by CMAC-Digest using CMAC_KEY_D of the active Key context..

PKMv3 Key Agreement MSG #2: The second message of Key Agreement. It is sent from the AMS to the ABS as a response to a PKMv3 Key Agreement MSG #1, protected by CMAC-Digest using CMAC_KEY_U of the freshest EAP-based authentication (same as active if only key agreement happens or new MSK in case of full EAP re-auth)

PKMv3 Key Agreement MSG #3: The last message of Key Agreement. It is sent from the ABS to the AMS as a response to a PKMv3 Key Agreement MSG #2 and it is protected by CMAC-Digest using CMAC_KEY_D of the freshest EAP-based authentication.

PKMv23 EAP Start: The message used by the AMS to request the BS to renew all the key hierarchy (PMK and derivatives) either by initiating EAP-based re-authentication or just new key agreement. If new key agreement is not completed within fresh key agreement timer (TBD), the AMS may re-send EAP Start.

PKMv3 EAP Transfer: This message is bidirectional and used for transmission of EAP packet. This message is sent unprotected in “Not Authenticated” state. In Authenticated state, CMAC Digest and Key Sequence Number attributes shall be included in the message.

15.2.5.6.3 Events

Start Authentication: After completion of basic capabilities negotiation, this event is generated to start the Authentication state machine. It is also issued when the HO Process Optimization Bit #1 of the RNG-RSP message is set to zero during HO or network reentry.

EAP Success: EAP FSM generates this event to notify the Authorization FSM that it received EAP Success message from the authenticator.

Key Agreement Timeout: This event is generated when the AMS does not receive PKMv3 Key Agreement MSG #3 from the ABS within Key Agreement Timer after transmitting a PKMv3 Key Agreement MSG #2. The AMS may resend the PKMv3 Key Agreement MSG #2 up to Key Agreement Max Resends times.

Key Agreement MSG #2 max resends elapsed: The Authorization state machine generates this event when the AMS has transmitted the PKMv3 Key Agreement MSG #2 up to Key Agreement MSG #2 Max Resends times and Key Agreement Timer expires.

Key context refresh Needed: An internal event to trigger new key hierarchy. This event can be derived from several sources such as Authorization Grace Timeout or other reason that makes authentication close to expiration.

Start Reentry: An event to inform the Authorization FSM that AMS is in reentry phase. The FSM should derive the new AK context for the target ABS.

EAPStart Timeout: A timer event that causes the AMS to resend a PKMv23 EAP-Start message in order to ask the ABS to refresh the key hierarchy from PMK and down. This event is used in the case key agreement is not completed successfully during EAPStart timer from transmitting the PKMv23 EAP Start message. This timer is active only after key context refresh needed event occurred.

Reentry Completed: An event to notify the Authorization FSM that reentry has finished successfully. This event is issued when the AMS receives a RNG-RSP message including HO Process Optimization Bit #1 and Bit #2 set to one and zero respectively and SA-TEK-Update TLV during HO or network re-entry from Idle mode. This event is also issued when the AMS receives a RNG-RSP message including HO Process Optimization Bit #1 and Bit #2 both set to one during HO.

HO Canceled: An event to notify the Authorization FSM that HO was canceled. The cached AK context for the serving ABS should be retrieved.

TBS (Target ABS) changed: An Event to notify the Authorization FSM that it needs to generate the AK con-text for the new target ABS.

Authentication Expired: This event indicates the PMK context became obsolete due to the expiration of PMK lifetime or PN sapce.

EAP Failure: This event indicates EAP-fail Has been received from the NW.

External Stop: The event to stop the Authorization FSM and terminate connection with ABS.

NOTE-The following events are sent by an authentication state machine to the EAP state machine:

[EAP] key agreement comleted: sent to the EAP FSM once key agreement is completed which means that new authentication is valid and old keys may be discaeded.

NOTE-The following events are sent by an Authorization state machine to the TEK state machine:

[TEK] Stop: Sent by the Authorization FSM to an active (non-START state) TEK FSM to terminate the FSM and remove the corresponding SAID’s keying material from the AMS’s key table.

[TEK] Authorized: Sent by the Authorization FSM to a nonactive (START state), but valid TEK FSM.

[TEK] Authorization Pending (Auth Pend): Sent by the Authorization FSM to a specific TEK FSM to place that TEK FSM in a wait state until the Authorization FSM can complete its reauthorization operation. This event shall be sent to the TEK FSM in the Operational Wait (Op Wait) or Rekey Wait states when Authorization FSM starts re-authentication.

[TEK] Authorization Complete (Auth Comp): Sent by the Authorization FSM to a TEK FSM in the Operational Reauthorize Wait (Op Reauth Wait) or Rekey Reauthorize Wait (Rekey Reauth Wait) states to clear the wait state begun by a TEK FSM Authorization Pending event. This event shall be sent to the TEK FSM in the Operational Reauthorize Wait or Rekey Reauthorize Wait states when Authorization FSM ends re-authentication.

15.2.5.6.4 Parameters

The timer which expires if the AMS does not receive a PKMv3 Key Agreement MSG #3 after sending a PKMv3 Key Agreement MSG #1.

EAPStart Timeout: Timeout period between sending PKMv23 EAP Start messages from Authenticated state.

Authorization Grace Time: Amount of time before the AK is scheduled to expire in order that the AMS may start re-authentication before the authentication expiry.

15.2.5.6.5 Actions

1-A: Stopped (Start Auth) −> Not Authenticated

a) Enable PKMv23 EAP-Transfer messages to be transferred.

1-F: Reentry Authentication Wait (Start Auth) -> Not Authenticated

a) Stop TEK FSMs

b) Re-initialize the Authorization FSM

c) Enable PKMv2 EAP-Transfer messages to be transferred.

2-B: Not Authenticated (PKMv3 Key Agreement MSG #1) −> SA-TEK-Response Wait Key Agreement MSG #3 Wait

a) Send a PKMv3 Key Agreement MSG #2.

b) Start SATEK Timer.

2-C: Key Agreement MSG #3 Wait (PKMv3 Key Agreement MSG #1) −> Key Agreement MSG #3 Wait

a) Send a PKMv3 Key Agreement MSG #2.

b) Start Key Agreement Timer.

2-D: Authenticated (PKMv3 Key Agreement MSG #1) −> Reauth Key Agreement MSG #3 Wait, Key Agreement MSG #3 Wait

a) Send a PKMv3 Key Agreement MSG #2.

b) Start Key Agreement Timer.

2-E: Reauth Key Agreement MSG #3 Wait (PKMv23 Key Agreement MSG #1) −> Reauth Key Agreement MSG #3 Wait

a) Send a PKMv3 Key Agreement MSG #2.

b) Start Key Agreement Timer.

3-C: Key Agreement MSG #3 Wait (PKMv23 Key Agreement MSG #3) −> Authenticated

a) Stop Key Agreement Timer

b) Start TEK FSMs

c) Start Authorization Grace Timer

3-E: Reauth Key Agreement MSG #3 Wait (PKMv3 Key Agreement MSG #3) −> Authenticated

a) Stop Key Agreement Timer

b) Start Authorization Grace Timer

c) Set the frame number for old AK context to be invalid.

4-B: Not Authenticated (EAP Success) −> Not Authenticated

a) Obtain the MSK

b) Derive the keys derived from the PMK

4-D: Authenticated (EAP Success) −> Authenticated

a) Obtain the new MSK

b) Derive the keys derived from the PMK

5-C: Key Agreement MSG #3 Wait (Key Agreement Timeout) -> Key Agreement MSG #3 Wait

a) Send a PKMv3 Key Agreement MSG #2.

b) Start Key Agreement Timer.

5-E: Reauth Key Agreement MSG #3 Wait (Key Agreement Timeout) -> Reauth Key Agreement MSG #3 Wait

a) Send a PKMv3 Key Agreement MSG #2.

b) Start Key Agreement Timer.

6-C: Key Agreement MSG #3 Wait (Key Agreement MSG #2 max resend elapsed) -> Stopped

a) Stop the Authorization FSM

6-E: Reauth Key Agreement MSG #3 Wait (Key Agreement MSG #2 max resend elapsed) -> Stopped

a) Stop TEK FSMs

b) Stop the Authorization FSM

7-D: Authenticated (Re-authentication Needed) −> Authenticated

a) Send a PKMv3 EAP-Start message

b) Start EAPStart Timer

8-D: Authenticated (Start Reentry) −> Reentry Authentication Wait

a) Generate the AK context for the target ABS.

8-E: Reauth Key Agreement MSG #3 Wait (Start Reentry) -> Reentry Authentication Wait

a) Remove the new AK context for the serving ABS generated during performing EAP-based re-authentication procedure

b) Generate the AK contexts for the target ABS generated from old PMK context and new PMK con-text

9-D: Authenticated (EAPStart Timeout) −> Authenticated

a) Send a PKMv23 EAP-Start message

b) Start EAPStart Timer

10-F: Reentry Authentication Wait (HO Canceled) -> Authenticated

a) Remove the AK context for the target ABS

b) Retrieve the cached AK context for the serving ABS

11-F: Reentry Authentication Wait (TABS changed) -> Reentry Authentication Wait

a) Generate the AK context of new target ABS

12-F: Reentry Authentication Wait (Reentry Completed) -> Authenticated

a) Update the AK context for the target ABS

13-D,E,F: Any state except Stopped, Key Agreement MSG #3 Wait and Not Authenticated (Authentication Expired) −> Stopped

a) Stop TEK FSMs

b) Stop the Authorization FSM

14-B: Not Authenticated (EAP Failure) −> Stopped

a) Stop the Authorization FSM

14-D: Authenticated (EAP Failure) −> Stopped

a) Stop TEK FSMs

b) Stop the Authorization FSM

15-B,C: Not Authenticated and Key Agreement MSG #3 Wait (External Stop) −> Stopped

a) Stop the Authorization FSM

15-D,E,F: Any state except Stopped, Not Authenticated, and SA-TEK-Response Key Agreement MSG #3 Wait (External Stop) −> Stopped

a) Stop TEK FSMs

b) Stop Authorization Grace Timer

c) Stop the Authorization FSM

15.2.5.7 TEK state machine

Contrary to the TEK state machine of legacy system, the TEK state machine of 16m consists of four states and eight events (including receipt of messages) that may trigger state transitions. Like the Authorization state machine, the TEK state machine is presented in both a state flow diagram (Figure 15.2.5.7.1) and a state transition matrix (Table 15.2.5.7.1). However, as defined in [1], all M(ulticast) & B(roadcast) related states, messages and events are not specified in 16m TEK state machine. As was the case for the Authorization state machine, the state transition matrix shall be used as the definitive specification of protocol actions associated with each state transition.

Shaded states in Figure 15.2.5.7.1 (Operational, Rekey Wait, Rekey Reauthorize Wait) have valid keying material and encrypted traffic may be sent.

The Authentication state machine starts an independent TEK state machine for each of its authorized SAIDs. As mentioned in [1], the ABS maintains two active TEKs per SAID.

For the TEK update of unicast service, the ABS includes in its Key Response EKS, AK Seq_Num, COUNTER_TEK. The BS encrypts DL traffic with the older of its two TEKs and decrypts UL traffic with either the older or newer TEK, depending upon which of the two keys the SS was using at the time. The SS encrypts UL traffic with the newer of its two TEKs and decrypts DL traffic with either the older or newer TEK. See 15.2.5.2.4 [1] for details on AMS and ABS key usage requirements.

Through operation of a TEK state machine, the AMS attempts to keep its copies of an SAID’s TEKs synchronized with those of its ABS. A TEK state machine issues Key Requests to refresh copies of its SAID’s keying material when either the DL or UL PN space is running out. To accommodate for SS/BS clock skew and other system processing and transmission delays, the AMS schedules its Key Requests a configurable number of seconds before either the DL or UL PN space is used up. With the receipt of the Key Reply, the AMS shall always update its records with the TEK parameters from TEK parameters contained in the Key Reply message.

Figure 15.2.5.7.1 TEK State Machine for PKMv3 single EAP

15.2.5.7.1 States

Start: This is the initial state of the FSM. No resources are assigned to or used by the FSM in this state - e.g., all timers are off, and no processing is scheduled.

Operational: The AMSS has valid keying material for the associated SAID.

Rekey Wait: The TEK Refresh Timer has expired and the AMS has requested a key update for this SAID. Note that the newer of its two TEKs has not expired and may still be used for both encrypting and decrypting data traffic.

Rekey Reauth Wait: The wait state the TEK state machine is placed in if the TEK state machine has valid traffic keying material, has an outstanding request for the latest keying material, and the Authorization state machine initiates a reauthorization cycle.

15.2.5.7.2 Messages

Note that the message formats are defined in detail in x.x.x.x.x.

Key Request: Request a TEK for this SAID. Sent by the AMSS to the ABS and authenticated with keyed message digest. The message authentication key is derived from the AK.

Key Reply: Response from the ABS carrying the parameters of traffic keying material for this SAID. Sent by the ABS to the AMSS, it includes the SAID’s EKS, AK Seq_Num, COUNTER_TEK. The Key Reply message is authenticated with a keyed message digest; the authentication key is derived from the AK.

TEK Invalid: The ABS sends an AMSS this message if it determines that the AMSS encrypted an UL PDU with an invalid TEK, i.e., an SAID’s TEK key sequence number, contained within the received PDU’s MAC header, is out of the ABS’s range of known, valid sequence numbers for that SAID.

15.2.5.7.3 Events

Stop: Sent by the Authorization FSM to an active (non-START state) TEK FSM to terminate TEK FSM and remove the corresponding SAID’s keying material from the AMS’s key table.

Authorized: Sent by the Authorization FSM to a non-active (START state) TEK FSM to notify TEK FSM of successful authorization.

Authorization Pending (Auth Pend): Sent by the Authorization FSM to TEK FSM to place TEK FSM in a wait state while Authorization FSM completes reauthorization.

Authorization Complete (Auth Comp): Sent by the Authorization FSM to a TEK FSM in or Rekey Reauthorize Wait states to clear the wait state begun by the prior Authorization Pending event.

TEK Invalid: This event is triggered by either an AMS’s data packet decryption logic or by the receipt of a TEK Invalid message from the ABS.

An AMS’s data packet decryption logic triggers a TEK Invalid event if it recognizes a loss of TEK key synchronization between itself and the encrypting ABS. For example, an SAID’s TEK key sequence number, contained within the received DL MAC PDU header, is out of the AMS’s range of known sequence numbers for that SAID.

A ABS sends an AMS a TEK Invalid message, triggering a TEK Invalid event within the AMSS, if the ABS’s decryption logic recognizes a loss of TEK key synchronization between itself and the AMS.

Retry Timer Timeout: A retry timer timeout. Generally, the particular request is retransmitted.

TEK Refresh Timer Timeout: The TEK refresh timer timed out. This timer event signals the TEK state machine to issue a new Key Request in order to refresh its keying material. The refresh timer is set to fire a configurable duration of time (TEK Grace Time)before either the DL or UL PN space is used up. This is configured via the BS to occur before the scheduled expiration of PN spaces of the two TEKs.

TEK Lifetime Timer Timeout: The TEK lifetime timer times out. This timer event signals the TEK state machine has no valid TEK for UL or DL traffic.

PN space exhausted: When the PN space for UL or DL traffic is exhausted and AMS hasn’t got new TEK for for this SA, this event will be triggered.

Table 15.2.5.7.1. TEK FSM state transition matrix for PKMv3

| State Event or Rcvd Message | (A) Start | (D) Op | (E) Rekey Wait | (F) Rekey Reauth Wait |
| --- | --- | --- | --- | --- |
| (1) Stop |  | Start | Start | Start |
| (2) Authorized | Op |  |  |  |
| (3) Auth Pend |  |  | Rekey Reauth Wait |  |
| (4) Auth Comp |  |  |  | Rekey Wait |
| (5) TEK Invalid |  | Rekey Wait |  |  |
| (6) Timeout |  |  | Rekey Wait |  |
| (7) TEK Refresh Timeout |  | Rekey Wait |  |  |
| (8) Key Response |  |  | Operational |  |

15.2.5.7.4 Parameters

All configuration parameter values take the default values from Table 554 or may be specified in Auth Reply message.

Rekey Wait Timeout: Timeout period between sending of Key Request messages from the Rekey Wait state (see 11.9.18.5).

TEK Grace Time: Time interval, in seconds, before the estimated expiration of either DL or UL of a TEK that the AMSS starts rekeying for a new TEK. TEK Grace Time takes the default value from Table 554 xxx or may be specified in a configuration setting within the Auth Reply message and is the same across all SAIDs (see 11.9.18.6).

TEK Lifetime Timer:

PN Threshhold:

15.2.5.7.5 Actions

Actions taken in association with state transitions are listed by <event> (<rcvd message>) --> <state>:

1-D Operational (Stop) → Start

a) Clear TEK refresh timer, which is timer set to go off “TEK Grace Time” seconds prior to the TEK’s scheduled

expiration time

b) Terminate TEK FSM

c) Remove SAID keying material from key table

1-E Rekey Wait (Stop) → Start

a) Clear Key Request retry timer

b) Terminate TEK FSM

c) Remove SAID keying material from key table

1-F Rekey Reauth Wait (Stop) → Start

a) Terminate TEK FSM b) Remove SAID keying material from key table

2-A Start (Authorized) → Op

a) Send Key Request message to ABS

b) Set Key Request retry timer to Operational Wait Timeout

3-E Rekey Wait (Auth Pend) → Rekey Reauth Wait

a) Clear Key Request retry timer

4-F Rekey Reauth Wait (Auth Comp) → Rekey Wait

a) Send Key Request message to ABS

b) Set Key Request retry timer to Rekey Wait Timeout

5-D Operational (TEK Invalid) → Rekey_Wait

a) Send a Key Request message to ABS

b) Set Key Request retry timer to Rekey Wait Timeout

6-E Rekey Wait (Timeout) → Rekey Wait

a) Send Key Request message to ABS

b) Set Key Request retry timer to Rekey Wait Timeout

7-D Operational (TEK Refresh Timeout) → Rekey Wait

a) Send Key Request message to ABS

b) Set Key Request retry timer to Rekey Wait Timeout

7-G M&B Rekey Interim Wait (TEK Refresh Timeout) -> Rekey Wait

a) Send Key Request message to ABS

b) Set Key Request retry timer to Rekey Wait Timeout

8-E Rekey Wait (Key Reply) → Operational

a) Clear Key Request retry timer

b) Process contents of Key Reply message and incorporate new keying material into key database

c) Set the TEK refresh timer to go off “TEK Grace Time” seconds prior to the newer key’s scheduled expiration

--------------------------------------------------- End of Proposed Text------------------------------------------------------