| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on Adaptive Granting and Polling Service (15.2.12.3) |  |
| Date Submitted | 2009-11-18 |  |
| Source(s) | Xiangying Yang Jie Hui Muthaiah Venkatachalam Intel | E-mail: xiangying.yang@intel.com |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.2.3” |  |
| Abstract | This contribution proposes text for AAI_NBR-ADV format |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal on Adaptive Granting and Polling Service (15.2.12.3)

Xiangying Yang, Jie Hui, Muthaiah Venkatachalam

Intel

- Introduction

The text change proposes to clarify Adaptive Granting and Polling Service scheduling behavior.

- Proposed Test

Text proposal

Modify Section 15.2.12.3 as follows

--------------------------------text start-----------------------------------------

15.2.12.3 Scheduling services

Scheduling services represent the data handling mechanisms supported by the MAC scheduler for data transport on a connection. Each service flow is associated with a single scheduling service as in WirelessMAN-OFDMA reference system. A scheduling service is determined by a set of SF QoS parameters that quantify aspects of its behavior. These parameters are established or modified using service flow management procedures.

15.2.12.3.1 Adaptive granting and polling service

The set of QoS parameters associated with adaptive granting and polling services are categorized into primary QoS parameters and secondary QoS parameters. The ABS may grant or poll AMS periodically and may negotiate only primary SF QoS parameters sets, or both primary and secondary QoS parameters sets with the AMS. Initially, ABS uses QoS parameters defined in the primary SF QoS parameters set including Primary Grant and Polling Interval (GPI) and Primary Grant Size. When both primary and secondary QoS parameters are negotiated, primary QoS parameters shall have more stringent QoS requirement than secondary QoS parameters. The admission control shall be done by considering the most stringent QoS requirement defined by the primary QoS parameters. The secondary QoS parameters are the minimal QoS guarantee that the scheduling service shall always provide.

During the service, the traffic characteristics and QoS requirement may change, for example silence-suppression enabled VoIP alternates between talk spurt and silence period, which triggers adaptation of SF QoS parameters the scheduling service state machine as described below. Adaptation of scheduling state includes switching between using primary and using secondary SF QoS parameters, sets or changing of GPI/Grant size to values other than what within the QoS flexibility range, i.e., without exceeding the maximal QoS requirement or violating the minimal QoS guarantee, that are defined by in the primary or secondary SF QoS parameters set when the traffic can be characterized by more than two QoS states.

--------------------------------text end-----------------------------------------