| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text proposal on EBB HO capability in AAI_REG-REQ (15.2.3.6) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Xiangying Yang Intel | E-mail: xiangying.yang@intel.com |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.3.6” |  |
| Abstract | This contribution proposes text for EBB HO capability |  |
| Purpose | Adopt proposed text. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text proposal on EBB HO capability in AAI_REG-REQ (15.2.3.6)

Xiangying Yang

Intel

- Introduction

The text change propose to complete the EBB HO capability negotiation in AAI_REG-REQ

- Proposed Test

Text proposal

Insert one row of capability in Table 676 within category “AMS capability negotiation parameters” as follows:

--------------------------------text start-----------------------------------------

15.2.3.6 AAI_REG-REQ

Table 676—Parameters for AAI_ REG-REQ

| Name |  |  |  | Value |
| --- | --- | --- | --- | --- |
| AMS capability negotiation parameters | ARQ parameters | ARQ_SN_MODULUS |  | the number of unique SN values; default =2^10 |
|  |  | ARQ_WINDOW_SIZE |  | the maximum number of ARQ block swith consecutive SN in the sliding window of ARQ blocks |
|  |  | ARQ_SUB_BLOCK_SIZE |  | ARQ sub-block length when ARQ block is fragmented into ARQ sub-blocks prior to retransmission with rearrangement |
|  |  | ARQ_BLOCK_LIFETIME |  | the maximum time interval an ARQblock shall be managed by the transmitter ARQ state machine, once initial transmission of the block has occurred |
|  |  | ARQ_RX_PURGE_TIMEOUT |  | the time interval the receiver shall wait after successful reception of a block that does not result in advancement ofARQ_RX_WINDOW_START, before advancingARQ_RX_WINDOW_START |
|  |  | MAXIMUM_ ARQ_BUFFER_SIZE |  | The maximum ARQ buffer size |
|  | LBS capabilities | GPS capability |  | 0: AMS is GPS capable. 1: AMS is not GPS capable. |
|  |  | Reserved |  | TBD |
|  | Co-located coexistence capability supported |  |  | Indicates if co-located coexistence is supported. |
|  | Capabilities for MS measurement |  |  | TBD |
|  | Capabilities for MIMO support |  |  | TBD |
|  | Capabilities for Handover EBB mode HO_Reentry_Mode = 1 |  | Minimal HO_Reentry_Interleaving_Interval | The minimal HO_Reentry_Interleaving_Interval measured in frames |
|  |  |  | Minimal HO_Reentry_Iteration | The minimal HO_Reentry_Iteration measured in frames |
|  |  |  | Reserved | TBD |

--------------------------------text end-----------------------------------------