| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text Changes to Primary Carrier Change (15.2.8.2.11.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Kamran Etemad Xiangying Yang, Intel Corporation | Kamran.etemad@intel.com Xiangying.Yang@intel.com |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.8.2.11.2” |  |
| Abstract | This document contains the place holder for possible input by TGm members |  |
| Purpose | Place holders for possible text input by TGm members through reply comments |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text Changes to Primary Carrier Change (15.2.8.2.11.2)

Kamran Etemad

Xiangying Yang,

Intel Corporation

Note:

The text in BLACK color: the existing text in AWD

The text in RED color: the removal of existing AWD text

The text in BLUE color: the new text added by Multi-Carrier DG

Modify Section 15.2.8.2.11.2 as follows.

------------------------------------------------------------Start of the Text--------------------------------------------------------

15.2.8.2.11.2 Primary Carrier Change

The ABS may instruct the AMS, through the AAI_CM-CMD MAC control message on the current primary carrier, to change its primary carrier to one of the assigned fully configured carriers within the same ABS for load balancing purpose, carriers' varying channel quality or other reasons.

The AMS may also request ABS for Primary carrier change if the AMS expects radio link failure of the serving primary carrier. AMS can indicate such a request by sending a RNG-REQ message on one of the target assigned carriers which the AMS received with good quality.

When an AMS receives the AAI_CM-CMD MAC control message, the AMS transmits the AAI_CM-IND MAC control message and the AMS switches to the target fully configured carrier at action time specified by the ABS.

If the target carrier is one of the active secondary carriers of the AMS, the AMS may receive data and control signal on the target carrier immediately after switching. Otherwise, the AMS may transmit a bandwidth request to notify its readiness of the target carrier to the ABS. If the AMS has UL data to be transmitted, it follows the normal bandwidth request procedure. If the AMS does not have any UL data, it transmits a bandwidth request with zero bandwidth request size.

The ABS may transmit data and control signal after a bandwidth request is received from the AMS through the target primary carrier. Given that a common MAC manages both serving and target primary carriers, network re-entry procedures at the target primary carrier is not required. The ABS may provide the system information of the target primary carrier that is different from the serving primary

carrier via the serving primary carrier. The logical carrier indices of the serving and target primary carrier

are swapped after the primary carrier change. The ABS may direct an AMS to change the primary carrier without scanning.The ABS may instruct the AMS to perform scanning on other carriers which are not serving the AMS. The AMS reports the scanning results back to the serving ABS, which may be used by the ABS to determine the carrier for the AMS to switch to. In this case, if the target carrier is not currently serving the AMS, the AMS may perform synchronization with the target carrier if required.

The AAI_CM-CMD MAC control message for the primary carrier change is transmitted on the primary carrier and shall include the following information:

- Target primary carrier index

- Indication of the next state of serving primary carrier

- Action Time

The serving primary carrier will be kept active or deactivated depending on the indication of the next state of serving primary carrier.

-------------------------------------------------------------End of the Text--------------------------------------------------------