| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Feedback Polling A-MAP IE (15.3.6.5.2.11) for the IEEE 802.16m/D2 |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | - Huaning Niu - Yuan Zhu - Alexei Davydov - Yang-seok Choi Intel Corporation | Huaning.niu@intel.com Yuan.y.zhu@intel.com |
| Re: | Call for comments for Latter Ballot 30a |  |
| Abstract | Text change proposal for feedback Polling A-MAP IE and related MAC control message |  |
| Purpose | To be discussed and adopted by TGm for D3 draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text for Feedback Polling A-MAP IE (15.3.6.5.2.14) for the IEEE 802.16m/D2

- Huaning Niu, Yuan Zhu, Alexei Davydov, Yang-seok Choi

- Intel Corporation

- Introduction

In this contribution, we propose to add the long term feedback period in Feedback Polling A-MAP IE to support correlation matrix transmission using MAC control message.

- References

[1] IEEE 802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems Advanced Air Interface”, Oct 2009

- Proposed Text

Instruction to editor;

Black text: existing D2 text

Red and strike out text: for deletion

Blue and underline text: for addition

--------------------------------Start of the text #1 ----------------------------------------

Make the following changes in section 15.3.6.5.2.14, page 397, Table 821

The information element shown in Table 821 is used by the ABS to schedule MIMO feedback transmission by the AMS. The AMS sends the MIMO feedback using a MAC control message or an extended header, depending on the requested feedback content.

If the feedback includes only the quantized transmit correlation matrix when the ABS is equipped with 2 or 4 transmit antennas, or only the wideband information for MIMO feedback modes 0, 1, 4 and 7, or only the subband information for 1 subband for MIMO feedback modes 2, 3, 5 and 6, then the MS shall use an extended header. No dedicated resource will be allocated for feedback header transmission. If the feedback includes only the quantized transmit correlation matrix when the ABS is equipped with 2 or 4 transmit antennas, then the MS can use either an extended header, or the MAC control message. When dedicated resource is allocated, the MS shall use the MAC control message. When no dedicated resource is allocated, the MS shall use the extended MAC header. In other cases, for the feedback of the quantized transmit correlation matrix when the ABS is equipped with 8 transmit antennas, or for the feedback of subband information for more than one subband for MIMO feedback modes 2, 3, 5 and 6, or for multi-BS feedback, the MS shall use a MAC control message. Dedicated resource allocation shall be allocated for MAC control message. In case of feedback for MIMO feedback modes 0, 1, 4 or 7, the MS shall feedback the CQI for FP0 if FPCT equals 1 or 4, or for FPk if FPCT is not equal to 1 or 4 3, where FPk is determined by k = floor(IDCell/255 256).

If MIMO_feedback_IE_type = 0b1, the AMS shall transmit using MIMO mode 0 if it has multiple transmit antennas, or using MIMO mode 1 with Mt=1 stream if it has a single transmit antenna. If MIMO_feedback_IE_type = 0b0, the AMS shall follows the MEF instruction in the IE.

Table 685 - Feedback Polling A-MAP IE

| Syntax | Size in bits | Notes |
| --- | --- | --- |
| A-MAP IE Type { | [4] | Feedback_Polling_IE |
| Allocation Duration (d) | 3 | The allocation is valid for 2(d–1) superframes starting from the superframe defined by allocation relevance. If d == 0b000, the pre-scheduled feedback header or MAC management message transmission is released. If d == 0b111, the pre-scheduled feedback header or MAC management message transmission shall be valid until the BS commands to release it. |
| If (d ==0b000){ |  |  |
| Resource Index | 11 | Confirmation of the resource index for a previously assigned persistent resource that has been deallocated 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| HFA | 3 | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } else if (d != 0b000){ |  |  |
| Dedicated UL allocation | 1 | 0b0: No dedicated UL resource is allocated. BS shall provide UL allocation for the MIMO feedback IE transmission through UL A-MAP IE at each designated transmitting frame defined by this IE. 0b1: Dedicated UL resource is included in this IE |
| If (Dedicated UL allocation ==0b1){ |  |  |
| ISizeOffset | 5 | Offset used to compute burst size index |
| Resource Index | 11 | 5 MHz: 0 in first 2 MSB bits + 9 bits for resource index 10 MHz: 11 bits for resource index 20 MHz: 11 bits for resource index Resource index includes location and allocation size |
| } |  |  |
| Period (p) | 3 | Transmit feedback header or MAC management message every 4p 2(p-1) frame. p=0b0 means no short term feedback |
| Long term transmission period (q) | 3 | Transmit correlation Matrix every 2(q-1) superframe. Long period puncture short period. q=0b0 means no long term feedback |
| MIMO_feedback_IE_type | 1 | - 0b0: feedback for single-BS MIMO oper - ation - 0b1: feedback for multi-BS MIMO operation |
| If (MIMO_feedback_IE_type == 0b0){ |  | - Single-BS MIMO feedback request |
| If (Dedicated UL allocation == 0b1){ |  |  |
| MEF | 1 | MIMO encoder format 0b0: SFBC 0b1: Vertical encoding with Mt = 2 if Nt=2 or 4, or Mt = 1 if Nt=1 - Non-adaptive precoding shall be used at the MS. Nt is the number of transmit antennas at the AMS. |
| } |  |  |
| Transmit_Correlation_Matrix | 1 | - 0b0: feedback of the transmit correlation matrix is indicated by CM - 0b1: feedback of the quantized BS transmit correlation matrix only - Transmit correlation matrix shall be feedback if CM = 0b1 |
| If (Transmit_Correlation_Matrix == 0b0 Period p>0b0){ |  | ABS requests AMS to feedback CQI And CSI for a specific MFM. |
| MaxMt | Variable 1 or 2 | Variable number of bits - depends on number of transmit antenna Nt - If Nt=2 (SU-MIMO and MU-MIMO): - 0b0: 1 - 0b1: 2 - If Nt=4 (SU-MIMO and MU-MIMO): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt=8 (SU-MIMO): - 0b000: 1 - 0b001: 2 - 0b011: 4 - 0b111: 8 - If Nt=8: (MU-MIMO): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 |
| MFM | 3 | - MIMO Feedback Mode for which the AMS shall feedback CQI and CSI to the ABS |
| If (MFM = 2, 3, 5, 6) { |  | - Feedback of CQI and CSI for localized resource units |
| Num_best_subbands | 3 | 0b000: report all subbands 0b001~0b111: Number of best subbands to report 1< Num_best_subbands ≤ Nsub |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | CL SU and MU MIMO |
| CM | [TBD] | [TBD] |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| } |  |  |
| If(MFM=0, 1, 2, 5){ |  |  |
| Measurement Method Indication | 1 | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| } |  |  |
| Else { |  | - Multi-BS MIMO feedback request |
| TRU | 2 | - Target RU indicating which RUs or which type of RU to work on for feedback 0b00: Latest best subbands reported for single BS MIMO 0b01: Whole bandwidth 0b10: FFR partition 0 0b11: boosted FFR partition |
| ICT | 2 | 0b00: PMI restriction for single-BS precoding; 0b01: PMI recommendation for single-BS precoding; 0b10: CL-MD for multi-BS precoding; 0b11: Co-MIMO for multi-BS precoding; |
| CS | 1 | 0b0: report PMI from the base codebook 0b1: report PMI from the codebook subset |
| N_multiBS_reports | 3 | N_multiBS_reports indicates the number of reports. |
| If (ICT = 0b10 or 0b11) { |  |  |
| CPI | 1 | Concatenating PMI Feedback indication; 0b0: feedback CPMI for N_multiBS_reports-1 diversity set members; 0b1: no feedback CPMI |
| If (ICT = 0b11) { |  |  |
| MaxUser | 2 | Maximum number of users supported in Co-MIMO in the same resource. 0b00: 2 users 0b01: 3 users 0b10: 4 users 0b11: reserved |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Reserved | 1 |  |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |

A 16 bit CRC is generated based on the contents of the Feedback Polling A-MAP IE. The CRC is masked by the Station ID.

-------------------------------  End of Text Proposal  ---------------------------------------------------

--------------------------------Start of the text #2 ----------------------------------------

Make the following changes in section 15.2.3.33, page 79, Table 700

15.2.3.33 AAI_SingleBS_MIMO_FBK

The AAI_SingleBS_MIMO_FBK message format is defined in Table 700. This message is used by AMS as a response to a Feedback Polling A-MAP IE requesting to feedback the subband information for MFM 2, 3, 5 or 6. It is also used for the feedback of the transmit correlation matrix when the ABS is equipped with 8 transmit antennas.

Table 700. AAI_SingleBS_MIMO_FBK message format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| AAI_SingleBS_MIMO_FBK_Message_Format { |  |  |
| Management Message Type = xx |  |  |
| If (Transmit_Correlation_Matrix == 1 Long term feedback period > 0b0){ |  | Only for 8Tx |
| For (i=1; i ≤ Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in Table 713 |
| For (j=i+1; j ≤ Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in Table 714 |
| } |  |  |
| } |  |  |
| }else (Period > 0b0) { |  |  |
| MFM_bitmap | 4 | Bitmap to indicate the MFMs for which the AMS is sending feedback. It shall be consistent with the MFM requested in a Feedback Polling IE. Only MFM 2, 3, 5 and 6 are allowed. |
| If (LSB #0 MFM_bitmap == 1){ |  | MFM 2 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | Nsub | Bitmap with Num_best_subbands 1’s |
| STC_Rate | 1 to 3 | STC_Rate for MFM 2, according to MaxMt |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband in bitmap |
| } |  |  |
| } |  |  |
| If (LSB #1 MFM_bitmap == 1){ |  | MFM 3 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | Nsub | Bitmap with Num_best_subbands 1’s |
| STC_Rate | 1 to 3 | STC_Rate for MFM 3, according to MaxMt |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband in bitmap |
| Subband PMI | 3 to 6 | PMI of m-th subband in bitmap, according to CM and CS Standard codebook is used when STC rate > 1, transformation codebook is used when STC rate = 1 |
| } |  |  |
| } |  |  |
| If (LSB #2 MFM_bitmap == 1){ |  | MFM 5 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | Nsub | Bitmap with Num_best_subbands 1’s |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband in bitmap |
| Stream index | 1 to 3 | Best stream index of m-th subband in bitmap, according to CS and MaxMt |
| } |  |  |
| } |  |  |
| If (LSB #3 MFM_bitmap == 1){ |  | MFM 6 as specified in Feedback Polling A-MAP IE |
| Best_subbands_bitmap | Nsub | Bitmap with Num_best_subbands 1’s |
| For (m=0; m < Num_best_subbands; m ++){ |  |  |
| Subband CQI | 4 | MCS of m-th subband in bitmap |
| Subband PMI | 3 to 6 | PMI of m-th subband in bitmap chosen from transformation codebook, according to CM and CS |
| } |  |  |
| } |  |  |
| If ((LSB #1 MFM_bitmap == 1) or (LSB #3 MFM_bitmap == 1)){ |  | MFM 3 or MFM 6 |
| If (CM == 0b1){ [TBD] |  |  |
| For (i=1; i ≤ Nt; i++){ |  |  |
| i-th diagonal entry of correlation matrix | 1 | As defined in Table 713 |
| For (j=i+1; j ≤ Nt; j++){ |  |  |
| (i,j)-th entry of correlation matrix | 4 | As defined in Table 714 |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |