| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text change for Feedback Allocation A-MAP IE (15.3.6.5.2.11) for the IEEE 802.16m/D2 |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | - Huaning Niu - Yuan Zhu - Alexei Davydov Intel Corporation | huaning.niu@intel.com yuan.y.zhu@intel.com |
| Re: | Call for comments for Latter Ballot 30a |  |
| Abstract | Text change proposal for feedback allocation A-MAP IE and related MIMO text |  |
| Purpose | To be discussed and adopted by TGm for D3 draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text for Feedback Allocation A-MAP IE (15.3.6.5.2.11) for the IEEE 802.16m/D2

- Huaning Niu, Yuan Zhu, Alexei Davydov, Yang-seok Choi

- Intel Corporation

- Introduction

In this contribution, we modify IEEE 802.16m/D2 [1] to support CQI feedback for sounding based beam forming, using MFM 0 with MaxMt = 0. In addition, we define that transformation codebook shall be used in single BS MIMO rank-1 transmission.

This contribution also proposes some clarification text to be applied to IEEE 802.16m/D2.

- References

[1] IEEE 802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems Advanced Air Interface”, Oct 2009

- Proposed Text

<Modify text in section 15.3.6.5.2.11 Feedback Allocation A-MAP IE>

--------------------------------Start of the text #1 ----------------------------------------

Make the following changes in section 15.3.6.5.2.11, page 392, Table 818

Table 818 -Feedback Allocation A-MAP IE

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| Feedback-Allocation-A-MAP_IE() { |  |  |
| A-MAP IE Type | [4] | Feedback Allocation A-MAP IE = 0b0010 |
| Channel Index | Variable 8 | Feedback channel index within the UL fast feedback control resource region (Dependent on LFB,FPi defined in 15.3.8.3.3.2) |
| Short-term feedback Period (p) | [3] | A feedback is transmitted on the FBCH every 2p frames |
| Long-term Feedback period (q) | [2] | A long-term feedback is transmitted on the FBCH every 2q short-term feedback opportunities. If q==0b00, long-term feedback is not used. |
| Frame offset | [3] | The AMS starts reporting at the frame of which the number has the same 3 LSB as the specified frame offset. If the current frame is specified, the AMS should start reporting in eight frames. |
| Allocation Duration (d) | [3] | An FBCH is transmitted on the FBCH channels indexed by Channel index for 8×2d frames. If d ==0b000, the FBCH is deallocated. If d == 0b111, the AMS should shall report until the ABS command for the AMS to stop. |
| ACK allocation flag | [1] | Indicated if one ACK channel is allocated |
| If (ACK allocation flag ==0b01){ |  |  |
| HFA | [3] | HARQ feedback channel allocation for Feedback Channel De-allocation confirmation |
| } |  |  |
| MaxMt | Variable [1-2] | Variable number of bits - depends on number of transmit antenna Nt - If Nt==2 (SU-MIMO and MU-MIMO): - 0b0: 1 - 0b1: 2 - If Nt==4 (SU-MIMO and MU-MIMO): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - If Nt==8 (SU-MIMO): - 0b000: 1 - 0b001: 2 - 0b011: 4 - 0b111: 8 - If Nt==8: (MU-MIMO): - 0b00: 1 - 0b01: 2 - 0b10: 3 - 0b11: 4 - MaxMt == 0 in sounding based CL SU MU MIMO |
| MFM | [3] | - MIMO Feedback Mode as defined in Table 671831 |
| If (MFM == 2, 3, 5, 6) { |  |  |
| Feedback Format | [2] |  |
| } |  |  |
| If (MFM == 0,1,4,7) { |  |  |
| FPI | [2] | Frequency partition indication: ABS indicate AMS to send wideband CQI and STC rate of the frequency partition and reuse factor in the future: 0b00: Frequency partition index 0 0b01: Frequency partition index 1 0b10: Frequency partition index 2 0b11: Frequency partition index 3 |
| } |  |  |
| If (MFM == 0,1 && long term feedback q!=0b00) { |  |  |
| Long term FPI | [2] | Frequency partition indication: ABS indicate AMS to send wideband CQI and STC rate for the second frequency partition using long term feedback: 0b00: Frequency partition index 0 0b01: Frequency partition index 1 0b10: Frequency partition index 2 0b11: Frequency partition index 3 |
| } |  |  |
| If (MFM == 3,4,6,7) { |  | CL SU and MU MIMO |
| CM | [2] | Codebook Feedback Mode and Codebook Coordination Enable 0b00: standard with CCE disabled 0b01: transformation with CCE disabled 0b00: standard when STC rate > 1, transformation when STC rate = 1, with codebook coordination disabled 0b10 01: differential with CCE codebook coordination disabled 0b11: standard with CCE codebook coordination enabled |
| CS | [1] | Codebook subset |
| } |  |  |
| If(MFM==0, 1, 2, 5){ |  |  |
| Measurement Method Indication | [1] | 0b0: Use the midamble for CQI measurements 0b1: Use pilots in OL region with MaxMt streams for CQI measurements |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |
| MCRC | [16] | 16 bit CRC masked by Station ID |
| } |  |  |

A 16 bit CRC is generated based on the contents of the Feedback Allocation A-MAP IE. The CRC is masked by the Station ID.

------------------------------- End of text # 1  -------------------------------------------

--------------------------------Start of the text #2 ----------------------------------------

Make the following changes in section 15.3.7.2.5.4, page 413, Table 831

15.3.7.2.5.4 MIMO feedback modes

Each MIMO transmission mode can be supported by one or several MIMO feedback modes. When allocating a feedback channel, the MIMO feedback mode shall be indicated to the AMS, and the AMS will feedback information accordingly.

The description of MIMO feedback modes and corresponding supported MIMO transmission modes is shown in Table 831. The detailed description of feedback and AMS processing are in the following subsections.

The feedback of the quantized wideband correlation matrix shall be requested by the ABS for operation with transformation codebook-based feedback mode. The ABS may request the feedback of the quantized wideband correlation matrix independently of the MIMO feedback mode requested in the FBCH_Alloc_IE. The quantized wideband correlation matrix may be used for wideband beamforming.

MIMO feedback mode 0 is used for the OL-SU SFBC and SM adaptation in diversity permutation. The AMS estimates the wideband CQI for both SFBC and SM, and reports the CQI and STC Rate. STC Rate 1 means SFBC with precoding and STC Rate 2 means rank-2 SM with precoding. MIMO feedback mode 0 is also used for CQI feedback for sounding based beamforming defined in session 15.3.7.2.5.7. The AMS shall estimate the wideband CQI for SFBC mode, and reports the CQI.

MIMO feedback mode 1 is used for the OL-SU SM with rank adaptation in diversity permutation. The transmission rank is determined by the STC Rate. STC Rate 1 means the rank-1 precoding.

MIMO feedback mode 2 is used for the OL-SU SM in localized permutation for frequency selective scheduling. The STC Rate indicates the preferred number of MIMO streams for SM. The subband CQI shall correspond to the selected rank.

Table 831—MIMO feedback modes

| MIMO Feedback Mode | Description and type of RU | Feedback content | Supported MIMO transmission mode outside the OL region (when Measurement Method Indication = 0b0) | Supported MIMO transmission mode inside the OL region (when Measurement Method Indication = 0b1) |
| --- | --- | --- | --- | --- |
| 0 | OL SU MIMO SFBC/SM (Diversity: DLRU, NLRU) Sounding based CL SU MU MIMO | 1. STC Rate 2. Wideband CQI | MIMO mode 0 and MIMO mode 1. Flexible adaptation between the two modes STC Rate = 1: SFBC CQI 2 <= STC Rate <= 4: SM CQI In DLRU: Mt=2 for SM. In Miniband based SLRU: Mt >= 2 for SM For sounding based CL SU MU MIMO, STC Rate = 1: SFBC CQI | MIMO mode 0 and MIMO mode 1. Flexible adaptation between the two modes STC Rate = 1: SFBC CQI STC Rate = 2: SM CQI In DLRU only. |
| 1 | OL SU MIMO SM (Diversity: NLRU) | 1. STC Rate 2. Wideband CQI | MIMO mode 1 1 <= STC <= 4 | MIMO mode 5 STC Rate = 1/2 |
| 2 | OL SU MIMO SM (localized: SLRU) | 1. STC Rate 2. Subband CQI 3. Subband Selection | MIMO mode 1 1 <= STC Rate <= 8 | MIMO mode 5 STC Rate = 1/2 |
| 3 | CL SU MIMO (localized: SLRU) | 1. STC Rate 2. Subband CQI 3. Subband PMI 4.Subband selection 5. Wideband correlation matrix | MIMO mode 2 1 <= STC Rate <= 8 | N/A |
| 4 | CL SU/MU MIMO (Diversity: NLRU) | 1. Wideband CQI 2. Wideband PMI 3. Wideband correlation matrix | MIMO mode 2 (Mt=1) | N/A |
| 5 | OL MU MIMO (localized: SLRU) | 1. Subband CQI 2. Subband Selection 3. MIMO stream indicator | MIMO mode 3 | MIMO mode 3 |
| 6 | CL MU MIMO (localized: SLRU) | 1. Subband CQI 2. Subband PMI 3. Subband Selection [4. Wideband PMI] 5. Wideband correlation matrix | MIMO mode 4 | N/A |
| 7 | CL MU MIMO (Diversity: SLRU, NLRU) | 1. Wideband CQI 2. Wideband PMI 3. Wideband correlation matrix | MIMO mode 4 | N/A |

MIMO feedback mode 3 is used for the CL-SU SM in localized permutation for frequency selective scheduling. The STC Rate indicates the preferred number of MIMO streams for SM. The subband CQI shall correspond to the selected rank.

The MIMO feedback mode 4 is used for the CL SU MIMO using wideband beamforming with rank 1. In this mode, AMS shall feedback the wideband CQI. The wideband CQI shall be estimated at the AMS assuming short-term or long-term precoding at the ABS, according to the feedback period. The channel state information may be obtained at the ABS via the feedback of the correlation matrix, or via the feedback of the wideband PMI.

The MIMO feedback mode 5 is used for OL MU MIMO in localized permutation with frequency selective scheduling. In the mode, AMS shall feedback the subband selection, MIMO stream indicator and the corresponding CQI.

The MIMO feedback mode 6 is used for CL MU MIMO in localized permutation with frequency selective scheduling. In the mode, AMS shall feedback the subband selection, corresponding CQI and subband PMI. The subband CQI refers to the CQI of the best PMI in the subband. Rank-1 base codebook (or its subset) is used to estimate the PMI in one subband.

The MIMO feedback mode 7 is used for CL MU MIMO in diversity permutation using wideband beamforming MU MIMO. In this mode, AMS shall feedback the wideband CQI. The wideband CQI shall be estimated at the AMS assuming short-term or long-term precoding at the ABS, according to the feedback period. The channel state information may be obtained at the ABS via the feedback of the correlation matrix. or via the feedback of the wideband PMI

--------------------------------Start of the text #3 ----------------------------------------

Make the following changes in section 15.3.7.2.5.5, page 416, Table 832

Table 832 – DL MIMO control parameters

| Parameter | Description | Value | DL Control channel | Notes |
| --- | --- | --- | --- | --- |
| Broadcast Information |  |  |  |  |
| Nt | Number of transmit antennas at the BS | 2, 4, 8 | SFH (system information) | Indicated in S-SFH (system information) SP3 IE |
| OL_Region | OL MIMO region, which signaling is used to indicate MS where is the predefined OL MIMO region and number of streams (1 or 2) | OL-Region-ON (1 bit): Signal the existence of OL region. OL-Rank1-Config (3 bit): to signal the combination of sub-band and mini-band in OL region type 1. Refer to table 691. SB-OL-Region-2-Size (4 bit) : signal the number of sub-bands in OL region type 2. | Additional broadcast information | OL region signaling is SFH-SP3 or ABI (TBD) Indicated in AAI_SCD Message |
| BC_SI | Rank-1 base codebook subset indication for interference mitigation with PMI coordination | BitMAP: 8 bits if Nt=2 16 bits if Nt=4, 8 | Additional broadcast information | Rank-1 codebook element restriction/recommendation information It shall be ignored if CM = 0b00, 0b01 or 0b10 Indicated in AAI_DL_IM Message |
| Unicast Information |  |  |  |  |
| MEF | MIMO encoder format | SFBC Vertical encoding Horizontal encoding | A-MAP IE | MIMO encoder format. |
| Mt | Number of streams in transmission | 1 to 8 |  | Number of streams in the transmission. |
| SI | Index of allocated pilot stream | 1 to 4 |  | SI shall be indicated if MEF is HE |
| Feedback Allocation IEs |  |  |  |  |
| MFM | MIMO feedback mode | Refer to Table 691 |  | To decide the feedback content and related MS processing |
| MaxMt | Maximum number of streams | If MFM indicates a SU feedback mode: 1, 2, 4 or 8. If MFM indicates a MU feedback mode: 1, 2, 3, or 4. |  | If MFM indicates a SU feedback mode: the maximum number of streams scheduled for each user If MFM indicates a MU feedback mode: the maximum number of users scheduled on each RU |
| CS indication | Codebook subset type for CL MIMO modes 2 and 4 | Base codebook or codebook subset |  | Depending on the MFM and CS indication, the MS shall feedback a PMI from the SU or MU base codebook, or from a subset of the SU or MU of the base codebook. |
| CM | Codebook feedback mode for CL MIMO modes 2 and 4 | 0b00: basestandard mode with codebook coordination disabled Standard codebook when STC rate > 1, transformation when STC rate = 1, with codebook coordination disabled 0b01: differential with codebook coordination disabled transformation mode with codebook coordination disabled 0b10: differential mode with codebook coordination disabled 0b11: standard mode with codebook coordination enabled |  | In codebook-based feedback mode: Codebook coordination disabled: MS finds PMI within whole codebook type entry indicated by CS indication Codebook coordination enabled: MS shall find the rate-1 rank-1 PMI within codebook entries indicated by BC_SI and CS indication |