| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for sub-band indexing (15.3.9.3.1.4) for the IEEE 802.16m/D2 |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Huaning Niu, Rongzhen Yang, Hujun Yin, Yang-seok Choi Intel Corporation Mohammad (Amir) Khojastepour NEC Labs | E-mail: huaning.niu@intel.com amir@nec-labs.com |
| Re: | Proposed text changes to P802.16m/D2 15.3.9.3.1.4 |  |
| Abstract | Text proposal for sub-band indexing |  |
| Purpose | To be discussed and adopted by TGm for D3 draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for sub-band indexing (15.3.9.3.1.4) for the IEEE 802.16m/D2

Huaning Niu, Rongzhen Yang, Hujun Yin, Yang-seok Choi

Intel Corporation

Mohammad (Amir) Khojastepour

NEC Labs

- Introduction

In current 802.16m D2, the number of bits to signal the best-M sub-band position is defined in the feedback format. The indexing equation is missing. This contribution defines the sub-band indexing for best-M feedback in current 802.16m D2.

In MFM 2, 3, 5, 6, AMS shall report the M selected subbands using an index r defined as

r=iMSii,

Where the set sii=1M, (0≤si≤N−1, si<si+1) contains the M sorted subband indices, xy=xyx≥y0x<y is the extended binomial coefficient, resulting in unique index r∈0,⋯,NM−1.

When ABS receives the report through SFBCH, ABS may calculate the subband location information sii=1M by the following reverse indexing algorithm:

Algorithm Initialization:

1) Input Parameters: N, M, r;

2) r=NM−1−r; i = M; v(1~M) = 0; BaseValue = 0;

Algorithm Body:

Repeat the following steps 1,2,3,  if  (i > 0) and (r >0):

- calculate IncreaseValue:

IncreaseValue=N−M+1−j=i+1Mvj−vi+i−2

- ThresholdValue = BaseValue + IncreaseValue;

- If r >= ThresholdValue then

vi=vi+1;

BaseValue = ThresholdValue;

else

r = r – BaseValue;

i=i−1;

BaseValue = 0;

end

Algorithm Output:

After algorithm body, we can get the vector: vMvM−1vM−2......v1

Then we can get:

v0=N−M−i=1Mvi

si=i−1+j=0ivj; i=1~M;

The set sii=1M, (0≤si≤N−1, si<si+1) contains the M sorted subband indices

- References

[1] Draft Amendment, P802.16m/D2, Oct 2009.

[Note to Editor: Suggested Remedy - add one new subsection 15.3.9.3.1.4 in page 502 as follows]

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

# 15.3.9.3.1.4 Representation of Subband Indices

For AMS selected subband feedback, the MS shall report the M selected subbands indices using a combinatorial index r defined as

r=i=1MSii,

Where

The set sii=1M, (0≤si≤N−1, si<si+1) contains the M selected subband indices,

xy=xyx≥y0x<y is the extended binomial coefficient

Resulting in unique label in the range: r∈0,⋯,NM−1.

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------