| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network Coding based E-MBS Retransmission (15.9.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Hung-Yu Wei Ching-Chun Chou Cheng-Chih Chao National Taiwan University Guosen Yue, Xiaodong Wang, Sampath Rangarajan, Mohammad A. (Amir) Khojastepour NEC Labs America Chun-Yen Wang Frank Ren ITRI | E-mail: hywei@cc.ee.ntu.edu.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> E-mail: yueg@nec-labs.com amir@nec-labs.com E-Mail: ChunYen@itri.org.tw frank_ren@itri.org.tw |
| Re: | Proposal to standard draft P802.16m/D2, on E-MBS Retransmission |  |
| Abstract | Network coding E-MBS retransmission mechanisms and corresponding packet header format are presented. The retransmission may be triggered with or without feedback. |  |
| Purpose | For discussion and approval by IEEE 802.16m TG |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Network Coding based E-MBS Retransmission

Hung-Yu Wei, Ching-Chun Chou, Cheng-Chih Chao

National Taiwan University

Guosen Yue, Xiaodong Wang, Sampath Rangarajan, Mohammad A. (Amir) Khojastepour

NEC Labs America

Chun-Yen Wang, Frank Ren

ITRI

# 1 Introduction

SDD specifies the E-MBS operation with network coding based retransmission. In Section 13.3.3 of SDD, it states “An ABS may use a network-coding based retransmission scheme that does not require a feedback channel.”

This document conforms with SDD specification and describes the proposed AWD text on network coding based E-MBS retransmission without feedback. This harmonized document includes text from contributions C80216m-09_1682, C80216m-09_1684, and C80216m-09_2111.

# 2 Simulation Results

We propose the harmonized scheme in this document. We compare different schemes here, including NTU (PRNC in C80216m-09_1682), NEC (C80216m-09_2111), Motorola (IEEE C802.16m-09/0072r1), without network coding, and the harmonized scheme (the proposed scheme in this document).

The simulation settings are as follows:

- One ABS broadcast to 10 AMSs.

- In each trial, the ABS transmits L original packets. L = 25, 50, and 100.

- We set different number of retransmission packets, from 1 to 25.

- The packet loss rate is 0.05

We use two metric for simulation:

- Received packets: the total number of packets decoded by 10 AMSs.

- All perfect ratio: a trial is successful if all AMSs decode all L packets. The all perfect ratio is the ratio of successful trials over total number of trials.

Fig. 1 shows the result of L= 25. We can see that harmonized scheme performs better than Motorola scheme and without network coding scheme (no NC). And fig. 2 shows the result that in all perfect ratio, harmonized scheme is also the best, and the ratio is much higher than Motorola and no NC.

Fig. 3 and fig. 4 are results of L = 50. Fig. 5 and fig. 6 are results of L = 100. The harmonized scheme also performs well.

We also simulate under different number of AMSs. And the harmonized scheme also performs well.

# Proposed Text to P802.16m Amendment Working Document

Add the following text into the amendment draft:

---------------------------------------------------Start of the proposed text------------------------------------------------------

# 15. 9.2.4 E-MBS Retransmission 15. 9.2.4. 1 Network Coding based E-MBS Retransmission

# 15. 9.2.4. 1.1 Network Coding based E-MBS Retransmission without Feedback

For E-MBS retransmission without feedback, the number of original packets and redundancies should be specified prior to the E-MBS retransmission. The pre-defined codebook is used to determine the coding coefficient for network coding transmission. An ABS first transmits L packets, P1,…, PL. After the L packets are transmitted, Q packet-slots are allocated for retransmission. The packets for retransmission R1,…,RQ are binary packet encoded from the original packet sequence P1,…, PL based on the following procedures. In the E-MBS network coding subheader, Mode should be set to 01, PKT_N should be set to L, and PKT_T should be set to (L+Q).

# 15. 9.2.4. 2 Codebook for Network Coding Based E-MBS Retransmission

The codebook C is an L x T matrix, where L is the number of original packets, and T is the codebook size.

C=C1,1C1,2C1,3...C1,LC2,1C2,2C2,3...C2,LC3,1C3,2C3,3...C3,L...CT,1CT,2CT,3...CT,L

Ci,j : i represents the code index, j represents the index of the original packets

Ci,j {0, 1}

Denote the original packets are P1, P2, P3, … , PL. One row of codebook C represents one set of coding coefficients. If Ci,j is 1, it means the coded packet i contains Pj. One row of coefficients of a codebook matrix represents one set of coding coefficient. For example, the 2nd set of coding coefficient is the 1 by N matrix [C2,1 C2,2 C2,3  … C2,L ]. The encoded packet is the XOR-based linear combination of the (P1C2,1 +P2C2,2 +P3C2,3 +… +PLC2,L). The coding coefficient index in the packet header of the encoded packet is set to 2, since the 2nd set of coding coefficient in the codebook is used.

An ABS first transmits L packets, P1,…, PL. Therefore, for row 1 to row L, Ci,j = 1 if i = j.

For row L+1 to L+M, the codebook generating function is as follows. If L > 50, set M=5, else set M=4.

- Group the packets P1,…, PL to the packet sets 0,…,2M-1 in order based on the given size of n, ln=|n|, n=0,…, 2M-1.

- Then retransmission packets R1, …, RM are generated as follows.

Rm=Pj∈Vm⊕Pj, 	with Vm=nbin(m)=1Ωn

where  denotes binary summation and nbin (m) denotes the m-th bit of binary representation of n. In other words, Rm is obtained by binary XORing of the data packet Pj if Pj  n and the m-th bit of binary representation of n. Therefore the codebook coefficients Cij for i=L+1,…,L+M are given by

Ci,j=1,    if Pj∈Ωn, nbin(m)=1, m=i-L 0,   otherwise

The code definition {ln},can be obtained from {l'n} as follows

ln=ln'−1,for n=20,21,...,2M−1,ln',other n,

where ln’=|n’| , n=02M−1ln'=L+M, and {n’} are the packet sets including the retransmission packets, defined as

Ωn'=Ωn∪Rm,for n=2m−1,Ωn,other n.

To obtain the code {ln’},

1) If L=25, 50, 75, or 100, {ln’} and M are obtained from Table x.

| L | T | M | { ln’ : ln’=¦n’ ¦, n=0,…2M-1} |
| --- | --- | --- | --- |
| 100 | 37 | 5 | {33 6 6 3 6 3 3 1 5 3 3 1 3 1 1 1 5 3 3 2 3 1 1 0 3 1 1 0 2 0 0 1} |
| 75 | 37 | 5 | {5 6 6 3 6 3 3 1 6 3 3 1 3 1 1 1 5 3 3 2 3 2 1 0 3 1 2 0 2 0 0 1 } |
| 50 | 37 | 4 | {0 6 6 4 6 4 3 2 6 3 3 2 4 2 2 1}; |
| 25 | 37 | 4 | {0 3 3 2 3 2 2 1 3 2 2 1 2 1 1 1} |

Table x: Code graph definition for generating first M retransmission packets.

2) For other L,

Find the minimum L* in {25,50,75,100} that is greater than L and the corresponding codebook set defined in Table x.

Then based on the following process, we remove the bit one by one from the set until n=02M−1ln'=L+M:

- Find maximum ln’ in the updated set and set ln’ = ln’ -1. If there are several ln’ with the same maximum value, we set ln’= ln’ -1 based on the descending order of the Hamming weight of the binary expansion of n. Repeat this until n=02M−1ln'=L+M.

For row i > L+M in the codebook,

The parameter a and k are positive integersa≥2,0≤k≤a−2, such that i=(L+M+1)+(a−1)(a−2)2+k

Ci,j=1,    if j mod a = k 0,   otherwise

For example,

a = 2, k = 0, CL+M+1,j = 1 if j mod 2 = 0

a = 3, k = 0, CL+M+2,j = 1 if j mod 3 =0

a = 3, k = 1, CL+M+3,j = 1 if j mod 3 =1

a = 4, k = 0, CL+M+4,j = 1 if j mod 4 =0

# 15. 9.2.4. 3 Packet Header Format for Network Coding E-MBS Retransmission

| Syntax | Size | Notes |
| --- | --- | --- |
| Mode | 2-bit | Mode = 00 : uncoded Mode = 01 : network coding without feedback Mode = 10 : network coding with limited feedback Mode = 11 : network coding with common feedback |
| PKT_T_Flag | 1-bit | This bit indicates that PKT_T is included in the header. PKT_T_Flag=0: PKT_T is NOT included. PKT_T_Flag=1: PKT_T is included. |
| Feedback_Info_Flag | 1-bit | This bit indicates the presence of Feedback Information. Feedback_Info_Flag=0: No Feedback Information is contained. Feedback_Info_Flag=1: There will be Feedback Information included in the header. |
| Index | 8-bit | This index indicates the row of coding coefficient in codebook C, if network coding is applied. If the packet is uncoded, this is the index of the original packet. |
| Batch_Sequence | TBD | Packets are encoded and decoded in the unit of batch. The batch sequence should be sent along with the header. It specifies the batch ID. Only packet within the same batch may be decoded using linear operation. |
| PKT_N | TBD | The number of original packet. |
| PKT_T(Optional) | TBD | The number of total transmitted packets in this batch. PKT_T is skipped if Feedback_Info_Flag |
| Feedback Information (Optional) | TBD | Feedback Information filed is skipped if Feedback_Info_Flag=0 Specify the operation taken for the feedback signal from AMS. 3 kinds of messages may be carried here. Feedback_Threshold: Threshold for the number of error packet to trigger AMS feedback Termination_of_current_batch: Indicate the end of transmission for current batch. Next downlink radio resource block will be scheduled for packets from next batch. Begin_of_Retransmission: The retransmission for current batch will be scheduled for next radio resource block in E-MBS DL transmission. |

Table Y: Network coding extended header format

---------------------------------------------------End of the proposed text-------------------------------------------------------

# References

[1] IEEE 802.16m-09_0034.pdf, SDD text section 13.3.3

[2] IEEE C802.16m-09/1682, Proposed Text for IEEE P802.16m/D1: Network Coding based E-MBS Retransmission (E-MBS) (Ching-Chun Chou, Hung-Yu Wei, Cheng-Chih Chao, Chun-Yen Wang, Frank Ren; 2009-08-30)

[3] IEEE C802.16m-09/1684,Proposed Text for IEEE P802.16m/D1: Network Coding Codebook Design for E-MBS (E-MBS) (Cheng-Chih Chao, Hung-Yu Wei, Ching-Chun Chou, Chun-Yen Wang, Frank Ren; 2009-08-30)

[4] IEEE C802.16m-09/2111, Network Coding Based Retransmission Scheme in E-MBS for IEEE 802.16m AWD (G. Yue, X. Wang, S. Rangarajan, A. Khojasterpour; 2009-08-30)

[5] IEEE C802.16m-09/0072r1, Network coding-based retransmission schemes for E-MBS with and without MS feedback (Hua Xu, Steven Xu, Naveen Arulselvan, Suresh Kalyanasundaram; 2009-01-13)