| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on coverage loss (section 15.2.x and 15.2.3.x) |  |
| Date Submitted | 2009-11-18 |  |
| Source(s) | Chunmei Liu, Masoud Olfat Clearwire Kelvin Chou MediaTek Inc. Stavros Tzavidas Joe Schumacher Motorola Inc. Muthaiah Venkatachalam, Xiangying Yang, Shantidev Mohanty Intel Corporation Zheng Yan-Xiu Inuk Jung | Chunmei.liu@clearwire.com Masoud Olfat@clearwire.com Kelvin.Chou@mediatek.com Stavros.tzavidas@motorola.com j.schumacher@motorola.com muthaiah.venkatachalam@intel.com, xiangying.yang@intel.com, shantidev.mohanty@intel.com zhengyanxiu@itri.org.tw cooper@lge.com |
| Re: | Proposed text on coverage loss |  |
| Abstract | Proposed text on coverage loss in the draft P802.16m amendment |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Coverage Loss

Chunmei Liu and Masoud Olfat

Clearwire

Kelvin Chou

MediaTek Inc.

Stavros Tzavidas, Joe Schumacher

Motorola Inc.

Muthaiah Venkatachalam, Xiangying Yang, Shantidev Mohanty

Intel Corporation

Zheng Yan-Xiu

ITRI

# Introduction

Coverage losses are common in practical network deployments. The device may lose signal temporarily due to various reasons, such as entering into an area without coverage, fading, etc.  According to the current spec, upon a coverage loss, the MS will lose PHY synchronization or DL/UL synchronization. In addition, if the periodic ranging procedure is conducted, the ranging procedure may fail.

However, there is a lack of sufficient specs for addressing this problem at MAC layer. First, the standard does not synchronize the network and the MS regarding each other’s state, including link status and MS’s context retained at the network. Actually there is no mechanism specified for the BS to detect an MS’s coverage loss.

In addition, the standard does not specify the network’s behavior after an MS’s coverage loss either.  Note that in practical deployments, it may be desirable for the network to release MS’s context in order to free resource occupied.

Furthermore, there is ambiguity in the previous standards, which are the base of 16m AWD, on coverage recovery procedure after the MS comes back into coverage, and this ambiguity may lead to significant application performance downgrade. For example, in 802-16 2009, after incurring a coverage loss, a MS will scan for a new channel, or reinitializes MAC.  However, the spec does not clearly define what follows channel scanning and what it means by “reinitializes MAC”. In C80216maint-09_0014r1.doc, after a coverage loss, a MS may perform initial network entry or network reentry.

The proposed text below addresses this coverage loss problem with the necessary clarity by providing solutions to maintain and synchronize state information between a MS and the network, and coverage loss recovery procedure.

# Proposed Amendment text

Black text: existing text in AWD (P802.16m/D2)

Red Strikethrough text: deleted text

Blue text: new text

[Insert the below new section]

15.2.x Coverage loss

An AMS may lose signal temporarily due to various reasons, such as entering into an area without coverage and fading. A coverage loss refers to such a situation.

15.2.x.1 Coverage loss detection at ABS and ABS’s behavior

For each AMS, the ABS shall maintain a timer called active_ABS_timer.  The initial value of the active_ABS_timer is determined through AAI-REG-REQ/AAI-REG-RSP handshake during initial network entry procedure.  The timer starts upon the completion of the AAI-REG-REQ/AAI-REG-RSP handshake during initial network entry.

Upon expiration of the active_ABS_timer, the ABS shall send an AAI-KEEP-ALV message to the AMS which shall include FEH with polling bit set to 1. The ABS shall allocate uplink bandwidth to the AMS for the AMS to respond to the AAI-KEEP-ALV message. Upon receiving AAI-KEEP-ALV message, the AMS shall respond by sending AAI-MSG-ACK (see section 15.2.3.3). Upon reception of AAI-MSG-ACK, the ABS shall restart the active ABS timer.

If the ABS does not receive AAI-MSG-ACK as expected, the ABS shall send unsolicited AAI-RNG-RSP message to request the AMS to perform ranging using periodic ranging codes by setting the Ranging Request bit to one. Upon receiving an AAI-RNG-CFM message with AMS’s STID, which indicates a successful periodic ranging initiated by this unsolicited AAI-RNG-RSP, the ABS shall respond with AAI-MSG-ACK and restart the active_ABS_timer. If the ABS does not receive the AAI-RNG-CFM message upon the expiration of the retry timer for the AAI-RNG-RSP message, the ABS resends the AAI-RNG-RSP message until its retry limit is reached.

Upon exhausted AAI-RNG-RSP retries, the ABS shall start the Resource Retain Timer. Upon expiration of the Resource Retain Timer, the ABS considers that the AMS is not in the network anymore, and releases AMS’s dynamic context and moves AMS’s static context to the network entity that stores AMS’s context.

If the ABS receives backbone context request for the AMS after expiration of the active_ABS_timer, the ABS considers that the AMS is performing HO.  Accordingly, the ABS cancels the above procedure and restarts the Resource Retain Timer.

In case of a coverage loss during HO, the ABS is able to detect an AMS’s coverage loss by either receiving a HO fail notification via backhaul, without any further response from AMS, or not receiving a HO completion notification within the Ranging_Initiation_Deadline via backhaul. In either case, the ABS shall follow the above coverage loss procedure, where the ABS shall send an AAI-KEEP-ALV message to the AMS which shall include FEH with polling bit set to 1. The ABS shall allocate uplink bandwidth to the AMS for the AMS to respond to the AAI-KEEP-ALV message. Upon receiving AAI-KEEP-ALV message, the AMS shall respond by sending AAI-MSG-ACK (see section 15.2.3.3). Upon reception of AAI-MSG-ACK, the ABS shall restart the active ABS timer.

15.2.x.2 Coverage loss detection at AMS and AMS’s behavior

The AMS can detect a coverage loss by losing PHY synchronization or DL synchronization or UL synchronization. In addition, if the periodic ranging procedure is conducted, the AMS can detect a loss by a failure ranging.

Upon receiving an AAI-KEEP-ALV message that includes FEH with polling bit set to 1, the AMS shall respond by sending AAI-MSG-ACK.

Upon receiving an unsolicited AAI-RNG-RSP message with Ranging Request bit to be one, the AMS shall perform periodic ranging. Upon successful ranging initiated by an unsolicited AAI-RNG-RSP message with Ranging Request bit to be one, indicated by receiving an AAI-RNG-ACK message with a success status and corresponding periodic ranging code, the AMS shall request bandwidth and send back a AAI-RNG-CFM message that includes its STID and FEH with polling bit set to 1, which requests the ABS to respond with AAI-MSG-ACK. If the AMS does not receive the AAI-MSG-ACK message upon the expiration of the retry timer for the AAI-RNG-CFM message, the AMS resends the AAI-RNG-CFM message until its retry limit is reached. Upon exhausted AAI-RNG-CFM retries, the AMS considers that the ABS considers that the AMS is not with the ABS anymore, and the AMS shall perform coverage loss recovery procedure as indicated in Section 15.2.x.3.

15.2.x.3 Coverage loss recovery procedure

Upon detection of a coverage loss, the AMS scans for a new channel. After achieving PHY synchronization and DL synchronization with the discovered ABS, which could be its previous serving ABS before the coverage loss, the AMS performs network reentry as indicated below.

During the network re-entry, the AMS shall perform CDMA ranging using HO ranging codes. Upon the discovered ABS’s sending AAI-RNG-ACK with success status, the ABS shall provide UL BW allocation. When receiving an UL BW allocation, the AMS shall send the AAI_RNG-REQ message, and set bit#7 in the Ranging Purpose Indication parameter, indicating that this is a network reentry after a coverage loss. If the AMS shares valid security context, then the AMS shall include its previous MSID* in AAI-RNG-REQ protected with a CMAC derived from new AK. If the discovered ABS is the AMS’s previous serving ABS, the AMS should not include the serving BSID parameter in the AAI-RNG-REQ message; otherwise, the AMS shall include the serving BSID parameter and set it to be the BSID of its previous serving ABS.

After receiving the AAI-RNG-REQ message, the discovered ABS identifies AMS’s reentry attempt after a coverage loss and checks its context availability.  The discovered ABS may request AMS’s context over backbone network which is beyond the scope of this standard. Based on AMS’s relevant context retained at the network, the target ABS shall place in AAI-RNG-RSP an HO Process Optimization parameter indicating which reentry management messages may be omitted. The AMS shall complete the processing of all indicated messages before entering normal operation with target ABS.

[Insert the below new section]

15.2.6.31 AAI-KEEP-ALV

The ABS shall send the AAI-KEEP-ALV message upon expiration of the active_ABS_timer.  It shall include the following parameters:

- STID of the AMS.

- CMAC tuple

Additionally, the AAI-KEEP-ALV message shall always include an FEH with the polling bit set to 1.

15.2.6.32 AAI-RNG-CFM

The AMS shall send the AAI-RNG-CFM message to the ABS upon successful ranging initiated by an unsolicited AAI-RNG-RSP message with the Ranging Request bit to be one. It shall include the below parameters:

- STID of the AMS.

Additionally, the AAI- RNG-CFM message shall always include an FEH with the polling bit set to 1.

[Change some entries of Table 674 in 15.2.3.1, as follows]

Table 674—parameters for AAI_ RNG-REQ

| Ranging Purpose Indication | The presence of this item in the message indicates the following AMS action: If Bit#0 is set to 1, it indicates that the AMS is currently attempting HO reentry, or, in combination with a Paging Controller ID, indicates that the MS is attempting network reentry from idle mode to the BS. In this case, Bit#1 shall be 0.. If Bit#1 is set to 1, it indicates that the AMS is initiating the idle mode location update process. In this case, Bit#0 shall be 0 If Bit#2 is set to 1, ranging request for emergency call setup. When this bit is set to 1, it indicates AMS action of Emergency Call process. If bit #5 is set to 1, it indicates that the AMS is initiating location update for transmission to DCR mode from idle mode. If bit #6 is set to 1 in combination with ID of the network entity that assigns/retains the context, it indicates that the AMS is currently attempting re-entry from DCR mode. <<<Editor’s note: reserved bits below are invalid>>> Bit#3-7 : reserved If Bit#7 is set to 1, it indicates that the AMS is currently attempting network reentry after experiencing a coverage loss. | It shall be included when the AMS is attempting to perform reentry, HO or location update |
| --- | --- | --- |
| Serving BSID | The BSID of the AMS’s previous serving ABS before incurring a coverage loss, or tThe BSID of the serving ABS to which the AMS is currently connected (has completed the registration cycle and is in normal operation). The serving BSID shall not be included if the aging timer is timed-out (serving BSID AGINGTIMER). Inclusion of serving BSID in the AAI_RNG-REQ message signals to the target ABS that the AMS is currently connected to the network through the serving ABS and is in the process of HO network reentry. | It shall be included when the AMS is attempting to perform HO reentry, or a network reentry after a coverage loss when the discovered ABS is not the AMS’s previous serving ABS before incurring the coverage loss. In the latter case, it shall be set to be the BSID of the AMS’s previous serving ABS before incurring the coverage loss. |
| Previous STID | The STID which the AMS uses in the previous serving ABS. | It shall be included when the AMS is attempting to perform unprepared HO reentry |
| CMAC Tuple | If included, the CMAC Tuple shall be the last attribute in the message. | It shall be included when the AMS is attempting to perform Network Re-Entry from idle mode, Secure Location Update, or HO, or a reentry after incurring a coverage loss, if the AMS has a CMAC tuple necessary to expedite security authentication. |
| MSID* | AMS’s previous MSID*. | It shall be included when the AMS is attempting to perform network reentry after a coverage loss |

[Change some entries of Table 675 in 15.2.3.2 and insert some new entries, as follows]

Table 675—parameters for AAI_ RNG-RSP

| HO Process Optimization | Identifies reentry process management messages that may be omitted during the current HO attempt or network reentry attempt after a coverage loss due to the availability of MS service and operational context information obtained by means that are beyond the scope of this standard, and the MS service and operational status post-HO completion. The AMS shall not enter normal operation with target ABS until completing receiving all network reentry, MAC management message responses as indicated in HO process optimization. | It shall be included when the AMS is attempting to perform network reentry or HO and the target ABS wishes to identify reentry process management messages that may be omitted during the current HO attempt. |
| --- | --- | --- |