| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Idle Mode AMS Operation Language Cleanup (15.2.17) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Stavros Tzavidas Motorola | stavros.tzavidas@motorola.com |
| Re: | LB30a |  |
| Abstract | The idle mode functionality is there but the language in the description is confusing and uses multiple terms to describe the same thing. This contribution proposes further clarification language. |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Idle Mode AMS Operation Language Cleanup (15.2.17)

Stavros Tzavidas

Motorola

# Introduction

The idle mode section contains some useful functionality which is described in some very very confusing language with inconsistent terminology. This contribution proposes clarification language.

The description of the AMS’ actions is problematic. This section reads:

“During its paging listening interval an idle mode AMS first wakes up during its primary paging offset and look for PGID information. If it does not find the primary PGID information but one secondary PGID then the AMS again wakes up during its secondary paging offset in the same paging cycle. If it does not find the primary PGID information but multiple secondary PGIDs, the AMS shall monitor the paging message transmitted at the shortest paging offset. If it does not find either primary or secondary PGIDs, the AMS shall do location update.”

Again there are important parts of the functionality that are not defined::

- The term “PGID” has not been defined in this section and “PGID information” is ambiguous. Furthermore there is no requirement that the AMS learns the PGID of its preferred ABS during a listening interval. The AMS may learn the PGID(s) of the preferred ABS earlier. What is required here is that simply the AMS determines if it is in the primary or the secondary paging group by monitoring the PGID of its preferred ABS and the text should say just that.

- The AMS does not “find” a PGID or a paging group. It can read a PGID in a message but it cannot “find” it. It is cleaner to describe how the AMS determines which paging group it is under and then describe the actions it should take depending on the results of this determination.

The standard should define the sequence of actions that the AMS should take clearly and precisely, as in the proposed text below.

# Text Proposal

Modify the text in paragraph 15.2.17 “Idle Mode” as follows:

----------------------------------------------Start of the text proposal ------------------------------------------------------

15.2.17 Idle mode

… [text omitted] …

When the AMS is assigned different paging offsets because it belongs to more than one paging group, the primary paging offset is less than the secondary paging offset. The distance between two adjacent paging offsets should be long enough so that the MS paged in the first paging offset can inform the network before the next paging offset in the same paging cycle so that the network avoids unnecessary paging the MS in the next paging offset An AMS determines if it is within its primary paging group or within a secondary paging group by monitoring the PGIDs advertised by its preferred ABS. During its paging listening interval an idle mode AMS first wakes up during its primary paging offset and look for PGID information. If the AMS determines that it is in its primary paging group, the AMS wakes up at its primary paging offset and responds to paging messages that are sent in the primary paging offset and are addressed to it. If it does not find the primary PGID information but one secondary PGID then the AMS again wakes up during its secondary paging offset in the same paging cycle. If the AMS determines that is in a secondary paging group, the AMS wakes up at its secondary paging offset within the same paging cycle and responds to paging messages that are sent in the secondary paging offset and are addressed to it. If it does not find the primary PGID information but multiple secondary PGIDs, the AMS shall monitor the paging message transmitted at the shortest paging offset. If the AMS determines that multiple secondary paging groups are present, the AMS wakes up at the shortest paging offset and responds to paging messages that are sent in during this paging offset and are addressed to it If it does not find either primary or secondary PGIDs, the AMS shall do location update. If the AMS determines that none of the paging groups it has been assigned to are present, the AMS shall perform a location update.

-------------------------------------------------- End of the text proposal --------------------------------------------------