| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification on the Way to Implement Sub-Carrier Alignment (15.3.3.6.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | I-Kang Fu, Pei-Kai Liao, Paul Cheng MediaTek Inc. | IK.Fu@mediatek.com |
| Re: | P802.16m/D2 |  |
| Abstract | Some members are confused on how to implement the sub-carrier alignment and worry that it will substantially impact on AMS hardware design. This contributions aims to clarify the possible misunderstanding and describe how this can be simply done by ABS minor adjustment without impacting on 16m single carrier AMS hardware design.. |  |
| Purpose | For TGm discussion and adoption into P802.16m/D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification on the Way to Implement Sub-Carrier Alignment

I-Kang Fu, Pei-Kai Liao, Paul Cheng

MediaTek Inc.

- Introduction

- Text Proposal

---------------------------------------------------------Start of the Text-----------------------------------------------------------

[Adopt the following text modification started from line#56, page#283 in P802.16m/D2]

Before deployment, ABS will configure its downlink signals for each carrier transmitted over the corresponding center frequencies to meet the sub-carrier alignment requirement. But AMS does not need to know the frequency offset but follow the normal channel scanning procedure (e.g. base on legacy channel raster) when performing the downlink synchronization. Because the frequency offset for achieving sub-carrier alignment is always smaller than a sub-carrier spacing. The sub-carrier alignment by AMS can be done as part of the normal frequency synchronization process without specific signaling.

----------------------------------------------------------End of the Text-----------------------------------------------------------