| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized Text on the Unicast Control Information Elements for Group Resource Allocation (15.3.6.5.2.6) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Shweta Shrivastava, Rath Vannithamby, Roshni Srinivasan Intel Corporation Hyunkyu Yu, Seho Kim Samsung Electronics Kevin Power Fujitsu | Shweta.shrivastava@intel.com Hk.yu@samsung.com Kevin.Power@uk.fujitsu.com |
| Re: | Comments on IEEE 802.16-09/D2 for IEEE 802.16 Working Group Letter Ballot Recirc #30a |  |
| Abstract | Proposed text for Unicast Control Information Elements for Group Resource Allocation (15.3.6.5.2.6) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized Text on the Unicast Control Information Elements

for Group Resource Allocation (15.3.6.5.2.6)

Shweta Shrivastava, Rath Vannithamby, Roshni Srinivasan

Intel Corporation

Hyunkyu Yu, Seho Kim

Samsung Electronics

Kevin Power

Fujitsu

- Introduction

This contribution proposes text to be included in the section 15.3.6.5.2 Unicast Control Information Elements of the IEEE 802.16m draft D2 [1].

- References

[1] Draft Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface IEEE 802.16m Draft, IEEE P802.16m/D2, October 2009.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m draft D2

Adopt proposed text in section 15.3.6.5.2.6 as shown and delete section 15.3.6.5.2.7 in 802.16m draft D2 [1].

Instructions to Editor

Existing text in the draft amendment [1] is shown in black, deleted text is shown in strikethrough red and proposed text is shown in underlined blue font. A ‘clean’ version of the table with all the proposed changes implemented has also been provided for reference.

____________________________Begin proposed text with editorial markup____________________________

15.3.6.5.2.6  Group Resource Allocation A-MAP IE

Group control information is used to allocate resources and/or configure resources to one or multiple AMSs within a user group. The group resource allocation A-MAP IE is shown in Table 807.

Group scheduling requires two operations

- Assignment of an AMS to a group. In order to add a AMS to a group in the DL or UL, the ABS shall transmit a Group Configuration A-MAP IE MAC management message.

- Allocation of resources to AMSs within a group. In order to assign resources to one or more AMSs in a group, the ABS shall transmit the DL/UL Group Resource Allocation A-MAP IE. The DL/UL Group Resource Allocation A-MAP IE is included in user-specific resource assignment in an A-MAP region. The GRA A-MAP IE contains bitmaps to indicate scheduled AMSs and signal nominal, MCS, MIMO mode, HARQ burst size and resource size.

Table 807–DL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | DL Group Resource Allocation A-MAP IE |
| Long TTI Length | 1 | Defines number of subframes spanned by the allocated resource. 0: 1 subframe (default) 1: 4 DL/UL subframes for FDD or all DL/UL subframe for TDD |
| Resource Offset | [6] 7 | Indicates starting LRU for resource assignment to this group |
| ACK Channel HFA Offset | TBD 6 | Indicates the start of ACK HARQ Feedback index used for scheduled allocations and de-allocations at this subframe in the group. The ACK channels for scheduled allocations occur first, in the order of user bitmap, followed by the ACK channel for de-allocations, followed by the ACK channels for shifted AMSs. |
| User Bitmap Size | [2][5] | Indicates the length of User Bitmap |
| User Bitmap | Variable | Bitmap to indicate scheduled AMSs in a group. The size of the bitmap is equal to the User Bitmap Size signaled to each AMS in the Group configuration MAC Control message. 0: AMS not allocated in this AAI subframe 1: AMS allocated in this AAI subframe |
| N_deallocated_AMS | 2 | Number of deallocated AMSs |
| For(i=0;i<N_Deallocated_AMS; i++) { |  |  |
| De-allocated AMS index | Variable | Signals the indices of de-allocated AMS among the inactive AMSs in the user bitmap. The length of De-allocated AMS Index field determined as Ceil{log2(Number of Inactive AMSs in User Bitmap)}. |
| } |  |  |
| Re-arrange Indicator | 1 | Signals whether the re-arrange bitmap is present in this GRA IE |
| If (Re-arrange Indicator == 1) { |  |  |
| Re-arrange Bitmap | Variable | This bitmap signals the occupied bits in the user bitmap. The size of the bitmap is equal to the number of 0’s in the user bitmap. |
| } |  |  |
| If( Group MIMO mode set ==0b01 or 0b11){ |  |  |
| MIMO Bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. 0: Mode 0 1: Mode 1 |
| } |  |  |
| If( Group MIMO mode set == 0b11){ |  |  |
| PSI Bitmap | Variable | Bitmap to indicate PSI for MU-MIMO |
| Pairing Bitmap | Variable | Bitmap to indicate AMS pair sharing same resource for MU-MIMO |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate burst size/resource size for each scheduled AMS |
| Padding | Variable | Padding to reach byte boundary |

*A 16 bit CRC is generated based on the contents of the DL Group Resource Allocation A-MAP IE. The CRC is masked by the Group ID for the DL Group Resource Allocation A-MAP IE.

A-MAP IE Type: Identifies this A-MAP IE as DL Group Resource Allocation IE.

Resource Offset: Signals the offset of the LRU where the allocation for the group starts. The offset is with respect to the start of the AAI subframe.

HFA Offset: Signals the starting HARQ feedback channel index for scheduled AMSs of the group (as identified by 1’s in the User bitmap). The exact allocation for a given AMS is determined by using the mechanism described in section <<<15.3.8.3.3.2>>>.

User Bitmap: A bitmap that uses 1 bit per AMS of the group to signal whether the AMS has an allocation in that AAI subframe. The size of the bitmap is determined by BS and can be 8, 16 or 32 bits. This size is signaled to each AMS in the Group configuration MAC control message when the user is added to the group. The AMSs for which the corresponding bit is set to 1 are referred to as scheduled AMSs in that subframe.

MIMO Bitmap: This bitmap is included in the group only when the group MIMO mode set is 0b01. The MIMO mode set is signaled to AMS in the Group configuration MAC control message when the user is added to the group. The size of this bitmap is equal to the number of scheduled AMSs in the group in that subframe. For each scheduled AMS, the value of corresponding bit in this bitmap signals the MIMO mode (mode 0 or mode 1).

Resource Allocation Bitmap: The resource allocation bitmap uses 5 bits per AMS to signal the HARQ burst size and the resource size for the AMS’s allocation in that subframe. The first 2 bits signal the HARQ burst size and the next 3 bits signal the resource size. The 2-bit and 3-bit codes for burst sizes and resource sizes respectively for the group are determined by each AMS based on the information in the Group Configuration MAC control message.

Table 808–UL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Resource Allocation A-MAP IE |
| Long TTI Length | 1 | Defines number of subframes spanned by the allocated resource. 0: 1 subframe (default) 1: 4 DL/UL subframes for FDD or all DL/UL subframe for TDD |
| Resource Offset | [6] 7 | Indicates starting LRU for resource assignment to this group |
| ACK Channel HFA Offset | TBD 6 | Indicates the start of ACK HARQ Feedback index used for scheduled allocations at this AAI subframe in the group. |
| User Bitmap Size | [2][5] | Size of the user bitmap; may not be needed if user bitmap size is included in configuration message/A-MAP IE. |
| User Bitmap | Variable | Bitmap to indicate scheduled AMSs in a group. The size of the bitmap is equal to the User Bitmap Size signaled to each AMS in the Group configuration MAC Control message. 0: AMS not allocated in this AAI subframe 1: AMS allocated in this AAI subframe |
| N_deallocated_AMS | 2 | Indicates the number of deleted AMSs in the group |
| For(i=0;i<N_Deallocated_AMS; i++) { |  |  |
| De-allocated AMS index | Variable | Signals the indices of de-allocated AMS among the inactive AMSs in the user bitmap. The length of De-allocated AMS Index field determined as Ceil{log2(Number of Inactive AMSs in User Bitmap)}. |
| } |  |  |
| Re-arrange Indicator | 1 | Signals whether the re-arrange bitmap is present in this GRA IE |
| If (Re-arrange Indicator == 1) { |  |  |
| Re-arrange Bitmap | Variable | This bitmap signals the occupied bits in the user bitmap. The size of the bitmap is equal to the number of 0’s in the user bitmap. |
| } |  |  |
| If (N_deallocated_AMS != 0b00 or Re-arrange Indicator == 1) { |  |  |
| HF Index | TBD | Starting HF Index for deallocated AMSs and re-arranged AMSs, if present. |
| } |  |  |
| If( Group MIMO mode set == 0b01){ |  |  |
| MIMO Bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. 0: Mode 0 1: Mode 1 |
| } |  |  |
| If( Group MIMO mode set == 0b10){ |  |  |
| PSI Bitmap | Variable | Bitmap to indicate PSI for MU-MIMO |
| Pairing Bitmap | Variable | Bitmap to indicate AMS pair sharing same resource for MU-MIMO |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate burst size/resource size for each scheduled AMS |
| Padding | Variable | Padding to reach byte boundary |

*A 16 bit CRC is generated based on the contents of the UL Group Resource Allocation A-MAP IE. The CRC is masked by the Group ID for the UL Group Resource Allocation A-MAP IE.

A-MAP IE Type: Identifies this A-MAP IE as DL Group Resource Allocation IE.

Resource Offset: Signals the offset of the LRU where the allocation for the group starts. The offset is with respect to the start of the AAI subframe.

HFA Offset: Signals the starting HARQ feedback channel index for scheduled AMSs of the group (as identified by 1’s in the User bitmap). The exact allocation for a given AMS is determined by using the mechanism described in section <<<15.3.6.3.2.3>>>.

User Bitmap: A bitmap that uses 1 bit per AMS of the group to signal whether the AMS has an allocation in that AAI subframe. The size of the bitmap is determined by BS and can be 8, 16 or 32 bits. This size is signaled to each AMS in the Group configuration MAC control message when the user is added to the group. The AMSs for which the corresponding bit is set to 1 are referred to as scheduled AMSs in that subframe.

MIMO Bitmap: This bitmap is included in the group only when the group MIMO mode set is 0b01. The MIMO mode set is signaled to AMS in the Group configuration MAC control message when the user is added to the group. The size of this bitmap is equal to the number of scheduled AMSs in the group in that subframe. For each scheduled AMS, the value of corresponding bit in this bitmap signals the MIMO mode (mode 0 or mode 1).

Resource Allocation Bitmap: The resource allocation bitmap uses 5 bits per AMS to signal the HARQ burst size and the resource size for the AMS’s allocation in that subframe. The first 2 bits signal the HARQ burst size and the next 3 bits signal the resource size. The 2-bit and 3-bit codes for burst sizes and resource sizes respectively for the group are determined by each AMS based on the information in the Group Configuration MAC control message.

15.3.6.5.2.7  Group Configuration A-MAP IE

The unicast group configuration A-MAP IE is used to assign an AMS to a group

Table 809–DL Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | DL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group identifier |
| Lowest MCS in group | 4 | The index of the lowest MCS that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| Highest MCS in group | 4 | The index of the highest MCS that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Periodicity | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: CL SU-MIMO 0b11: CL SU-MIMO and CL MU-MIMO |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | [4] | Indicates the start of ACID used for group resource allocation. |
| N_ACID | [3] | Indicates the number of ACIDs used for group resource allocation. |
| Padding | Variable | Padding to reach byte boundary. |

*A 16 bit CRC is generated based on the contents of the DL Group Configuration A-MAP IE. The CRC is masked by the Station ID for the DL Group Configuration A-MAP IE.

Table 810–UL Group Configuration A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group identifier. |
| Lowest MCS in group | 4 | The index of the lowest MCS that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| Highest MCS in group | 4 | The index of the highest MCS that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Periodicity | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: OL MU-MIMO 0b11: reserved |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | [4] | Indicates the start of ACID used for group resource allocation. |
| N_ACID | [3] | Indicates the number of ACIDs used for group resource allocation. |
| Padding | Variable | Padding to reach byte boundary. |

*A 16 bit CRC is generated based on the contents of the UL Group Configuration A-MAP IE. The CRC is masked by the Station ID for the UL Group Configuration A-MAP IE.

____________________________End proposed text with editorial markup____________________________

______________________________Begin ‘clean’ version of proposed text_____________________________

15.3.6.5.2.6  Group Resource Allocation A-MAP IE

Group control information is used to allocate resources to one or multiple AMSs within a user group. The group resource allocation A-MAP IE is shown in Table 807.

Group scheduling requires two operations

- Assignment of an AMS to a group. In order to add a AMS to a group in the DL or UL, the ABS shall transmit a Group Configuration MAC management message.

- Allocation of resources to AMSs within a group. In order to assign resources to one or more AMSs in a group, the ABS shall transmit the DL/UL Group Resource Allocation A-MAP IE. The DL/UL Group Resource Allocation A-MAP IE is included in user-specific resource assignment in an A-MAP region. The GRA A-MAP IE contains bitmaps to indicate scheduled AMSs and signal MIMO mode, HARQ burst size and resource size.

Table 807–DL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | DL Group Resource Allocation A-MAP IE |
| Resource Offset | 7 | Indicates starting LRU for resource assignment to this group |
| HFA Offset | 6 | Indicates the start of HARQ Feedback index used for scheduled allocations. |
| User Bitmap | Variable | Bitmap to indicate scheduled AMSs in a group. The size of the bitmap is equal to the User Bitmap Size signaled to each AMS in the Group configuration MAC Control message. 0: AMS not allocated in this AAI subframe 1: AMS allocated in this AAI subframe |
| If( Group MIMO mode set ==0b01){ |  |  |
| MIMO Bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. 0: Mode 0 1: Mode 1 |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate burst size/resource size for each scheduled AMS |

A-MAP IE Type: Identifies this A-MAP IE as DL Group Resource Allocation IE.

Resource Offset: Signals the offset of the LRU where the allocation for the group starts. The offset is with respect to the start of the AAI subframe.

HFA Offset: Signals the starting HARQ feedback channel index for scheduled AMSs of the group (as identified by 1’s in the User bitmap). The exact allocation for a given AMS is determined by using the mechanism described in section <<<15.3.8.3.3.2>>>.

User Bitmap: A bitmap that uses 1 bit per AMS of the group to signal whether the AMS has an allocation in that AAI subframe. The size of the bitmap is determined by BS and can be 8, 16 or 32 bits. This size is signaled to each AMS in the Group configuration MAC control message when the user is added to the group. The AMSs for which the corresponding bit is set to 1 are referred to as scheduled AMSs in that subframe.

MIMO Bitmap: This bitmap is included in the group only when the group MIMO mode set is 0b01. The MIMO mode set is signaled to AMS in the Group configuration MAC control message when the user is added to the group. The size of this bitmap is equal to the number of scheduled AMSs in the group in that subframe. For each scheduled AMS, the value of corresponding bit in this bitmap signals the MIMO mode (mode 0 or mode 1).

Resource Allocation Bitmap: The resource allocation bitmap uses 5 bits per AMS to signal the HARQ burst size and the resource size for the AMS’s allocation in that subframe. The first 2 bits signal the HARQ burst size and the next 3 bits signal the resource size. The 2-bit and 3-bit codes for burst sizes and resource sizes respectively for the group are determined by each AMS based on the information in the Group Configuration MAC control message.

Table 808–UL Group Resource Allocation A-MAP IE

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Resource Allocation A-MAP IE |
| Resource Offset | 7 | Indicates starting LRU for resource assignment to this group |
| HFA Offset | 6 | Indicates the start of HARQ Feedback index used for scheduled allocations at this AAI subframe in the group. |
| User Bitmap | Variable | Bitmap to indicate scheduled AMSs in a group. The size of the bitmap is equal to the User Bitmap Size signaled to each AMS in the Group configuration MAC Control message. 0: AMS not allocated in this AAI subframe 1: AMS allocated in this AAI subframe |
| If( Group MIMO mode set == 0b01){ |  |  |
| MIMO Bitmap | Variable | Bitmap to indicate MIMO mode for the scheduled AMSs. 0: Mode 0 1: Mode 1 |
| } |  |  |
| Resource Assignment Bitmap | Variable | Bitmap to indicate burst size/resource size for each scheduled AMS |

A-MAP IE Type: Identifies this A-MAP IE as DL Group Resource Allocation IE.

Resource Offset: Signals the offset of the LRU where the allocation for the group starts. The offset is with respect to the start of the AAI subframe.

HFA Offset: Signals the starting HARQ feedback channel index for scheduled AMSs of the group (as identified by 1’s in the User bitmap). The exact allocation for a given AMS is determined by using the mechanism described in section <<<15.3.6.3.2.3>>>.

User Bitmap: A bitmap that uses 1 bit per AMS of the group to signal whether the AMS has an allocation in that AAI subframe. The size of the bitmap is determined by BS and can be 8, 16 or 32 bits. This size is signaled to each AMS in the Group configuration MAC control message when the user is added to the group. The AMSs for which the corresponding bit is set to 1 are referred to as scheduled AMSs in that subframe.

MIMO Bitmap: This bitmap is included in the group only when the group MIMO mode set is 0b01. The MIMO mode set is signaled to AMS in the Group configuration MAC control message when the user is added to the group. The size of this bitmap is equal to the number of scheduled AMSs in the group in that subframe. For each scheduled AMS, the value of corresponding bit in this bitmap signals the MIMO mode (mode 0 or mode 1).

Resource Allocation Bitmap: The resource allocation bitmap uses 5 bits per AMS to signal the HARQ burst size and the resource size for the AMS’s allocation in that subframe. The first 2 bits signal the HARQ burst size and the next 3 bits signal the resource size. The 2-bit and 3-bit codes for burst sizes and resource sizes respectively for the group are determined by each AMS based on the information in the Group Configuration MAC control message.

______________________________End ‘clean’ version of proposed text_____________________________