| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Harmonized Text on Group Configuration MAC Control Message for Group Resource Allocation (15.2.3) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Shweta Shrivastava, Rath Vannithamby, Roshni Srinivasan Intel Corporation Hyunkyu Yu, Seho Kim Samsung Electronics Kevin Power Fujitsu Youngsoo Yuk, Jeongki Kim LG Electronics Yanfeng Guan ZTE Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung ETRI | Shweta.shrivastava@intel.com Hk.yu@samsung.com Kevin.Power@uk.fujitsu.com sixs@lge.com guan.yanfeng@zte.com.cn jsp@etri.re.kr |
| Re: | Comments on IEEE 802.16-09/D2 for IEEE 802.16 Working Group Letter Ballot Recirc #30a |  |
| Abstract | Proposed text on Group Configuration MAC Control Message for Group Resource Allocation (15.2.3) |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Harmonized Text on Group Configuration MAC Control Message for Group Resource Allocation (15.2.3)

Shweta Shrivastava, Rath Vannithamby, Roshni Srinivasan

Intel Corporation

Hyunkyu Yu, Seho Kim

Samsung Electronics

Kevin Power

Fujitsu

Youngsoo Yuk, Jeongki Kim

LG Electronics

Yanfeng Guan

ZTE

Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung

ETRI

- Introduction

This contribution proposes text to be included in the section 15.2.3 of the IEEE 802.16m draft D2 [1] under a new subsection 15.2.3.41 Group Configuration MAC Control Message (AAI_GRP-CFG).

- References

[1] Draft Amendment to IEEE Standard for Local and metropolitan area networks Part 16: Air Interface for Broadband Wireless Access Systems, Advanced Air Interface IEEE 802.16m Draft, IEEE P802.16m/D2, October 2009.

[2] IEEE 802.16m-07/002r7, “802.16m System Requirements”

[3] IEEE 802.16m-08/003r6, “The Draft IEEE 802.16m System Description Document”

[4] IEEE 802.16m-08/043, “Style guide for writing the IEEE 802.16m amendment”

- Text proposal for inclusion in the 802.16m draft D2

Adopt proposed text in section 15.2.3 under a new subsection 15.2.3.41 in 802.16m draft D2 [1].

The proposed text is shown in underlined blue font.

____________________________Begin proposed text with editorial markup____________________________

15.2.3.41 Group Configuration MAC Control Message (AAI_GRP-CFG)

Group Configuration MAC Control message is used for group management when using Group Resource Allocation. This message is used to inform an AMS that it has been added to or deleted from a GRA group. The format of the message is given in <<table xxx>>.

Table xxx–Group Configuration MAC Control Message Format

| Syntax | Size in bits * | Description/Notes |
| --- | --- | --- |
| MAC Control Message type | 8 |  |
| Deletion Flag | 1 | Flag to signal whether this message includes addition or deletion information. 0: Flow is added to a group 1: Flow is explicitly deleted from a group |
| Flow ID | 4 | Identifies the flow that is added to group for GRA |
| If (Deletion Flag == 0) { |  |  |
| Group ID | 5 | ID of the group to which the flow is added |
| Long TTI Indicator | 1 | Defines number of subframes spanned by the allocated resource. 0: 1 subframe (default) 1: 4 DL/UL subframes for FDD or all DL/UL subframe for TDD |
| Periodicity | 2 | Periodicity of allocation of this group. This specifies the periodicity with which the corresponding GRA A-MAP IE will be transmitted. 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| MIMO mode set | 2 | Signals the 2-bit MIMO mode set corresponding to this group as shown in tables <<728>> and <<729>> |
| User Bitmap Size | 2 | The size of user bitmap in bits. The size remains fixed for a group. The size determines the maximum number of flows that can be supported in one group. 0: 4 1: 8 2: 16 3: 32 |
| User Bitmap Index | 5 | Signals index of the flow in group’s user bitmap |
| Initial_ACID | 4 | Signals the starting ACID of the range of ACIDs assigned to the GRA flow |
| N_ACIDs | 3 | The number of ACIDs assigned to the GRA flow |
| Burst Size Indicator (b) | 1 | Signals whether the HARQ burst sizes supported in the group are included in this message or are same as the last GRA allocation for this flow. 0: Burst sizes same as last GRA allocation for this flow, hence not included in the message 1: Burst sizes explicitly included in the message |
| If (b == 1) { |  |  |
| For (i=1; i<=4; i++) { |  |  |
| Burst size i | 5 | ith burst size of the 4 burst sizes supported in the group |
| } |  |  |
| } |  |  |
| Resource size indicator (r) | 1 | Signals whether the resource sizes supported in the group are included in this message or are same as the previous group 0: Resource sizes same as last GRA allocation for this flow, hence not included in the message 1: Resource sizes explicitly included in the message |
| If (r == 1) { |  |  |
| For (i=1; i<=4; i++) { |  |  |
| Resource Bitmap Flag (f) | 1 | 0b0: The resource size inclusion bitmap for this burst size is the same as the bitmap defined for the (i-1) th burst size. 0b1: Define a resource size inclusion bitmap for the i-th burst size |
| If (f == 0b1) { |  |  |
| Resource size inclusion bitmap | 16 | Bitmap to signal which 8 resource sizes are supported in the group out of the range of [1,16] LRUs supported for GRA. If nth bit in the bitmap is set to 1, it signals that a resource size of n LRUs is supported in the group. |
| } |  |  |
| } |  |  |
| } |  |  |
| } |  |  |
| Padding | Variable | Padding to align to byte boundary |

The Group Configuration MAC Control message signals all the information necessary for the AMS to receive allocations as part of a group via GRA (when the Deletion Flag is set to 0). The encoding of HARQ burst sizes and resource sizes supported in the group is determined as follows. The Group Configuration MAC control message signals four HARQ burst sizes. These burst sizes are assigned 2 bits code in increasing order from 00 to 11 in the order in which they appear in the message.

Similarly, 3-bit resource size codes from 000 to 111 are assigned to the supported resource sizes in increasing order. The Resource size inclusion bitmap should have only 8 bits set to 1 and the rest to 0.

____________________________End proposed text with editorial markup____________________________