| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | # Proposed Text for Section 15.3.9.4 E-MBS Configuration Indicators |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Kaushik Josiam, Qi Wu, Taeyoung Kim, Jaejeong Shim Samsung Electronics | Email : kjosiam@sta.samsung.com qi.wu@samsung.com ty33.kim@samsung.com brian.shim@samsung.com |
| Re: | Contribution in support of a comment on P802.16m/D2 currently under LB#30 |  |
| Abstract | This contribution defines the text and format for E-MBS Configuration Message |  |
| Purpose | For review and adoption by IEEE 802.16m TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text for Section 15.3.9.4 E-MBS Configuration Indicators

Kaushik Josiam, Qi Wu, Taeyoung Kim, Jaejeong Shim

Samsung Electronics

---------------------------------------------------- Start of the Text Proposal ------------------------------------------------

Make the following changes to Section 15.9.3.1 in Page 584 beginning Line 39 in P802.16m/D2.

15.9.3.1 E-MBS Configuration InformationIndicators

The information regarding E-MBS configuration is transmitted to the AMSs using AAI-E-MBS-CFG message. configuration indicators. While most E-MBS configuration indicators necessary for E-MBS operation are of interest only to the AMSs interested in E-MBS, a few E-MBS related configuration information may be broadcasted to all AMSs.

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isizeoffset, E-MBS MAP MIMO Mode and MSTID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content. Some of these E-MBS configuration information are transmitted in a new MAC management message called the AAI-E-MBS_CFG message to be read only by the E-MBS AMSs.The E-MBS configuration information specify the resources reserved for E-MBS traffic in the downlink physical resources which is of interest to all AMSs and additional information necessary for E-MBS operation.

15.9.3.1.1 AAI-E-MBS-CFG message

An AAI-E-MBS-CFG message is transmitted by the ABS at periodic intervals to define E-MBS configuration information.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| AAI-E-MBS-CFG_Message_Format() { |  |  |
| Num_E-MBS_Zones | 4 | Number of E-MBS Zones with which the ABS is associated. |
| E-MBS_CFG_LIFETIME | 4 | Indicates the duration in the multiple of MSIs for which the configuration parameters of the E-MBS zone do not change (in unit of MSI) |
| E-MBS_SUBFRAME_INDICATOR | 3 | Number of subframes reserved for E-MBS within a frame 0b000: Last 1 subframe of the DL zone 0b001: Last 2 subframe s of the DL zone 0b010: Last 3 subframe s of the DL zone 0b011: Last 4 subframe s of the DL zone 0b100: Last 5 subframes of the DL zone 0b101: Last 6 subframes of the DL zone 0b110: Last 7 subframes of the DL zone 0b111: Last 8 subframes of the DL zone |
| for (i = 0; i< Num_E-MBS_Zones; i++) { |  |  |
| E-MBS_Zone_ID | 7 | The EMBS_Zone_ID to which this EMBS MAP applies. |
| E-MBS_SUBBAND_INDEX | 8 | Indicates the number of subbands in the E-MBS subframe reserved for the E-MBS Zone. The number of subbands reserved for the E-MBS zone is specified in E-MBS_SUBFRAME_INDICATOR. |
| E-MBS MAP Resource Index | 11 | Resource index includes location and allocation size. |
| E-MBS MAP ISizeOffset | 5 |  |
| } |  |  |
| } |  |  |

-----------------------------------------------------End of the Text Proposal ---------------------------------------------------