| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Network synchronization for Femto ABS (15.4.6) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Guang Han, Stavros Tzavidas Motorola | guang.han@motorola.com stavros.tzavidas@motorola.com |
| Re: | LB 30a |  |
| Abstract | This contribution proposes solutions for supporting Femtocell BSs in 16m |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Network synchronization for Femto ABS (15.4.6)

Guang Han, Stavros Tzavidas

Motorola

# Introduction

Section 15.4.6 of AWD discusses about network synchronization for Femto ABS. It is proposed that Femto ABS can perform scanning of the A-Preamble of the Macro ABS during the unavailable interval when the Femto ABS is in the low duty operation mode. However, according to section 15.4.10.1, “the Femto ABS may enter low-duty operation mode either if all MSs attached to the Femto ABS are in idle or sleep mode, or if no MS is in the service range of the Femto ABS at all”. If some attached MS is in active mode, the Femto ABS will not enter low duty operation mode and may not have the opportunity to perform synchronization for a long time, thus leading to clock inaccuracy and performance degradation.

We thus propose that Femto ABS need to periodically perform network synchronization. When the synchronization timer is about to expire and some attached AMSs are in active mode, the Femto ABS needs to command the active mode AMSs to enter sleep mode and scan the A-Preamble during the sleep windows.

# Text Proposal

Modify the following text in802.16m amendment working document (P802.16m_D1) section15.4.6:

----------------------------------------------Start of the text proposal ------------------------------------------------------

15.4.6 Network synchronization for Femto ABS

A Femto ABS should be synchronized with the overlay Macro ABS network. The network synchronization may be achieved by Femto ABS scanning the A-Preamble transmitted by the Macro ABSs. If the Femto ABS can successfully detect the Macro ABS A-Preamble, it shall synchronize its downlink transmission with the received A-Preamble signal from Macro ABSs. The A-Preamble scanning for Femto ABS network synchronization may be performed before Femto ABS activation and periodicallyor during the normal operation by low duty mode. When the synchronization timer is about to expire and some attached AMSs are in active mode, the Femto ABS needs to command the active mode AMSs to enter sleep mode and scan the A-Preamble during the sleep windows. The Femto ABS may also achieve network synchronization from GPS or backhaul network (e.g. IEEE 1588).

-------------------------------------------------- End of the text proposal --------------------------------------------------