| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Optimizing the overhead of FPEH for VoIP and other small packet traffic (15.2.2.2.1) |  |
| Date Submitted | 2009-11-05 |  |
| Source(s) | Muthaiah Venkatachalam, Hujun Yin, Joey Chou, Roshni Srinivasan, Shantidev Mohanty | Intel Corporation |
| Re: | Optimizing the overhead of FPEH for VoIP and other small packet traffic (15.2.2.2.1) |  |
| Abstract | Optimizing the overhead of FPEH for VoIP and other small packet traffic (15.2.2.2.1) |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Optimizing the overhead of FPEH for VoIP and other small packet traffic

# Introduction

FPEH is always used with GMH today. 2B of FPEH is excessive for VoIP.

Proposal is to add one more switch bit in FPEH to allow 7 bit SN. This will allow a shorter version of FPEH (8 bits), it will reduce SN from 10 bits to 9 bits in the longer 2B version of FPEH.

# Proposed Text with change marks

[-------------------------------------------------Start of Text Proposal---------------------------------------------------]

15.2.2.2.1 Fragmentation and packing extended header (FPEH)

The FPEH shall be used when MAC PDU contains payload from single transport connection. The FPEH exists after the last extended header (i.e. the extended header with 'Last' = '1') if 'EH' in AGMH set to '1' or after the AGMH if 'EH' in AGMH set to '0'. The FPEH format is defined in Table 657. 'RI' in FPEH is used for indicating whether the payload contains ARQ sub-blocks or not. If 'RI' bit set to '1', 'LSI' and 'SSN' shall be included in FPEH.

FPEH has 2 modes of operation: Small payload mode and regular mode of operation. In order to indicate the mode of operation, FPEH shall contain the mode bit in the beginning.

The format of small payload mode of FPEH is shown in table xyz. Small payload mode is meant for small payloads such as VoIP traffic. Small payload mode does not support ARQ.

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| FPEH (){ |  |  |
| Mode | 1 | Payload mode of FPEH |
| SN | 7 | SN is maintained per connection. For non ARQ connection, 'SN' represents the MAC PDU Payload Sequence Number and the 'SN' value increments by one (modulo 1024) for each MAC PDU. For ARQ connection, 'SN' represents the ARQ block sequence number. |
| } |  |  |

Table xyz: Small payload mode of FPEH

…

[Change the first 5 rows of table 657 as follows]

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| FPEH (){ |  |  |
| Mode | 1 | Payload mode of FPEH |
| RI | 1 | Re-arrangement information indicator |
| SN | 9 | SN is maintained per connection. For non ARQ connection, 'SN' represents the MAC PDU Payload Sequence Number and the 'SN' value increments by one (modulo 1024) for each MAC PDU. For ARQ connection, 'SN' represents the ARQ block sequence number. |

[change first 3 rows of table 658 as follows]

| Names | Size (bits) | Description |
| --- | --- | --- |
| Mode | 1 | Payload mode of FPEH Mode=0: Regular mode Mode=1: Small payload mode |
| SN | 9 | Payload sequence number |

[-------------------------------------------------End of Text Proposal----------------------------------------------------]