| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Enhanced FPEH |  |
| Date Submitted | 2009-11-18 |  |
| Source(s) | Muthaiah Venkatachalam, Hujun Yin, Joey Chou, Roshni Srinivasan, Shantidev Mohanty, Aran Bergman Intel Corporation Anil Agiwal, Youngbin Chang, Sungjin Lee, Rakesh Taori Samsung Electronics | Joey.chou@intel.com anilag@samsung.com |
| Re: | IEEE 802.16-09/0057, IEEE 802.16 Working Group Letter Ballot Recirc #30a / Topic: FPEH (Section 15.2.2.2.1) |  |
| Abstract | This contribution proposes enhanced FPEH and Length extension header |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Enhanced FPEH

# Introduction

This contribution enhances FPEH to support ARQ disable connections. It also proposes a MPDU Length Extended Header to extend the size of MPDU.

Proposed FPEH Overview:

| Non ARQ Connection (mode 0): 1 byte minimum overhead Supports fragmentation and packing Short SN | Non ARQ connection (mode 1): 2 byte minimum overhead Supports fragmentation and packing Long SN Format is identical to FPEH for ARQ connection (using reserved bits) | ARQ connection: 2 byte minimum overhead |
| --- | --- | --- |
| Mode bit | Mode bit | Same fields as in D2 |
| FC | FC |  |
| 4 bit SN | 10 bit SN |  |
| End (and optionally) Length fields | End and Length fields |  |

# Proposed Text with change marks

[-------------------------------------------------Start of Text Proposal------------------------------------------]

[Change 1: Modify section 15.2.2.2.1 on page 19, lines 52 to 63 as follows]

15.2.2.2.1 Fragmentation and packing extended header (FPEH)

The FPEH shall be used when MAC PDU contains payload from singlea transport connection. The FPEH exists after the last extended header (i.e. the extended header with 'Last' = '1') if 'EH' in AGMH set to '1' or after the AGMH if 'EH' in AGMH set to '0'. The FPEH format is defined in Table 657. 'RI' in FPEH is used for indicating whether the payload contains ARQ sub-blocks or not. If 'RI' bit set to '1', 'LSI' and 'SSN' shall be included in FPEH.

[Change 2: Replace the table 657 on page 20-21 with the table given below]

Table 657: FPEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| FPEH (){ |  |  |
| If (Non ARQ enabled connection) { | - | Non ARQ enabled connection |
| Mode | 1 | 0 = short SN mode 1 = Long SN mode Either short or long mode shall be used for the non ARQ enabled connection payload in the MAC PDU |
| FC | 2 | Fragmentation Control bits (see Table 659) |
| If (mode == 0) { |  |  |
| SN | 4 | SN is maintained per connection. 'SN' represents the Payload Sequence Number and the 'SN' value increments by one (modulo 32) for each MAC PDU. |
| } else { |  |  |
| SN | 10 | SN is maintained per connection. 'SN' represents the Payload Sequence Number and the 'SN' value increments by one (modulo 1024) for each MAC PDU. |
| Reserved | 2 | Shall be set to 0 |
| } |  |  |
| Do { |  |  |
| End | 1 | Indication of more information 0 = Indicating another 'Length' and 'End' fields are followed 1 = Indicating no more 'Length' and 'End' fields are followed |
| If (End == 0) { |  |  |
| Length | 11 | This field indicates the length of SDU or SDU fragment. If a payload consists of 'N' SDU/SDU fragments, N-1 'Length' fields are present in FPEH. These represent the length of first 'N-1' SDU/SDU fragments in payload. The length of the last SDU or SDU fragment in the payload is = MAC PDU payload length - sum of 'N-1' length fields in FPEH. |
| } |  |  |
| } while (!End) |  |  |
| } |  |  |
| Padding | Variable | For byte allignment |
| } |  |  |
| If (ARQ enabled connection ) { |  | ARQ enabled connection |
| RI | 1 | ARQ Re-arrangement information indicator 0= no ARQ re-arrangement 1= ARQ re-arrangement |
| FC | 2 | Fragmentation Control bits (see Table 659) |
| SN | 10 | Represents the ARQ block sequence number. |
| AFI | 1 | ARQ feedback IE indicator 0 = ARQ feedback IE is not present in the MAC PDU 1 = ARQ feedback IE follows after FPEH |
| AFP | 1 | ARQ feedback poll indicator 0 = No ARQ feedback poll 1 = ARQ feedback poll for the connection indicated in AGMH or MEH if multiplexing is used |
| If (RI == 1) { |  |  |
| LSI | 1 | Last ARQ sub-block indicator 0 = indicating the last ARQ sub-block from the single ARQ block is not included in this MAC PDU 1 = indicating the last ARQ sub-block from the single ARQ block is included in this MAC PDU |
| SSN | 8 | SUB-SN of the first ARQ sub-block |
| } |  |  |
| Do { |  |  |
| End | 1 | Indication of more information 0 = Indicating another 'Length' and 'End' fields are followed 1 = Indicating no more 'Length' and 'End' fields are followed |
| If (End == 0) { |  |  |
| Length | 11 | This field indicates the length of SDU or SDU fragment. If a payload consists of 'N' SDU/SDU fragments, N-1 'Length' fields are present in FPEH. These represent the length of first 'N-1' SDU/SDU fragments in payload. The length of the last SDU or SDU fragment in the payload is = MAC PDU payload length - sum of 'N-1' length fields in FPEH. |
| } |  |  |
| } while (!End) |  |  |
| Padding | Variable | For byte allignment |
| } |  |  |
| } |  |  |

[Change3: Add the new section shown below after line 38 on page 29]

15.2.2.2.10 MAC PDU length extended header (MLEH)

The MAC PDU length extended header (MLEH) is added to MAC PDU when the MAC PDU length is greater that 2047 bytes. The MLEH if present in the MAC PDU, shall be the first extended header in the MAC PDU. The format of MLEH is defined in table xxx. The Length field in MLEH gives the 3 MSBs of 14 bit extended length of MAC PDU. The length field in AGMH gives the 11 LSBs of 14 bit extended length of MAC PDU.

Table xxx: MLEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MLEH (){ |  |  |
| Last | 1 | Last Extended Header indication: 0 = one or more extended header follows the current extended header unless specified otherwise; 1 = this extended header is the last extended header unless specified otherwise |
| Type | 4 | Extended header type |
| Length | 3 | The 3 bit length is to be appended as MSB to the 11 bit length field in AGMH : MPDU length = MLEH (length(3)) .. AGMH(Length(11). |
| } |  |  |

[-------------------------------------------------End of Text Proposal--------------------------------------------]