| 3Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MAC PDU Length EH |  |
| Date Submitted | 2009-11-18 |  |
| Source(s) | Muthaiah Venkatachalam, Hujun Yin, Joey Chou, Roshni Srinivasan, Shantidev Mohanty, Aran Bergman Intel Corporation Anil Agiwal, Youngbin Chang, Sungjin Lee, Rakesh Taori Samsung Electronics Jeongki Kim, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim, JinSam Kwak LG Electronics | Joey.chou@intel.com anilag@samsung.com chegal@lge.com |
| Re: | IEEE 802.16-09/0057, IEEE 802.16 Working Group Letter Ballot Recirc #30a / Topic: FPEH (Section 15.2.2.2.1) |  |
| Abstract | This contribution proposes Length extension header |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MAC PDU Length EH

# Introduction

This contribution proposes a MPDU Length Extended Header to support MPDUs up to 14350 bytes.

# Proposed Text with change marks

[-------------------------------------------------Start of Text Proposal------------------------------------------]

[Change: Add the new section shown below after line 38 on page 29]

15.2.2.2.10 MAC PDU length extended header (MLEH)

The MAC PDU length extended header (MLEH) is added to MAC PDU when the MAC PDU length is greater that 2047 bytes. The MLEH if present in the MAC PDU, shall be the first extended header in the MAC PDU. The format of MLEH is defined in table xxx. The Length field in MLEH gives the 4 MSBs of 15 bit extended length of MAC PDU. The length field in AGMH gives the 11 LSBs of 15 bit extended length of MAC PDU.

Table xxx: MLEH Format

| Syntax | Size (bits) | Notes |
| --- | --- | --- |
| MLEH (){ |  |  |
| Type | 4 | Extended header type |
| Length | 4 | The 4 bit length is to be appended as MSB to the 11 bit length field in AGMH : MPDU length = MLEH (length(4)) .. AGMH(Length(11). |
| } |  |  |

[-------------------------------------------------End of Text Proposal--------------------------------------------