| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Host Configuration Parameters for the IEEE 802.16m Amendment (15.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Jicheol Lee (Samsung) Muthaiah Venkatachalam (Intel) | Jicheol Lee [jicheol.lee@samsung.com] |
| Re: | IEEE802.16m/D2 |  |
| Abstract |  |  |
| Purpose |  |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Host Configuration Parameters 
for the IEEE 802.16m AWD

Jicheol Lee (Samsung),

Muthaiah Venkatachalam (Intel)

- Text proposal for inclusion in the 802.16m amendment working document

-------------------------------  Text Start  ---------------------------------------------------

[Note to Editor: Please add the following paragraph at the end of 15.2.15.6 right before 15.2.15.6.1 in page 222 line 45]

If the AMS wants to configure additional host configuration parameters, the AMS may indicate those parameters by including Requested-Host-Configurations IE in AAI_REG-REQ message. If AMS indicates additional host configuration parameters or the network decides to configure additional host configuration parameters, ABS may include Additional-Host-Configurations IE in AAI_REG-RSP message.

The format of the value field of Requested-Host-Configurations IE and Additional-Host-Configurations IE is defined RFC2132 for IPv4, RFC3315 for IPv6. Note that list of parameters can be found in http://www.iana.org/assignments/bootp-dhcp-parameters/ and http://www.iana.org/assignments/dhcpv6-parameters/.

[Note to Editor: Please modify the following rows of Table 676 - Parameters for AAI_REG-REQ” under the section 15.2.3.6 in page 42 line 18]

| Name |  | Value |
| --- | --- | --- |
| … |  | … |
| Convergence sublayer capabilities | Supported-IP-Versions IE | Indicates whether the AMS supports IPv4, IPv6 or both. 0b00: reserved 0b01: IPv4 support 0b10: IPv6 support 0b11: both IPv4/IPv6 support |
| Host configuration capabilities and parameters | Host-Configuration-Capability-Indicator IE | Indicates whether the AMS supports the capability of configuring host using the received parameters through the AAI_REG-RSP message. (One bit indicator) |
|  | Requested-Host-Configurations IE | Includes requested host configuration options in DHCP Options format |
| Notes If the Supported-IP-Versions IE indicates both IPv4 and IPv6 capable, it means the AMS supports dual stack of IPv4/IPv6. |  |  |

[Note to Editor: Please modify the following rows of “Table 677-parameters for AAI_REG-RSP message” under the section 15.2.8.7 in page 44 line 21]

| Name |  | Value |
| --- | --- | --- |
| … |  | … |
| Convergence sublayer capabilities | IP-Service-Type IE | Indicates which IP service will be used, the value is one of following: IPv4-Service IPv6-Service IPv4/IPv6-Dual-Mode-Service |
| Host configuration capabilities and its parameters | IPv4-Host-Address IE | The allocated IPv4 Host Address for the AMS. Size: 4 octet |
|  | IPv6-Home-Network-Prefix IE | The allocated IPv6 Home Network Prefix for the AMS. Size: 8 octet |
|  | Additional-Host-Configurations IE | Includes additional host configuration options in DHCP Options format |

-------------------------------  Text End  ---------------------------------------------------