| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on assignment A-MAP segmentation in the 802.16m AWD (16.3.6) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Yi Hsuan, Shweta Shrivastava, Tom Harel, Aran Bergman, Roshni Srinivasan, Hujun Yin Intel Corporation | yi.hsuan@intel.com |
| Re: | Category: AWD comments / Area: Chapter 16.3.6 (DL-CTRL) “Comments on AWD 16.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes text to describe how to segment large A-A-MAP IE to smaller segments |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text on assignment A-MAP segmentation in the 802.16m AWD (16.3.6)

Yi Hsuan, Shweta Shrivastava, Tom Harel, Aran Bergman, Roshni Srinivasan, Hujun Yin

Intel Corporation

- Introduction

In IEEE P802.16m/D2, non user-specific A-MAP IE has been defined to signal the size of each A-A-MAP group, which is composed by A-A-MAP IEs of the same size and transmitted with the same MCS. However the design of non use-specific A-MAP IE only allows A-A-MAP group signaling assuming a single size of A-A-MAP IE. To allow more sizes, the size of non user-specific A-MAP or the blind detection complexity has to increase. While most of A-A-MAP IEs are not larger than 56 bits, some, e.g., GRA A-MAP IE, can be bigger than 56 bits. It is therefore necessary to describe transmission and reception of such A-A-MAP IEs.

- A-A-MAP IE Segmentation

The basic idea in our proposal to deal with large A-A-MAP IE is to segment an IE into several smaller segments, each with 56 bits. These segmented IEs are sequentially transmitted using logically contiguous MLRUs in an A-MAP region. At the receiver side, AMS should perform normally blind detection to decode these segmented IEs and concatenated them into the original IE. There are several considerations in the segmentation design:

- A-A-MAP IE segmentation design should allow MS to recover the original IE easily.

  - Segments of an A-A-MAP IE should be coded with the same MCS and occupy a number of logically continuous MLRUs.

  - AMS A-A-MAP blind detection process should not be complicated by IE segmentation. With this in mind, we decide to include CRC in each segment so MS can perform the same blind detection operation for segmented IEs.

- If one of the segments is not decoded correctly, MS should be able to detect that the IE is not complete.

With these considerations in mind, we propose the following segmentation process:

- If an A-A-MAP IE is larger than 40 bits (excluding CRC), the IE, without IE type, is segmented into 35 bits piece first. The last piece may have less than 35 bits and zero padding is applied to obtain 35 bits.

- At the beginning of each piece, 4 bit A-A-MAP IE type and a 1 bit flag are inserted. The 1 bit flag is to indicate whether this segment is the first or not.

- For each segment, a 16-bit CRC created from the 40-bit segment is appended to the end to form a 56 bit segmented IE.

- All segmented IEs are coded with the same MCS and occupy a number of logically contiguous MLRUs.

The 1 bit to indicate the first segment or not allows AMS to do error handling if the first segment is not received correctly. The first segment contains information regarding total number of segments belonging to the same IE. If other segments are not received correctly, AMS can detect missing segments during IE parsing.

-------------------------------  Start Text Proposal ---------------------------------------------------

- [ Recommended AWD Text Proposal 1 (Section 16.3.6.2.2.4, page 369, line 32) ]

The assignment A-MAP IE shall be transmitted with one MLRU or multiple concatenated MLRUs in the A-MAP region. The number of logically contiguous MLRUs is determined based on the assignment A-MAP IE size and channel coding rate, where channel coding rate is selected based on AMS’ link condition.

Assignment A-MAP IEs with less than 40 bits are zero-padded to 40 bits. Assignment A-MAP IEs with more than 40 bits are divided into several segmented IE, each with 40 bits. Segments of an A-A-MAP IE should be separately coded with the same MCS and occupy a number of logically contiguous MLRUs. The process of assignment A-MAP IE segmentation is described in 16.3.6.5.2.4.14.

Assignment A-MAP IEs are grouped together based on channel coding rate and A-MAP IE sizes. Assignment

A-MAP IEs in the same group are transmitted in the same frequency partition with the same channel coding rate and contain the same A-MAP IE size. Each assignment A-MAP group contains several logically contiguous MLRUs. The number of assignment A-MAP IEs in each assignment A-MAP group is signaled through non-user specific A-MAP in the same AAI subframe.

- [ Recommended AWD Text Proposal 2 (New Section 16.3.6.5.2.4.14, page 443, line 50) ]

16.3.6.5.2.4.14

A type of assignment A-MAP IE is segmentable if its size can be larger than 40 bits. A segmentable assignment A-MAP IE should be parsed and coded in the following steps:

- The IE, without IE type, is divided into 35-bit segments one by one. If the last segment has less than 35 bits, zero padding is applied to obtain 35 bits.

- At the beginning of each segment, a 4-bit A-A-MAP IE type and a 1-bit flag are inserted to form 40-bits segments. The value of the 1-bit flag is 1 for the first segment and 0 for other segments.

- For each segment, a 16-bit CRC created from the 40-bit segment is appended to the end to form a 56 bit segment.

- All segments from the assignment A-MAP IE are coded with the same MCS and occupy a number of logically contiguous MLRUs in sequence.

-------------------------------------- End of Text Proposal -------------------------------------------