| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to change the location of non user-specific A-MAP in the 802.16m AWD (15.3.6) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Yi Hsuan and Hujun Yin Intel Corporation | yi.hsuan@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes to change the location of non user-specific A-MAP in an A-MAP region |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to change the location of non user-specific A-MAP in the 802.16m AWD (15.3.6)

Yi Hsuan and Hujun Yin

Intel Corporation

- Motivation

In the current A-MAP region design, different types of A-MAP channels are assigned resources in the following logical order: non user-specific A-MAP, HARQ feedback A-MAP, power control A-MAP, and assignment A-MAP. The size of each type of A-MAP is defined in the following:

- Non user-specific A-MAP has a default size of 12 bits. Given the configuration indicated by SFH, the total resource occupied by the 12 bits can be exactly determined. However non user-specific A-MAP can be extended to another 12 bits, based on the last bit value of 12 bits of the default size.

- Resource occupied by HARQ feedback A-MAP can be derived from SFH configuration.

- Resource occupied by power control A-MAP can be derived from SFH configuration.

- Assignment A-MAP resource is indicated by non user-specific A-MAP.

As shown above, the resource of HARQ feedback A-MAP and power control A-MAP is fixed over at least one superframe period, while resource of non user-specific A-MAP and assignment A-MAP can change every subframe. It makes more sense to re-arrange different A-MAP in the order of HARQ feedback A-MAP, power control A-MAP, non user-specific A-MAP and assignment A-MAP because

- Non user-specific A-MAP can be extended to a variable size, so incorrect decoding of non user-specific A-MAP can lead to wrong starting location of HARQ feedback and power control A-MAP. It is preferable to isolate HARQ feedback and power control A-MAP from non user-specific A-MAP decoding errors.

- Non user-specific A-MAP only controls the size of A-A-MAP. The size of HF-AMAP and PC-A-MAP is signaled in SFH, so it makes more sense to allocate non user-specific A-MAP after HF- and PC-A-MAP.

-------------------------------  Start Text Proposal ---------------------------------------------------

[ Recommended AWD Text Proposal 1 (Section 15.3.6.2.2, page 334, Figure 509) ]

- [ Recommended AWD Text Proposal 2 (Section 15.3.6.3.2.1, page 337, line 31) ]

For each Tx antenna, symbols at the output of MIMO encoder, denoted by SNUS[0] to SNUS[LNUS-1], are mapped to tone-pairs from RMP[(LHF+LPC)/2] to RMP[(LHF+LPC+LNUS)/2-1] RMP[0] to RMP[LNUS/2-1], where RMP is the renumbered A-MAP tone-pairs described in 15.3.5.3.4 and ; LHF is the number of tones required to transmit all HARQ feedback A-MAP; LPC is the number of tones required to transmit all power control A-MAP; LNUS  LNUS is the number of tones required to transmit the non-user specific AMAP IE.

- [ Recommended AWD Text Proposal 3 (Section 15.3.6.3.2.3, page 338, line 49) ]

For each Tx antenna, symbols at the output of MIMO encoder, denoted by SHF[0] to SHF[LHF-1], are mapped to tone-pairs from RMP[0] to RMP[LHF/2-1] RMP[LNUS/2] to RMP[(LNUS+LHF)/2-1], where RMP is the renumbered A-MAP tone-pairs described in 15.3.5.3.4 and LHF LHF is the number of tones required to support all HF-A-MAP channels in an A-MAP region. Clusters of HF-A-MAP are indexed sequentially from index 0 within a HF-A-MAP region in the mapping process.

- [ Recommended AWD Text Proposal 4 (Section 15.3.6.3.2.4, page 339, line 49) ]

For each Tx antenna, symbols at the output of MIMO encoder, denoted by SPC[0] to SPC[LPC-1], are mapped to tone-pairs from RMP[LHF/2] to RMP[(LHF+LPC)/2-1] RMP[(LNUS+LHF)/2] to RMP[(LNUS+LHF+LPC)/2-1], where RMP is the renumbered A-MAP tone-pairs described in 15.3.5.3.4 and LPC LPC is the number of tones required to support all HF-A-MAP channels in an A-MAP region. Clusters of PC-A-MAP are indexed sequentially in the mapping process.

-------------------------------------- End of Text Proposal -------------------------------------------