| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal for negotiation of Sleep Cycle setting together with DSx transaction (section 15.2.16.2.4) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Yeongmon Son, Hyunjeong Kang, Jungje Son, Rakesh Taori Samsung Electronics Co., Ltd. | ym1004.son@samsung.com +82-31-279-5845 |
| Re: | IEEE 802.16-09/0057, “IEEE 802.16 Working Group Letter Ballot#30a” |  |
| Abstract | The contribution proposes Sleep Cycle Setting included in AAI_DSx-REQ/RSP message for changing/switching Sleep Cycle setting. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal for negotiation of Sleep Cycle setting together with DSx transaction

Yeongmoon Son, Hyunjeong Kang, Jungje Son, Rakesh Taori

Samsung Electronics Co., Ltd.

- Introduction

In the current D2, it is described that AMS or ABS in Sleep Mode may include SCID in DSx-REQ/RSP message in order to switch the Sleep Cycle setting. That usage scenario needs to be changed

- The current Sleep Cycle setting in DSx transaction is incomplete solution because DSx-REQ/RSP message doesn’t include even Start_Frame_Number to indicate when to start Initial Sleep Cycle.

- The current Sleep Cycle setting in DSx transaction is confined to only Sleep Cycle switching.

If we clarify a little about the Sleep Cycle setting in DSx transaction, the AMS or ABS can fully negotiate Sleep Cycle setting through DSx-REQ/RSP as if they negotiate it through AAI_SLO-REQ/RSP message. As a result, we can achieve advantage such as the reduced signaling procedure and overhead.

Therefore, I propose to extend the usage scope of Sleep Cycle setting in DSx transaction as follows.

- When there is DSx transaction, an AMS or an ABS may initiate Sleep Mode negotiation together by including Sleep Cycle setting paramters in DSx-REQ

- In case that the AMS sends DSx-REQ with Sleep Cycle Setting, the ABS shall regards sleep cycle setting in DSx-REQ as negotiation of Sleep Cycle setting in SLP-REQ. Therefore, if the BS decides to accept sleep cycle setting, the ABS shall include the Response Code = 0b01(i.e. Approval) and the parameters which are different from the AMS's request. Otherwise, the ABS shall either omit entire sleep cycle setting information or include both the Response Code = 0b10and REQ_duration in DSA-RSP message.

- In case that the ABS sends DSx-REQ with Sleep Cycle setting, the AMS shall regard Sleep Cycle Setting in DSx-REQ as an unsolicited sleep mode request by the ABS. Therefore, the AMS shall apply the Sleep Cycle setting sent by ABS in the frame specified by Starf_Frame_Number in Sleep Cycle setting included in DSx-REQ sent by ABS.

- When DSx transaction is failed with Confirmation Code = Non-zero, the AMS and the ABS shall ignore the Sleep Cycle setting in DSx-REQ/RSP message, as well.

- Text to change in D2

-------------------------------  Text Start  ---------------------------------------------------

[Add the section 15.2.16.2.4 on the page 228, line 39 as follows:]

15.2.16.2.4 Sleep Mode parameter update

……..

…... The Sleep Cycle change/switching may be performed with the exchange of Service Flow Control message when the AMS is in Sleep Mode. In case that the AMS in Sleep Mode sends an AAI_DSx-REQ with Sleep Cycle Setting(refer xxx) , the ABS shall regards Sleep Cycle Setting included in the AAI_DSx-REQ as negotiation of Sleep Cycle parameters in AAI_SLP-REQ. If the ABS decides to accept sleep cycle setting, the ABS shall include the Response Code = 0b01(i.e. Approval) and the parameters which are different from the AMS's request. Otherwise, the ABS shall either omit the entire Sleep Cycle Setting or include both the Response Code = 0b10 and REQ_duration in AAI_DSx-RSP message, as rejection of the AMS’s request. If the ABS rejects Sleep Cycle setting negotiation switching while accepting creation/change/deletion of service flow, the ABS or AMS may initiate another Sleep Mode transaction to change/switch Sleep Cycle setting by AAI_SLP-REQ/RSP. On the other hand, if the ABS sends the AMS AAI_DSx-REQ with Sleep Cycle Setting, the AMS shall regard Sleep Cycle Setting in AAI_DSx-REQ as negotiation of Sleep Cycle parameters in unsolicited AAI_SLP-RSP. Therefore, the AMS shall apply the Sleep Cycle setting sent in the frame specified by Starf_Frame_Number in Sleep Cycle setting included in DSx-REQ sent by ABS.

If DSx transaction is failed with Confirmation Code = Non-zero, the AMS and the ABS shall ignore the Sleep Cycle setting in AAI_DSx-REQ/RSP message, as well

…..

Table XXX —Sleep Cycle Setting

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Sleep Cycle Setting | [145./146].x | Variable | Sleep Cycle Setting may be included in AAI_DSx-REQ/RSP message for changing/switching Sleep Cycle parameters. It contains part of sub-parameters which are defined in Table XXX+1 |

Table XXX+1 Sleep Cycle parameters in Sleep Cycle Setting

| Name | Type | Length | Value |
| --- | --- | --- | --- |
| Operation | [145./146].x.1 | 1 | Refer to the table 694 and 695. |
| SCID | [145./146].x.2 | 1 | Refer to the table 694 and 695. |
| Start Frame Number | [145./146].x.3 | 1 | Refer to the table 694 and 695. |
| LWEF | [145./146].x.4 | 1 | Refer to the table 694 and 695. |
| TIMF | [145./146].x.5 | 1 | Refer to the table 694 and 695. |
| NISCF | [145./146].x.6 | 1 | Refer to the table 694 and 695. |
| Initial Sleep Cycle | [145./146].x.7 | 1 | Refer to the table 694 and 695. |
| Final Sleep Cycle | [145./146].x.8 | 2 | Refer to the table 694 and 695. |
| Listening Window | [145./146].x.9 | 1 | Refer to the table 694 and 695. |
| New Initial Sleep Cycle | [145./146].x.10 | 1 | Refer to the table 694 and 695. |
| T_AMS | [145./146].x.11 | 1 | Refer to the table 694 and 695. |
| Response_Code | [145./146].x.12 | 1 | Refer to the table 695. |
| SLPID | [145./146].x.13 | 2 | Refer to the table 695. |
| REQ_duration | [145./146].x.14 | 1 | Refer to the table 695. |

-------------------------------  Text End  ---------------------------------------------------