| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Seamless HO Data Packet Handling (15.2.6.3.5.2) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Stavros Tzavidas Motorola | stavros.tzavidas@motorola.com |
| Re: | LB30a |  |
| Abstract | This contribution clarifies a portion of the text that refers to mutual authentication during HO. |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Seamless HO Data Packet Handling (15.2.6.3.5.2)

Stavros Tzavidas

Motorola

# Introduction

In section 15.2.6.3.5.2 “Network Re-entry Procedure” the text can be clearer on what the sender should do with transmitted packets in the various authentication scenarios.

# Text Proposal

Modify paragraph 15.2.6.3.5.2 “Network Re-entry Procedure” as indicated below:

----------------------------------------------Start of the text proposal ------------------------------------------------------

15.2.6.3.5.2 Network Re-entry Procedure

…[text omitted until the beginning of page 131]…

all cases, if the sender is authenticated, the decrypted data packets are released to the upper layer in the recipient, and if the sender is not authenticated the data packets are discarded.

If data packets are exchanged between AMS and target ABS before the AAI_RNG-REQ/RSP transaction is completed, the sender (AMS or target ABS) should store the sent data packets and not discard them before the receiver is authenticated. If a service flow exists in the reverse direction and that service flow is associated with an SA that supports data authentication (as indicated by the data authentication algorithm identifier in the cryptographic suite of the SA), the sender can authenticate the other side (the receiver) by verifying that a data packet received by the other side contains a ciphertext authentication code that was produced with the TEK associated with this SA. Alternatively, the sender can authenticate the other side (the receiver) when the AAI_RNG-REQ/RSP transaction completes successfully. In all cases, if the sender fails to authenticate the other side, then it shall consider all sent packets as never transmitted and retransmit them to a new ABS or AMS when a new connection is established.; If the sender successfully authenticates the other side, the sender shall discard the packets that were sent and stored.

-------------------------------------------------- End of the text proposal --------------------------------------------------