| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Close-Loop Power Control Message for the IEEE 802.16m AWD (16.2.3) |  |
| Date Submitted | 2009-12-31 |  |
| Source(s) | Yi Hsuan, Rongzhen Yang, Hujun Yin Intel Corporation | yi.hsuan@intel.com |
| Re: | Proposed text changes to P802.16m/D3 16.2.3 MAC Control Messages |  |
| Abstract | Add a MAC unicast control message for close loop power control configuration |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D4 Draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for Close-Loop Power Control Message for the IEEE 802.16m AWD (16.2.3)

Yi Hsuan, Rongzhen Yang, Hujun Yin

Intel Corporation

- Introduction

In 802.16m D3, the assignment mechanism of PC-AMAP-IE and other close-loop power control parameters are not defined. In the supporting slides (S80216m-09/2584), we discuss the 2 assignment methods for PC-A-MAP: explicit assignment vs. implicit assignment from the aspect of power control performance. The conclusion is that the explicit assignment is necessary for 802.16m CLPC, based on the tradeoff among signaling overhead and flexibility.

Moreover it is not clear how implicit PC-A-MAP indexing should work in TDD and H-FDD modes, where different DL to UL ratios makes implicit indexing more complicate.

The MAC management message for CLPC configuration, including PC-A-MAP assignment, is defined in the contribution.

- References

[1] Draft Amendment, P802.16m/D3, Dec 2009.

[Note to Editor: Suggested Remedy – plug in one new subsection 16.2.3.31 in page 96 of (D3) as follows]

----------------------------------------- Start of the Proposed Amendment Text --------------------------------------------

- [ Recommended AWD Text Proposal (On page 96, line 11, add a new section 16.2.3.31) ]

# 16.2.3.31 AAI_CLPC-SET (CLPC configuration setting)

An AAI_CLPC-SET message shall be transmitted by the ABS to instruct AMS to perform UL power control using PC-A-MAP IE.

--ASN1START

AAI_CLPC-SET Message ::= SEQUENCE {

CLPCDuration			INTEGER (0..15)

StartFrameOffset			INTEGER (1..32)

SubframeOffset			INTEGER (0..7)

AssignmentPeriod		INTEGER (0..7)

indexOfPCAMAPIE 		INTEGER (0..N-1), where N is the number of PC-A-MAPs in a subframe

deltaStepIndication		INTEGER (0..1)

}

--ASN1STOP

AAI_CLPC-SET field descriptions

CLPCDuration The definition of the CLPC assignment duration, the duration length is calculate by: Duration Length = 2^(CLPCDuration Value) frames

StartFrameOffset The assignment will be effective after “StartFrameOffset” frames

SubframeOffset The subframe location of assigned PC-A-MAP IE

AssignmentPeriod The assignment period of PC-A-MAP IE, defined as Period Length = 2^( AssignmentPeriod value) frames

indexOfPCAMAPIE PC-A-MAP index in a subframe

deltaStepIndication: It define the power changes step for CLPC 0: delta power change is defined as 0.5 dB ( -0.5 dB, 0, 0.5 dB, 1 dB) 1: delta power change is defined as dB ( -1dB, 0, 1 dB, 2 dB)

------------------------------------------- End of the Proposed Amendment Text --------------------------------------------