| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Femto ABS Low-duty Operation Mode (15.4.10.2) |  |
| Date Submitted | 2009-13-06 |  |
| Source(s) | Guang Han, Stavros Tzavidas Motorola | guang.han@motorola.com stavros.tzavidas@motorola.com |
| Re: | LB 30a |  |
| Abstract | This contribution proposes solutions for supporting Femtocell BSs in 16m |  |
| Purpose | For review and adoption |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Femto ABS Low-duty Operation Mode (15.4.10.2)

Guang Han, Stavros Tzavidas

Motorola

# Introduction

Section 15.4.10.2 of AWD defines default LDM pattern and proposes that three parameters are included in the Default LDM pattern:

–Available interval (in units of four frames)

–Unavailable interval (in units of four frames)

–Start superframe number.

However, the Start superframe number of the femto ABS can not be used by the AMS when it is outside the femto ABS coverage. To solve this problem, we think the offset between the femto ABS superframe number and the overlay ABS superframe number shall also be included in the Default LDM pattern.

# Text Proposal

Modify the following text in802.16m amendment working document (P802.16m_D1) section15.4.6:

----------------------------------------------Start of the text proposal ------------------------------------------------------

15.4.10.2 Default LDM pattern(s)

A Sequence of availability and unavailability intervals forms an LDM pattern. The Default LDM pattern is the iteration of one available interval and one unavailable interval. An available interval for Default LDM pattern begins with the frame including the PA-Preamble. The Default LDM pattern is a subset of the available intervals at the femto ABS.

The Default LDM pattern parameters include:

–Available interval (in units of four frames)

–Unavailable interval (in units of four frames)

–Start superframe number

–Offset between the femto ABS superframe number and the overlay ABS superframe number

Once a femto ABS enters low duty mode, the Default LDM pattern of the femto ABS may be activated.

There may be one or more default LDM patterns in a femto ABS deployment. The Default LDM pattern(s) parameters can be pre-provisioned or unicasted to the during initial network entry with the femto ABS in the AAI_REG-RSP message.

-------------------------------------------------- End of the text proposal --------------------------------------------------