| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text on coverage loss (section 15.2.10) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Yeongmon Son, Hyunjeong Kang, Jungje Son, Rakesh Taori Samsung Electronics Co., Ltd. | ym1004.son@samsung.com |
| Re: | Counter proposal to the proposed text on coverage loss submitted by Clearwire |  |
| Abstract | This contribution provides 16e-like solution for detection and recovery of Link Loss |  |
| Purpose | Discussion and adoption by TGm |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text on Coverage Loss

Yeongmoon Son, Hyunjeong Kang, Jungje Son, Rakesh Taori

Samsung Electronics Co., Ltd.

# Introduction

I am trying to utilize 16e-based solution for detection and recovery of Link Loss. It is much simpler in standard and implementation perspective.

# Proposed Amendment text

Black text: existing text in AWD (P802.16m/D2)

Red Strikethrough text: deleted text

Blue text: new text

[Insert the below new section]

15.2.10 Coverage loss

An AMS may lose signal temporarily due to various reasons, such as entering into an area without coverage and fading. A coverage loss refers to such a situation.

15.2.10.1 Coverage loss detection at AMS and AMS’s behavior

When an AMS newly begins a synchronization procedure with an ABS, the AMS shall start the Lost SFH Interval (refer to Table xxx) which is reset whenever the AMS decodes SFH from the ABS.

If the AMS cannot decode contiguous SFHs until Lost SFH Interval expires, the AMS shall regard it as Link Loss from the ABS. In this case, the AMS shall start Resource Retain Time (refer to the table 677) provided by the ABS in AAI_REG-RSP message and try to reestablish synchronization with any ABS (i.e. ABS re-selection).

15.2.10.2 Coverage loss detection at ABS and ABS’s behavior

For each AMS, the ABS shall maintain a T27 timer which is reset whenever the ABS receives any data (e.g. UL MPDU or feedback information) from the AMS.

The ABS shall start T27 timer when the ABS sends RNG-RSP message with Ranging Status = 3 (i.e. Success) in response to RNG-REQ message sent from the AMS. At each expiration of the timer, the ABS shall grant UL burst to the AMS and wait for any MPDU on the granted UL burst from the AMS. The timer is restarted each time a unicast grant is made to the AMS. When the AMS is assigned UL burst, the AMS shall transmit a padding PDU or MPDU on the granted UL burst. If the ABS doesn’t receive any data until ‘T27_UL_grant retries’ are exhausted, the ABS shall regard it as Link Loss with the AMS. In this case, the ABS shall start Resource Retain Time specified in AAI_REG-RSP message and stop the DL/UL scheduling for the AMS until the ABS detects a return-back of the AMS from Link Loss (i.e. reception of any data from the AMS).

Even if the ABS recognizes the AMS is successfully connected to other ABS, the ABS shall keep the AMS’s context at least until Resource Retain Time expires

15.2.10.3 Coverage loss recovery procedure

When the AMS successfully finishes a PHY synchronization procedure with a preferred ABS, the AMS may perform the network re-entry (refer to 15.2.6.3.5 and ) with the ABS if the Resource Retain Time associated with the previous Serving ABS doesn’t expires yet. If the Resource Retain Time expired, the AMS shall perform the initial network entry with the ABS.

When the ABS receives RNG-REQ message which the AMS transmits according to network re-entry procedure, if the ABS cannot get the AMS context from the serving ABS specified in the RNG-REQ message, the ABS shall instruct the AMS to perform the full network entry by sending RNG-RSP message (see Full network entry w/ or w/o traffic IP address refresh in page 460 and 461 of IEEE802.16-2009). In that case, the AMS doesn’t need to transmit RNG-REQ message again.

15.11. Parameters and constants

15.11.1 Global values

The ABS and AMS shall meet the requirements contained in Table xxx.

Table xxx —Parameters and constants

| System | Name | Time reference | Minimum Value | Default value | Maximum value |
| --- | --- | --- | --- | --- | --- |
| AMS | Lost SFH Interval | Maximum time since last received SFH before DL synchronization is considered lost | - | - | TBD[e.g. 320ms (i.e. 16 x the length of a SuperFrame)] |
| ABS | T27 | time between unicast grants to AMS | TBD[e.g. 100ms] |  |  |
| ABS | T27_UL_grant retries | Maximum Number of UL grants | TBD[e.g. 16] | TBD[e.g. 16] | - |