| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Transmission of Broadcast MAC Control Messages (15.2.4.7) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Eunjong Lee, Jeongki Kim, Youngsoo Yuk, Kiseon Ryu and Jinsoo Choi LG Electronics | Mehappy3@lge.com, chegal@lge.com, sixs@lge.com, ksryu@lge.com, emptylie@lge.com |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | Proposes text for Transmission of Broadcast MAC Control Messages. |  |
| Purpose | To be adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text on Transmission of Broadcast MAC Control Messages (15.2.4.7)

Eunjong Lee, Jeongki Kim, Youngsoo Yuk, Kiseon Ryu and Jinsoo Choi

- LG Electronics

- Introduction

This contribution proposes the transmission scheme of broadcast MAC control messages.

In current D2, there is no specific text related to transmission of broadcast MAC control messages.

To minimize the signaling overhead, all broadcast message should be sent in a burst which can be known to the AMSs.  If we have a dedicated burst for broadcast messages, the FID field is not required anymore. In addition, all broadcast messages are not always required for all AMSs. In addition, EH field is also not required.

We propose to use these unnecessary fields for other purpose to maintain the header structure. That is to indicate the types of the MAC control messages. It can help to know where the MAC control message exists.

In case of multiplexed MAC control messages, we propose to use broadcast MEH (The structure is similar with original MEH except Message Type field) to inform the type of the

Figure 1. MPDU structure of the broadcast MAC control message

- Proposed Text

------------------------------------------------------Proposed Text  1-------------------------------------------------------

[On the IEEE P802.16m/D2, page 585, line 24, add the following subsections]

15.2.4.7 Transmission of Broadcast MAC Control Messages

There is only one burst which includes broadcast MAC control message in a subframe, if exist. All broadcast MAC control messages need to be transmitted in a subframe shall be multiplexed or be concatenated into a burst.

The type value of 1~31 of MAC control message are reserved only for the broadcast MAC control messages.

When one broadcast MAC control message is shown in the burst, the FID field and the EH field in the AGMH are interpreted as the type of the messages.

When multiple broadcast MAC control messages are transmitted in a burst, the FID field and the EH field in the AGMH are set to 0.  A broadcast multiplexing extended header (BMEH) follows AGMH for indicating multiplexing structure. The BMEH carries the type and the length of the corresponding broadcast MAC control messages.

The type field of broadcast MAC control message is not included in the message.

------------------------------------------------------Proposed Text 1--------------------------------------------------------

------------------------------------------------------Proposed Text 2--------------------------------------------------------

[On the IEEE P802.16m/D2, page 24, line 9, add the following subsections]

15.2.2.2.4 Broadcast Multiplexing Extended Header (BMEH)

The format of BMEH is defined in Table 666. The BMEH is used when multiple broadcast MAC control messages are multiplexed in the same MAC PDU. The FID of broadcast MAC control messages are always ‘0’, and BMEH carries the type and the length of the corresponding broadcast MAC control messages.

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| BMEH(){ |  |  |
| Last | 1 | 0 = another extended header follows BMEH 1 = another extended header does not follow BMEH |
| Type | TBD | BMEH Type |
| Num Messages | 4 | Number of Messages present in the BMEH |
| for(i=0;i<Num Messages; i++ ){ |  |  |
| Message Type | 5 | Message Type of the broadcast MAC control message. |
| Length | 11 | Length of the broadcast MAC control messages. |
| } |  |  |
| Reserved | Variable |  |
| } |  |  |

------------------------------------------------------Proposed Text 2--------------------------------------------------------