| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to Use a Single CRC for Both P-SFH and S-SFH in the 802.16m AWD (15.3.6) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Yi Hsuan, Tom Harel, Hujun Yin, Shantidev Mohanty Intel Corporation | yi.hsuan@intel.com |
| Re: | Category: AWD comments / Area: Chapter 15.3.6 (DL-CTRL) “Comments on AWD 15.3.6 DL-CTRL” |  |
| Abstract | The contribution proposes to use a single CRC for both P-SFH and S-SFH |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to Use a Single CRC for Both P-SFH and S-SFH

Yi Hsuan, Tom Harel, Hujun Yin, Shantidev Mohanty

Intel Corporation

- Introduction

In the current SFH physical channel design, a P-SFH CRC is generated based on P-SFH IE and encoded together with P-SFH IE. A separate CRC is generated based on S-SFH IE and encoded with S-SFH IE. When P-SFH and S-SFH exist together in a subframe, two CRCs create extra overhead and reduce the overall SFH IE capacity. This contribution is to propose a design where P-SFH and S-SFH can share one CRC, thus providing more room for S-SFH IE.

- How to Avoid CRC in P-SFH

In the current AWD text, two CRCs are appended to and coded with P-SFH and S-SFH respectively. As shown in the following figure, if P-SFH and S-SFH coexist in one superframe, two CRCs are needed, while one CRC is enough to provide error detection for both P-SFH and S-SFH. Different colors in the figure stand for different code blocks.

Superframe 1

Superframe 2

Superframe 3

Superframe 4

P-SFH

CRC

S-SFH SP1

CRC

P-SFH

CRC

P-SFH

CRC

S-SFH SP2

CRC

P-SFH

CRC

S-SFH SP1

CRC

Figure 1: CRC in P-SFH and S-SFH of the current design.

Figure 2 illustrates our proposal to use a single CRC for both P-SFH and S-SFH. The proposal has one CRC coded with S-SFH only. The CRC is generated from both P-SFH IE and S-SFH IEs of the same superframe. At the receiver side, MS decodes P-SFH first and then decodes S-SFH based on the information in P-SFH. If decoding error happens in either P-SFH or S-SFH, the CRC check in S-SFH would fail. In some superframes, there may be no S-SFH IE at all, in this case, a CRC is generated, coded and transmitted separately to allow error detection for P-SFH as shown in superframe 4 of Figure 2.

P-SFH

S-SFH SP1

CRC

P-SFH

CRC

Superframe 1

Superframe 2

Superframe 3

Superframe 4

P-SFH

S-SFH SP2

CRC

P-SFH

S-SFH SP1

CRC

Figure 2: Proposal to use a single CRC for P-SFH and S-SFH

-------------------------------  Start Text Proposal ---------------------------------------------------

- [ Recommended AWD Text Proposal 1 (Section 15.3.6.3.1.1, page 336, line 1) ]

Figure 510 – Physical processing block diagram for the P-SFH

- The P-SFH IE shall be appended with NCRC,P-SFH bits CRC.

- The resulting sequence of bits P-SFH IE shall be encoded by the TBCC described in << 15.3.12.2>> 15.3.11.2 with parameter …

- [ Recommended AWD Text Proposal 2 (Section 15.3.6.3.1.1, page 336, line 48) ]

The S-SFH IE shall be appended with a 16-bit CRC. The CRC is generated as described in 15.3.11.1.1, based on concatenation of P-SFH and S-SFH IE in the same subframe with the first bit of P-SFH IE as MSB and the last bit of S-SFH IE as LSB. If S-SFH IE is not present, the CRC is generated based on P-SFH IE only and passed to the next processing block without S-SFH IE.

-------------------------------------- End of Text Proposal -------------------------------------------