| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Methods For Explicit Listening Window Extension by AMS during Sleep Operation in IEEE 802.16m |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Shantidev Mohanty, Maruti Gupta, Muthaiah Venkatachalam Intel Corporation | E-mail: shantidev.mohanty@intel.com |
| Re: |  |  |
| Abstract | This contribution proposes methods using which AMS in sleep mode can ensure that it has up-to-date system information in IEEE 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m Letter Ballot 30a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Methods For Explicit Listening Window Extension by AMS during Sleep Operation in IEEE 802.16m

Shantidev Mohanty, Maruti Gupta, Muthaiah Venkatachalam

- Intel Corporation

- Introduction

Currently the explicit signaling to extend the listening window is sent by the ABS. This comment proposes the AMS to have the ability to use explicit signaling to extend/terminate the listening window.

- Text proposal for inclusion in the P802.16m/D2

========================== Start of Proposed Text ==============================

[Editor’s Note: Add the following text after line 16 in page 228 in “15.2.16.2.3.2 Listening Window extension”]

The MBS may send an explicit signaling in Sleep Control Extended header or Sleep Control header including the number of frame for extended listening window to control extension of Listening Window during the Default Listening Window. Upon receiving the explicit signaling, the ABS shall extend its Listening Window until the frame specified in signaling or terminate the listening window.

[Editor’s Note: Add the following text after line 18 in page 228 in “15.2.16.2.3.2 Listening Window extension”]

The AMS may send an explicit indication using Sleep Control Extended Header or Sleep Control Header to terminate the current Listening Window. When an AMS has a last PDU in the DL buffer during the listening window, the AMS may transmit an explicit indication provided that it allows to terminate the current Listening Window. In this case, the ABS shall regard the AMS as returning to sleep (i.e. the Sleep Window starts).

========================== End of Proposed Text ==============================