| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Collision Avoidance in UL HARQ (15.2.14.2.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Hyunkyu Yu, Mihyun Lee, Jaeweon Cho Samsung Electronics | hk.yu@samsung.com |
| Re: | Comments on IEEE P802.16m/D2 for IEEE 802.16 Working Group Letter Ballot Recirc #30a |  |
| Abstract | The contribution proposes text changes IEEE P802.16m/D2 in Section 15.2.14.2.1 (HARQ Signaling) |  |
| Purpose | To be discussed and adopted by the IEEE 802.16 Working Group |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16 |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Collision Avoidance in UL HARQ (15.2.14.2.1)

Hyunkyu Yu, Mihyun Lee, Jaeweon Cho

Samsung Electronics

Introduction

In UL HARQ, the collision may happen between Persistent Allocation (PA) initial data burst and retransmission data burst because of different timing interval. In current D2, it is only allowed to change the resource index in the same subframe. However, this cannot solve the whole problem. This contribution proposes skip instruction as a solution.

Skip instruction is a command from ABS to AMS not to transmit a HARQ subpacket in the corresponding subframe, and it is only related to retransmission. To perform this operation, we recommend to use the UL basic assignment A-MAP IE with slight modification. In current D2, because the HARQ feedback channel region has minimum 6 channels and the size of UL control channel region is indicated through S-SFH, UL control channel region is present every UL subframe and it occupies more than or equal to 1 LRU. Thus, RI that indicates the highest LRU index (24/48/96 for 5MHz/10MHz/20MHz) will not be used to allocate data bursts and it can be replaced to indicate skip operation.

Text Proposal

[Remedy1: Adopt the text changes described below starting on page 198, line 55]

________________________________________Begin proposed text with markup_________________________________________

# 15.2.14.2.1.2 Uplink

…

In the retransmission procedure, if AMS does not receive a UL Basic Assignment A-MAP IE for the HARQ data burst in failure, AMS shall transmits the next subpacket through the resources in the next frame at the same subframe resource location assigned to the previous subpacket transmission with the same ACID. A UL Basic Assignment A-MAP IE may be sent to signal control information for retransmission with the corresponding ACID and AI_SN being not toggled. Upon receiving the UL Basic Assignment A-MAP IE, AMS shall perform the HARQ retransmission as instructed in this UL Basic Assignment A-MAP IE. As an example, ABS may change the resource index of the HARQ data burst or may command to skip retransmission in the corresponding subframe.

__________________________________________End proposed text with markup________________________________________

[Remedy2: Adopt the text changes described below starting on page 365, line 40]

________________________________________Begin proposed text with markup_________________________________________

| … | … | … |
| --- | --- | --- |
| Resource Index | 11 | 5MHz: 0 in first 2 MSB bits + 9 bits for resource index 10MHz: 11 bits for resource index 20MHz: 11 bits for resource index Resource index includes location and allocation size If all bits are set to 1, the AMS shall not transmit the corresponding HARQ subpacket for the ACID in this IE. The retransmission number is incremented by one for the skipped retransmission. |
| … | … | … |

__________________________________________End proposed text with markup________________________________________