| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Additional Frame Configuration for Legacy Support Mode |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Jaeweon Cho, Hokyu Choi Samsung Electronics | jaeweon.cho@samsung.com |
| Re: | IEEE 802.16-09/0057, “IEEE 802.16 Working Group Letter Ballot #30a” |  |
| Abstract | This contribution proposes additional frame configurations for legacy support mode. The proposed frame configuration allows a different mix ratio of 16e and 16m between adjacent ABSs while keeping the same frame offset. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Additional Frame Configuration for Legacy Support Mode

Jaeweon Cho, Hokyu Choi

Samsung Electronics

- Introduction

In this contribution, we propose additional frame configurations for legacy support mode. The proposed frame configuration allows a different mix ratio of 16e and 16m between adjacent ABSs while keeping the same frame offset, so that loading of 16e and 16m terminals can be balanced in each cell and the preamble and SFH timing is kept aligned among ABSs as well.

- Additional frame configurations for legacy support

The current frame configurations defined in P802.16m/D2 comprise one legacy zone and one 16m zone in DL, and the same frame offset shall be applied to all BSs in the same deployment region, i.e. the same division between legacy zone and 16m zone shall be applied to the all ABSs.

In order to allow different divisions of 16e and 16m among adjacent ABSs while keeping the same timing of preamble and SFH, we may introduce another legacy zone following the 16m zone. Figure 1 depicts a frame configuration having such two legacy zones and one 16m zone. We can see in this figure that by operating two separate legacy zones, a different mix ratio of 16e and 16m between adjacent ABSs is allowed while fixing the frame offset.

Figure 1. Frame configurations for 16e and 16m mix mode

- Proposed Text Changes in P802.16m/D2

[Remedy 1: Insert the following text, into line 60 on the page 277, in 15.3.3.5.1]

The subset of DL AAI subframes dedicated to the WirelessMAN-OFDMA operation should either one group of contiguous DL AAI subframes or two separate groups of contiguous DL AAI subframes. When comprising the two separate groups, the second group shall include the last DL AAI subframe.

[Remedy 2: Change the content of Table 765, on the page285, in 15.3.3.8, as follows:]

Table 765 – Frame Configuration and Indexing (5/10/20MHz channel bandwidth).

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN- - OFDMA Support |  |  | - Subframe Type |  |  |  |  |  |  |  | - TTG /RTG - (s) |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix | - UL MUX | - UL Mix | - #0 | - #1 | - #2 | - #3 | - #4 | - #5 | - #6 | - #7 |  |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |
| - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … | - … |
| - 11 | - 5/10/20 | - 1/8 | - 5 | - TDD | - 5:3 | - 3:2 (3) | - FDM | - 21:14 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - Not used | - 105.714 - / 60 |
| - 12 | - 5/10/20 | - 1/8 | - 6 | - TDD | - 5:3 | - 3:2 (3) | - TDM | - 2:1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - Not used | - 105.714 - / 60 |
| - 13 | - 5/10/20 | - 1/8 | - 7 | - TDD | - 5:3 | - 2:3 (2) | - FDM | - 14:21 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 14 | - 5/10/20 | - 1/8 | - 8 | - TDD | - 5:3 | - 2:3 (2) | - TDM | - 1:2 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 - / 60 |
| - 15 | - 5/10/20 | - 1/8 | - 9 | - TDD | - 5:3 | - 3:2 (2) | - FDM | - 21:14 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - Not used | - 105.714 / 60 |
| - 16 | - 5/10/20 | - 1/8 | - 10 | - TDD | - 5:3 | - 3:2 (2) | - TDM | - 2:1 | - DL Type1 | - DL Type1 | - Not used | - Not used | - Not used | - UL Type1 | - Not used | - Not used | - 105.714 / 60 |
| - 1517 | - 5/10/20 | - 1/8 | - 911 | - TDD | - 5:3 | - 1:4 (1) | - FDM | - 8:27 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 1618 | - 5/10/20 | - 1/8 | - 1012 | - TDD | - 5:3 | - 1:4 (1) | - TDM | - 1:2 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - Not used | - UL Type1 | - UL Type1 | - Not used | - 105.714 - / 60 |
| - 1719 | - 5/10/20 | - 1/8 | - 1113 | - TDD | - 6:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - 208.571 - / 60 |
| - 1820 | - 5/10/20 | - 1/8 | - 1214 | - TDD | - 5:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - 208.571 - / 60 |
| - 1921 | - 5/10/20 | - 1/8 | - 1315 | - TDD | - 5:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type3 | - DL Type3 | - DL Type3 | - UL Type1 | - UL Type1 | - UL Type1 | - 311.428 / 60 |
| - 2022 | - 5/10/20 | - 1/4 | - 0 | - TDD | - 5:2 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 |  | - 139.988 - / 60 |
| - 2123 | - 5/10/20 | - 1/4 | - 1 | - TDD | - 4:3 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 139.988 - / 60 |
| - 2224 | - 5/10/20 | - 1/4 | - 2 | - TDD | - 3:4 | - N/A | - N/A | - N/A | - DL Type1 | - DL Type1 | - DL Type1 | - UL Type1 | - UL Type1 | - UL Type1 | - UL Type1 |  | - 139.988 - / 60 |
| - 2325 | - 5/10/20 | - 1/4 | - 3 | - FDD | - N/A | - N/A | - N/A | - N/A | - D/U Type1 | - D/U Type1 | - D/U Type1 | - D/U Type2 | - D/U Type1 | - D/U Type1 | - D/U Type1 |  | - N/A |