| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for Paging Operation in Multi-carrier Operation in 802.16m (15.2.8.2.10.2) |  |
| Date Submitted | 2009-11-19 |  |
| Source(s) | Masoud Olfat Clearwire Muthaiah Venkatachalam, Etemad Kamran, Shantidev Mohanty Intel Corporation Giwon Park, Eunjong Lee, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim and Jin Sam Kwak LG Electronics Hyunjeong Kang Samsung Electronics | E-mail: shantidev.mohanty@intel.com gwpark@lge.com hyunjeong.kang@samsung.com |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.8.2.10.2” |  |
| Abstract | This contribution proposes amendment text describing paging operation in multi-carrier system for ABS Type in 802.16m. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m LB#30a. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

- Proposed Text for Paging Operation in Multi-carrier Operation

- in 802.16m (15.2.8.2.10.2)

Masoud Olfat

Clearwire

Muthaiah Venkatachalam, Etemad Kamran, Shantidev Mohanty

- Intel Corporation

Giwon Park, Eunjong Lee, Kiseon Ryu, Youngsoo Yuk, Ronny Yongho Kim and Jin Sam Kwak

- LG Electronics

Hyunjeong Kang

Samsung Electronics

- Introduction

This contribution proposes amendment text describing the paging operation when a 16m ABS supports multiple carriers.

.

- References

- IEEE P802.16-2009 “IEEE Standard for Local and Metropolitan Area Networks: Air Interface for Broadband Wireless Access,”.

- IEEE 802.16m-08/003r9, “The Draft IEEE 802.16m System Description Document”

- IEEE P802.16m/D1, “Part 16: Air Interface for Fixed and Mobile Broadband Wireless Access Systems – DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for inclusion in the P802.16m/D2

===============   Start of Proposed Text ==========================

[Note to Editor: Add the following text after line 33 in Page 159 in 15.2.8.2.10.2]

- 15.2.8.2.10.2 Idle Mode

In a multi-carrier system the PGID-Info message is transmitted in all the carriers. The AAI_PAG-ADV message for an AMS shall be transmitted in only one of the carriers. An idle mode AMS determines the carrier index for monitoring the paging message within AMS’s paging listening interval where its paging message is transmitted using the following equations:

Paging carrier index = DID modulo N  ----Eq (x)

The value of N is the number of carriers per PGID or a set of PGIDs in a ABS that are used for transmitting paging message for an idle mode AMS. This can be all the carriers or a subset of carriers used by a multi-carrier ABS. Paging carrier indication bit is used to specify if a carrier is a paging carrier or not. When the Paging carrier indication =1, then the corresponding carrier is paging carrier. The Paging carrier indication of different carriers is included in the PGID-Info message and may be included in the AAI_NBR-ADV message.

When an idle mode AMS moves to a new ABS and determines that the SFH change count of the new ABS is same as the information it received through the AAI_NBR-ADV message, then the AMS use the paging carrier indication received through AAI_NBR-ADV message if this paging carrier indication is included in the AAI_NBR-ADV message. If paging carrier indication is not included in the AAI_NBR-ADV message, then the AMS uses the paging carrier indication in the PGID-Info message.

When the AMS uses paging carrier indication in the PGID-Info message, the time gap between PGID_info message and the AAI_PAG-ADV message is more than one frame.

For an E-MBS AMS the AAI_PAG-ADV message may be transmitted in the same carrier as the dedicated carrier for E-MBS. In this case, the AMS does not use Eq. (x).

[Note to Editor: Add the following text after line 30 in Page 49 in 15.2.3.11]

- 15.2.3.11 AAI_NBR-ADV

For each ABS, Multicarrier capability and configurations for paging carrier indication are carried as follow

- Number of physical carriers

- Physical carrier index

- Paging carrier indication

=================== End of Proposed Text =======================