| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Text Proposal for SFH content change and update procedures for IEEE 802.16m (15.2.23) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Sassan Ahmadi, Hujun Yin, Aran Bergman, Hsuan Yi, Shantidev Mohanty Intel | Emails: Sassan.ahmadi@intel.com Shantidev.mohanty@intel.com |
| Re: | Call for LB #30a on “ P802.16m/D2”: Target topic: “15.2.23” |  |
| Abstract | The contribution proposes the modifications to the DL Control Structure section to be included in 802.16m amendment. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Text Proposal for SFH content change and update procedures for IEEE 802.16m (15.2.23)

Sassan Ahmadi, Hujun Yin, Aran Bergman, Hsuan Yi, Shantidev Mohanty

Intel

- 1 Text Proposal for Change of S-SFH IE information fields

This contribution proposes guidelines for the duration over which the contents of the S-SFH SPs remain the same.

Proposed Text

-------------------------------  Proposed Text Start  ---------------------------------------------------

[Note to Editor: Add the following text after line 54 in Page 258 in “15.2.23 Update of S-SFH IEs”]

15.2.23.1 Change of information contents of S-SFH SPs

To avoid frequent changes to S-SFH SP contents that will adversely affect HO latency, and power saving during sleep/idle operations, the rate of change of information of S-SFH SPs have an upper bound, i.e., the information can not change more frequently than this upper bound. “Rate of change of SP (1-3) Info” field of S-SFH SP3 indicates the minimum durations over which the contents of different S-SFH SPs does not change. Table xxx shows this time for different S-SFH SPs.

Table xxx: Minimum duration over which the contents of the S-SFH SPs does not change.

| Rate of change of SP (1-3) Info | S-SFH SP1 | S-SFH SP2 | S-SFH SP3 |
| --- | --- | --- | --- |
| 0000 | 200 ms | 1 min | 30 min |
| 0001-1111: reserved |  |  |  |

-------------------------------  Proposed Text End ---------------------------------------------------