| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | MZone indication for a mixed mode ABS (15.2.6.4.1.1.1) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Fang-Ching (Frank) Ren Yung-Han Chen ITRI | frank_ren@itri.org.tw *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | This contribution proposes text on topology advertisement for MZone existence through LZone FCH |  |
| Purpose | To be discussed and adopted in IEEE 802.16m draft |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

MZone indication for a mixed mode ABS (15.2.6.4.1.1.1)

Fang-Ching (Frank) Ren, Yung-Han Chen

ITRI

- Introduction

In P802.16m/D2, subclause 15.2.6.4.1.1.1 said “An ABS uses one reserved bit in FCH to indicate the presence of its MZone.” However, there is no clear definition of which reserved bit to be used.

- Proposed Text

--------------------------------------------------------  Text Start  ------------------------------------------------------------

[Add text as follows in subclause 15.2.6.4.1.1.1]

An ABS uses one reserved bit in FCH to indicate the presence of its MZone as follows.

Table YYY: OFDMA DL Frame Prefix format in LZone of Advanced WirelessMAN-OFDMA System

[Editorial note: The following table is modified from Table 314 in IEEE Std 802.16-2009]

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| DL_Frame_Prefix_Format() { |  |  |
| Used subchannel bitmap | 6 | Bit #0: Subchannel group 0 Bit #1: Subchannel group 1 Bit #2: Subchannel group 2 Bit #3: Subchannel group 3 Bit #4: Subchannel group 4 Bit #5: Subchannel group 5 |
| Reserved MZone Indicator | 1 | Shall be set to zero 0: MZone not enabled 1: MZone enabled |
| Repetition_Coding_Indication | 2 | 0b00: No repetition coding on DL-MAP 0b01: Repetition coding of 2 used on DL-MAP 0b10: Repetition coding of 4 used on DL-MAP 0b11: Repetition coding of 6 used on DL-MAP |
| Coding_Indication | 3 | 0b000: CC encoding used on DL-MAP 0b001: BTC encoding used on DL-MAP 0b010: CTC encoding used on DL-MAP 0b011: ZT CC encoding used on DL-MAP 0b100: CC encoding with optional interleaver 0b101: LDPC encoding used on DL-MAP 0b110 to 0b111: Reserved |
| DL-Map_Length | 8 |  |
| Reserved | 4 | Shall be set to zero |
| } |  |  |

--------------------------------------------------------  Text End  ------------------------------------------------------------