| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text in Sounding Channel Power Control Section for the IEEE 802.16m/D2 (15.3.9.4.4, 15.2.3.28) |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Jeongho Park, Keunchul Hwang, Hokyu Choi Samsung Electronics | jeongho.jh.park @samsung.com |
| Re: | IEEE 802.16-09/0057, “IEEE 802.16 Working Group Letter Ballot Recirc #30a: Announcement” |  |
| Abstract | This contribution provides proposal of text modification in power control section. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text in Sounding Channel Power Control Section

for the IEEE 802.16m/D2 (15.3.9.4.4, 15.2.3.28)

Jeongho Park, Keunchul Hwang, Hokyu Choi

Samsung Electronics

- Introduction

This contribution provides proposal of text modification in power control section.

- Text proposal for the 802.16m/D2

[Change the text from line 44 to 65 on the page 515, in 15.3.9.4.4, as follows:]

============================== Start of Proposed Text =================================

15.3.9.4.4 Sounding Channel Power Control

Power control for the UL sounding channel is supported to manage the sounding quality. AMS’s transmit power for UL sounding channel is controlled separately according to its sounding channel target CSINR value. The power per subcarrier shall be maintained for the UL sounding transmission as shown in Equation (272) of 15.3.9.6.115.3.9.4.1.

In Equation (272), SINRtarget is the sounding channel target SINR, which is set according to the DL SINR of the AMS. In order to maintain the UL sounding quality, the different target SINR values are assigned according to the DL SINR of each AMS; the AMS with high DL SINR applies relatively high target SINR and the AMS with low DL SINR applies relatively low target SINR.

The SINRtarget for sounding channel shall be calculated from equation <<(246273)>> using the following parameter settings: SINRMIN is the minimum SINR requirement expected by ABS which is set to 0dBSINRmin_SOUNDING, =1.2 is the fairness and set to IoT control factor for sounding channel GAMMA_IOT_SOUNDING, SINRDL is the ratio of the downlink signal vs. noise and interference power, measured by the AMS, and parameters shall be set to 0.

When calculated sounding SINRtarget is higher than 10 dBSINRmax_SOUNDING, the SINRtarget shall be set to 10 dBSINRmax_SOUNDING.

All parameters necessary for sounding channel power control are transmitted through AAI_SCD message in 15.2.3.28.

============================== End of Proposed Text =================================

[Change the text from line 1 to 35 on the page 77 in section 15.2.3.28, as follows:]

============================== Start of Proposed Text =================================

-- ASN1START

AAI_SCDMessage :: SEQUENCE {

Change Configuration Change			INTEGER (0 ~ 15)

BS_Restart_Count					INTEGER (0 ~ 15)

SA_PreamblePartitionforBStype

Trigger TLV encoding :: = SEQUENCE{}

DefaultHORSSI_CINRaveraggingparameters :: = SEQUENCE {}

NormalizedCINR :: = SEQUENCE {}

OL_MIMO_Parameters :: = SEQUENCE {

OL-Region-ON				INTEGER (0..1) OPTIONAL

OL-Rank1-Configuration			INTEGER (0..7) OPTIONAL

SB-OL-Region-2Size			INTEGER (0..15) OPTIONAL

}

Parameters_GRA :: = SEQUENCE {}

PeriodicRNGParameters :: = SEQUENCE {}

GAMMA_IOT_FP0				INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP1				INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP2				INTEGER (0..15) OPTIONAL

GAMMA_IOT_FP3				INTEGER (0..15) OPTIONAL

Alpha (a)						INTEGER (0.. 7) OPTIONAL

Beta (ß)						INTEGER (0..1) OPTIONAL

SINRmin						INTERGER (0..15) OPTIONAL

GAMMA_IOT_SOUNDING			INTEGER (0..15) OPTIONAL

SINRmin_	SOUNDING				INTERGER (0..15) OPTIONAL

SINRmax_SOUNDING				INTERGER (0..15) OPTIONAL

}

-- ASN1STOP

============================== End of Proposed Text =================================