| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Information of available UL PUSC Subchannel for FDM based UL PUSC-Zone Support (15.3.8.3.5.3) |  |
| Date Submitted | 2009-11-06 |  |
| Source(s) | Jeongho Park, Jaeweon Cho, Hokyu Choi Samsung Electronics | jeongho.jh.park @samsung.com |
| Re: | IEEE 802.16-09/0057, “IEEE 802.16 Working Group Letter Ballot Recirc #30a: Announcement” |  |
| Abstract | This contribution proposes a method of providing information of available UL PUSC subchannels in case of FDM based UL PUSC-Zone support. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m D2 |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Information of available UL PUSC Subchannel

for FDM based UL PUSC-Zone Support (15.3.8.3.5.3)

Jeongho Park, Jaeweon Cho, Hokyu Choi

Samsung Electronics

- Introduction

This contribution proposes a method of letting AMS know what UL PUSC subchannels are available in case of FDM-based UL PUSC-Zone support. Since one PUSC subchannel consists of 6 subchannels which are distributed over whole frequency bandwidth, it is hard to find any reason for selecting particular subchannels for system operation. Therefore we propose a set of consecutive PUSC subchannel index according for available subchannels in the sector.

- Backgrounds and Proposal

In a big picture, there are two approaches for dividing PUSC subchannels into WirelessMAN-OFDMA MS’s and AMS’s.

The first approach is to have full flexibility on usage PUSC subchannels. According to IEEE802.16-2009, ‘UL allocated subchannels bitmap’ is transmitted through UCD to indicate the physical subchannels allocated to the segment in the uplink. If ABS broadcast a message like this to AMS, then ABS can have full flexibility to select available PUSC subchannels for each WirelessMAN-OFDMA MS and AMS.

However there is a problem for this. Since SFH doe not have enough room for accomodating this big size information, additional broadcast information is the only way. If this information is transmitted through ABI, the delay of initial/handover ranging will increase because AMS needs to wait until AMS gets this information for ranging. Note that currently all necessary information for ranging is included SFH for the sake of minimizing delay.

The second approach is to restrict flexibility and predefine the usable subchannelization index according to allowed subchannel portion. A drawback is that ABS cannot enjoy full flexibility of 16e/16m ratio in UL. On the other hand, there are two merits. There are no additional signaling overheads. And there will be no impact on delay in ranging process.

Taking all of what is described above into consideration, this contribution proposes a method of the second approach. The basic priciples of the proposal are

- The ratio of 16e/16m in uplink follows that in downlink

- Contiguous subchannel set is used for making a division of resource between 16e and 16m

The following tables show frame configurations in case of FDM-based PUSC-Zone support. And each configuration case includes PUSC subchannel index of 16e and 16m. For example, frame configuration index 5 in 10MHz BW provides subchannel index set {0,…,20} and {21,…,34} for 16e and 16m respectively. This shall be interpreted as PUSC subchannel index 21…34 are going to be used for AMS while index 0…20 to be used for WirelessMAN-OFDMA MS. Of course, in this case, UCD in 16e shall contain corresponding subchannels bitmap.

- Frame configuration for 5/10/20MHz with 1/8 CP.

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN- - OFDMA` Support |  |  | - PUSC Subchannel Index - for BW 10MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix | - UL MUX | - UL Mix | - 16e | - 16m |
| - 1 | - 10 | - 1/8 | - 5 | - TDD | - 5:3 | - 3:2 (3) | - FDM | - 21:14 | - {0,…,20} | - {21,…,34} |
| - 2 | - 10 | - 1/8 | - 7 | - TDD | - 5:3 | - 2:3 (2) | - FDM | - 14:21 | - {0,…,13} | - {14,…,34} |
| - 3 | - 10 | - 1/8 | - 9 | - TDD | - 5:3 | - 1:4 (1) | - FDM | - 8:27 | - {0,…,7} | - {8,…,34} |

Note that this contribution considers only 10MHz BW because 5MHz and 20MHz BW are not deployed up to now.

- Frame configuration for 7MHz with 1/8 CP

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN- - OFDMA` Support |  |  | - PUSC Subchannel Index - for BW 7MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix | - UL MUX | - UL Mix | - 16e | - 16m |
| - 1 | - 7 | - 1/8 | 3 | TDD | 3:2 | 1:2 (1) | FDM | 12:23 | - {0,…,11} | - {12,…,34} |

- Frame configuration for 8.75MHz with 1/8 CP

| - No. | - BW | - CP | - Frame configuration index | - Duplex | - D:U | - WirelessMAN- - OFDMA` Support |  |  | - PUSC Subchannel Index - for BW 8.75MHz |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  | - DL Mix | - UL MUX | - UL Mix | - 16e | - 16m |
| - 1 | - 8.75 | - 1/8 | - 4 | - TDD | - 5:2 | - 3:2 (3) | - FDM | - 21:14 | - {0,…,20} | - {21,…,34} |
| - 2 | - 8.75 | - 1/8 | - 6 | - TDD | - 5:2 | - 2:3 (2) | - FDM | - 14:21 | - {0,…,13} | - {14,…,34} |
| - 3 | - 8.75 | - 1/8 | - 8 | - TDD | - 5:2 | - 1:4 (1) | - FDM | - 8:27 | - {0,…,7} | - {8,…,34} |

- Proposed Text Changes in P802.16m/D1

[Change the text from line 16 to 37 on the page 463, in 15.3.8.3.5.3, as follows:]

-------------------------------  Text Starts  ---------------------------------------------------

### 15.3.8.3.5.3 Subchannelization for FDM based UL PUSC Zone Support

When supporting FDM based UL PUSC zone, UL subchannelization shall conform the following rules:

1)  For the WirelessMAN-OFDMA system bandwidth, all usable subcarriers given in Table 868, Table 869, and Table 870 are divided into PUSC tiles.

2)  UL PUSC subchannelization is performed as described in section <<8.4.6.2.2>>.

3)  Available subchannels for Advanced Air Interface AMS are provided in table XXX according to bandwidth and frame configuration index. AAI ABS shall transmit ‘UL allocated subchannels bitmap’ through UCD to WirelessMAN-OFDMA MS, which indicates the remained subchannel index. shall be specified through subchannel bitmap broadcasted by [system descriptor, TBD].

4)  All PUSC tiles of specified subchannels from step 3 are extended in time domain from 3 OFDM symbols to Nsym OFDM symbols, where Nsym is dependent of AAI subframe type.

5)  Based on specified subchannels of step 3 with symbol extension tiles of step 4, DRUs for Advanced Air Interface are made up.

6)  Repeat step 4 and step 5 for remained OFDMA symbols of every uplink AAI subframe.

Table XXX- Available physical subchannel index

| - BW | - Frame configuration index | - Available PUSC physical subchannel index |
| --- | --- | --- |
| - 10MHz | - 5 | - 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34 |
|  | - 7 | - 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34 |
|  | - 9 | - 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34 |
| - 8.75MHz | - 4 | - 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34 |
|  | - 6 | - 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34 |
|  | - 8 | - 8, 9, 10, 11, 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34 |
| - 7MHz | - 3 | - 12, 13, 14, 15, 16, 17, 18, 19, 20, 21, 22, 23, 24, 25, 26, 27, 28, 29, 30, 31, 32, 33, 34 |

Overall process of subcarrier to subchannel mapping is shown in Figure 525.

-------------------------------  Text End  ---------------------------------------------------