| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text about AAI-E-MBS_CFG Message Format (15.9.3.1) |  |
| Date Submitted | 2009-11-6 |  |
| Source(s) | Chun-Yen Wang, Chun-Yuan Chiu, Fang-Ching (Frank) Ren, Chie-Ming Chou, Yung-Han Chen, ITRI Wern-Ho Sheen Chaoyang University of Technology (CYUT) | E-mail: ChunYen@itri.org.tw, ccy@itri.org.tw |
| Re: | Call for Contributions on Project 802.16m Amendment Content Topic: P802.16m/D2 |  |
| Abstract | This contribution provides a text about AAI-EMBS_CFG message format |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text about AAI-E-MBS_CFG Message Format (15.9.3.1)

Chun-Yen Wang, Chun-Yuan Chiu, Fang-Ching (Frank) Ren, Chie-Ming Chou, Yung-Han Chen,

ITRI

Wern-Ho Sheen

Chaoyang University of Technology

# Proposed Text

The following text is proposed to be captured in the IEEE P802.16m/D2 document.

------------------------------------------------------- Start of the proposed text ------------------------------------------------

- 15.9.3.1 E-MBS Configuration Indicators

The information regarding E-MBS configuration is transmitted to the AMSs using E-MBS configuration indicators. While most E-MBS configuration indicators necessary for E-MBS operation are of interest only to the AMSs interested in E-MBS, a few E-MBS related configuration information may be broadcasted to all AMSs.

The E-MBS configuration information necessary for E-MBS operation include E-MBS-Zone_IDs of serving and neighboring ABSs, E-MBS MAP time offset (frequency offset), E-MBS MAP resource allocation, E-MBS MAP Isizeoffset, E-MBS MAP MIMO Mode and MSTID and FID mappings between serving E-MBS Zone and neighboring E-MBS Zone for the same content.  Some of these E-MBS configuration information are transmitted in a new MAC management message called the AAI-E-MBS_CFG message to be read only by the E-MBS AMSs.

Table xxx: AAI-E-MBS_CFG Message Format

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| AAI-E-MBS_CFG () { | - | - |
|  | [TBD] |  |
|  |  | - |
| } |  |  |
| Padding | variable | Padding to reach byte boundary |

-------------------------------------------------------- End of the text ------------------------------------------------------