| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text about AAI-E-MBS_CFG Message Format (15.9.3.1) |  |
| Date Submitted | 2009-11-13 |  |
| Source(s) | Chun-Yen Wang, Chun-Yuan Chiu, Fang-Ching (Frank) Ren, Chie-Ming Chou, Yung-Han Chen, ITRI Wern-Ho Sheen Chaoyang University of Technology (CYUT) | E-mail: ChunYen@itri.org.tw, ccy@itri.org.tw |
| Re: | Call for Contributions on Project 802.16m Amendment Content Topic: P802.16m/D2 |  |
| Abstract | This contribution provides a text about AAI-EMBS_CFG message format |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text about AAI-E-MBS_CFG Message Format (15.9.3.1)

Chun-Yen Wang, Chun-Yuan Chiu, Fang-Ching (Frank) Ren, Chie-Ming Chou, Yung-Han Chen,

ITRI

Wern-Ho Sheen

Chaoyang University of Technology

- Introduction

Current P802.16m/D2 [1] specifies (in Section 15.9.3.1) some E-MBS configuration information are transmitted in a new MAC management message called the AAI-E-MBS_CFG message to be read only by the E-MBS AMSs for E-MBS operation. However, the message format for AAI_EMBS-CFG is not specified in P802.16m/D2. This contribution provides a proposal for AAI-EMBS_CFG message format.

- References

- IEEE P802.16m/D1, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks; Part 16: Air Interface for Broadband Wireless Access systems; Advanced Air Interface,” October 2009

- Proposed Text

------------------------------------------------------- Start of the proposed text ------------------------------------------------

[Add the following subsection in the end of 15.2.3:]

- 15.2.3.x AAI_E-MBS-CFG

AAI-E-MBS_CFG message is read only by the E-MBS AMSs for E-MBS operation. The AAI_HO-CMD message shall include the following parameters:

Table xxx – parameters for AAI_E-MBS-CFG message

| Name | Value | Usage |
| --- | --- | --- |
| Num_E-MBS_Zones of serving ABS | Number of E-MBS Zones with which the serving ABS is associated. |  |
| E-MBS_Zone_ID of serving ABS |  | Shall be included for each E-MBS Zone when Num_E-MBS_Zones of serving ABS > 0 |
| E-MBS MAP allocation information | E-MBS MAP allocation information of each E-MBS Zone with which the serving ABS is associated, which includes: E-MBS MAP time offset (frequency offset); E-MBS MAP resource allocation; E-MBS MAP Isizeoffset; E-MBS MAP MIMO Mode; Physical Carrier Index. | Shall be included for each E-MBS Zone when Num_E-MBS_Zones of serving ABS > 0 |
| Num_Neighboring_ABSs | Number of neighboring ABSs |  |
| Num_E-MBS_Zones of neighboring ABS | Number of E-MBS Zones with which the neighboring ABS is associated. | Shall be included for each neighboring ABS when Num_Neighboring_ABSs > 0 |
| E-MBS_Zone_ID of neighboring ABS |  | Shall be included for each E-MBS Zone when Num_E-MBS_Zones of neighboring ABS > 0 |
| Mapping Indication | 1: The mappings of MSTID and FID between serving E-MBS Zone and neighboring E-MBS Zone for the same content is included in this message 0: no mapping information |  |
| Num_E-MBS_Flows | Number of E-MBS Flows which have mapping information | Shall be included for each neighboring ABS when Mapping Indication = 1 |
| E-MBS Flow Mapping Information | Mapping information of each E-MBS flow, which includes: Original E-MBS Zone Identifier; Original Multicast STID; Original Flow ID; New E-MBS Zone Identifier; New Multicast STID; New Flow ID; Physical Carrier Index of the neighboring ABS where the E-MBS flow is provided. | Shall be included when N_E-MBS_Flow > 0 |

-------------------------------------------------------- End of the text ------------------------------------------------------