| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes Related to TDD HARQ timing in 802.16m/D2 (15.2.14.2.2.2) |  |
| Date Submitted | 2009-11-07 |  |
| Source(s) | Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung ETRI Mihyun Lee, Jaeweon Cho, Hokyu Choi Samsung Electronics Co., Ltd. | jsp@etri.re.kr mihyun.mac.lee@samsung.com |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #30a on IEEE P802.16m/D2 |  |
| Abstract | The contribution proposes the text changes to TDD HARQ timing in 802.16m/D2. |  |
| Purpose | To discuss and adopt the proposed text in the next revision of 802.16m amendment draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes Related to TDD HARQ timing in 802.16m/D2
(15.2.14.2.2.2)

Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung ETRI

Mihyun Lee, Jaeweon Cho, Hokyu Choi, Samsung Electronics Co., Ltd.

- Introduction

This contribution proposes the text changes to TDD HARQ timing defined in the current 802.16m draft [1]. Since A-MAP regions shall be located in all DL AAI subframes and DL data allocations corresponding to an A-MAP region occupy resources in the AAI subframe where the A-MAP region is located [2], HARQ timing was changed by the contribution [3] to be consistent with the periodicity of A-MAP (i.e. NA-MAP = 1). But, the parameter K of reference interval to be used in TDD HARQ timing was not changed by the contribution [3]. So, this contribution provides the text changes for the clarification to modify the parameter K of TDD HARQ timing.

- References

- IEEE P802.16m/D2, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” 2009-10-14.

- Hujun Yin, Yi Hsuan, Roshni Srinivasan, Jing Zhu, Yujian Zhang, “Proposed 802.16m AWD Text for A-MAP periodicity,” C802.16m-09/1357, 2009-07-06.

- Mihyun Lee, Jaeweon Cho, Hokyu Choi, “Proposed changes to A-MAP Relevance and HARQ Timing (Section 15.3.9.2.2),” C802.16m-09/1931, 2009-08-29.

- Proposed Changes for inclusion in the 802.16m/D2

------------------------------------------------------ Start of the Text -------------------------------------------------------

[Change 1: Change the text in page 202, section 15.2.14.2.2.2.1 Downlink as follows]

…

DL HARQ subpacket transmission corresponding to a DL Basic Assignment A-MAP IE in l-th DL subframe of the i-th frame shall begin in the m-th DL subframe of the i-th frame. A HARQ feedback for the DL HARQ subpacket shall be transmitted in the n-th UL subframe of the j-th frame. The subframe index m, n and frame index j shall be determined by using l and i, as shown in Table 744—. In the table, if  D is less than U, K = –ceil((U – D)/2). Otherwise, K = floor((D – U)/2) . Note that the subframe index l shall range from 0 to D – 1.

Note that when a DL Basic Assignment A-MAP IE in the l-th DL subframe of the i-th frame indicates the long TTI transmission and l is 0, the subframe index m and the frame index j shall be calculated as shown in Table 744.

…

[Change 2: Change the text in page 204, 15.2.14.2.2.2.2 Uplink as follows]

…

UL HARQ subpacket transmission corresponding to a UL Basic Assignment A-MAP IE in l-th DL subframe of the i-th frame shall begin in the m-th UL subframe of the j-th frame. A HARQ feedback time for the HARQ subpacket shall be transmitted in the l-th DL subframe of the k-th frame. When the UL HARQ feedback indicates a negative acknowledgement, retransmission of the UL HARQ subpacket shall begin in the m-th UL subframe of the p-th frame. The subframe index m, n and frame index j, k, p shall be calculated as shown in Table 745—.

In the table, if D is less than U, K = –ceil((U – D)/2). Otherwise, K = floor((D – U)/2). Note that the subframe index l shall range from 0 to D – 1.

When a UL Basic Assignment A-MAP IE in the l-th DL subframe of the i-th frame indicates the long TTI transmission, the UL HARQ subpacket transmission shall begin in the 0-th UL subframe of the j-th frame. A HARQ feedback for this long TTI transmission shall be transmitted in the l-th DL subframe of the k-th frame. When the UL HARQ feedback indicates a negative acknowledgement, retransmission of the UL HARQ subpacket shall begin in the 0-th UL subframe of the p-th frame. The frame index j, k, p shall be calculated according to equation in Table 745, with replacing the subframe index m by 0.

…

------------------------------------------------------- End of the Text -------------------------------------------------------