| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Changes Related to UL Group Configuration A-MAP IE in 802.16m/D2 (15.3.6.5.2.7) |  |
| Date Submitted | 2009-11-07 |  |
| Source(s) | Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung, ETRI | Voice: +82-42-860-5748 jsp@etri.re.kr |
| Re: | IEEE 802.16 Working Group Letter Ballot Recirculation #30a on IEEE P802.16m/D2 |  |
| Abstract | The contribution proposes the text changes to UL Group Configuration A-MAP IE in 802.16m/D2. |  |
| Purpose | To discuss and adopt the text changes in the next revision of 802.16m amendment draft. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Changes Related to UL Group Configuration A-MAP IE in 802.16m/D2 (15.3.6.5.2.7)

Jisoo Park, Namsuk Lee, Sookjin Lee, Yongseouk Choi, Nakwoon Sung, ETRI

- Introduction

This contribution proposes the text changes to UL Group Configuration A-MAP IE defined in the current 802.16m draft [1]. The A-MAP regions are present in all DL AAI subframes [1][2]. Since allocation relevance indication is not required when the A-MAP region occurs every subframe, as a result, it was proposed that signaling associated with the transmission of A-MAP region every 2 frames was deleted by [3], and then it was adopted in the basic DL and UL assignment A-MAP IEs. But UL Group Configuration A-MAP was not considered for allocation relevance field, so this contribution proposes the text changes related to ‘Allocation Relevance’ in UL Group Configuration A-MAP IE. As a result of the proposed change, 2 bits can be saved by the deletion of ‘Allocation Relevance’ field.

- References

- IEEE P802.16m/D2, “Draft Amendment to IEEE Standard for Local and Metropolitan Area Networks- Part 16: Air Interface for Broadband Wireless Access Systems,” October 2009.

- Hujun Yin, Yi Hsuan, Roshni Srinivasan, Jing Zhu, Yujian Zhang, “Proposed 802.16m AWD Text for A-MAP periodicity”, C802.16m-09/1357, July 2009.

- Roshni Srinivasan, Hujun Yin, Yi Hsuan, Hyunkyu Yu, “Proposed Changes Related to Allocation Relevance in the DL and UL Basic Assignment A-MAP Information Elements in the 802.16m AWD (AWD - 15.3.6.5.2.2),” C802.16m-09/1329r1, July 2009.

- Proposed Changes for inclusion in the 802.16m/D2

------------------------------------------------------ Start of the Text -------------------------------------------------------

[Change 1: Change the text in Table 810 UL Group Configuration A-MAP IE Format, page 377 in section 15.3.6.5.2.7 as follows]

Table 810— UL Group Configuration A-MAP IE

| Syntax | Size in bits | Description/Notes |
| --- | --- | --- |
| A-MAP IE Type | 4 | UL Group Configuration A-MAP IE |
| Group ID | 5 | Indicates group identifier. |
| Lowest MCS in group | 4 | The index of the lowest MCS that shall be allowed in this group from the list of nominal MCSs defined in Table 744. |
| Highest MCS in group | 4 | The index of the highest MCS that shall be allowed in this group from the list of nominal MCSs defined in Table 744 |
| HARQ Burst Size Set ID | [2] | Indicates HARQ data burst size set supported in the group that is selected from the configured HARQ data burst size set candidates in additional broadcast message. |
| GRA Periodicity | [2] | Indicate the period of transmitting GRA A-MAP IE 0b00: 1 frame 0b01: 2 frame 0b10: 4 frame 0b11: 8 frame |
| Group MIMO Mode | 2 | Indicate Group MIMO mode set supported in the group. 0b00: SFBC 0b01: SFBC and Vertical encoding 0b10: OL MU-MIMO 0b11: reserved |
| if (NAAI subframe, A-MAP == 2){ |  |  |
| if (DL:UL != 3:5){ |  |  |
| Allocation Relevance | 1 | AAI subframe index when an A-MAP region occurs every 2 AAI subframes (NAAI subframe, A-MAP =2) 0b0: Allocation in the first DL AAI subframe relevant to an A-MAP region 0b1: Allocation in the second DL AAI subframe relevant to an A-MAP region |
| }else if (DL:UL != 3:5){ |  |  |
| Allocation Relevance | 2 | AAI subframe index when an A-MAP region occurs every 2 AAI subframes (NAAI subframe, A-MAP =2) and DL:UL AAI subframe ratio is 8:0, 6:2, 4:4 or 5:3 0b0: Allocation in the first UL AAI subframe relevant to an A-MAP region 0b1: Allocation in the second UL AAI subframe relevant to an A-MAP region |
| } |  |  |
| } |  |  |
| User Bitmap Index | [5] | Indicates User Bitmap index to the AMS. An AMS may have multiple User Bitmap Indexes in a group. |
| Initial ACID | [4] | Indicates the start of ACID used for group resource allocation. |
| N_ACID | [3] | Indicates the number of ACIDs used for group resource allocation. |
| Padding | Variable | Padding to reach byte boundary |

------------------------------------------------------- End of the Text -------------------------------------------------------