| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Clarification of inclusion of femtocell BS in the AAI_NBR-ADV message |  |
| Date Submitted | 2009-11-17 |  |
| Source(s) | Youngbo Cho, Heewon Kang, Jaehyuk Jang, Jung Je Son, Rakesh Taori Samsung Electronics | E-mail: jack.jang@samsung.com *<http://standards.ieee.org/faqs/affiliationFAQ.html> |
| Re: | IEEE 802.16-09/0057: IEEE 802.16 Working Group Letter Ballot Recirc #30a |  |
| Abstract | This contribution provides clarifications not for OSG femtocell BS not always to be included in the broadcast AAI_NBR-ADV message. |  |
| Purpose | To be discussed and adopted by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Clarification of inclusion of femtocell BS in the AAI_NBR-ADV message

Youngbo Cho, Heewon Kang, Jaehyuk Jang, Jung Je Son, Rakesh Taori
Samsung Electronics

# Introduction

The number of neighbor femtocell BSs and/or hotzone BSs would be too many, and they may not always be included in the AAI_NBR-ADV message. In addition, macro hotzone is defined in the SA-preamble section, but this type is missing in the AAI_NBR-ADV message. Current text needs to be clarified.

# Overview of macro hotzone

- Why macro hotzone is needed?

  - Increasing demands on high data rates

  - Such demands are usually concentrated at spots

  - OSG femto do not consider the access of fast moving AMSs because it is established in indoor environment in general

  - Service provider establishes it by cell planning

- Properties of macro hot zone

  - It is macro ABS with low Tx power

  - Possibly overlaid under a macro ABS with larger coverage

  - The same or different operaing frequency of macro ABS

  - Public access to all users

- Difference to OSG-femto ABS

  - Coverage is bigger than OSG-femto ABS

  - It can be established in out-door environment

    - Fast moving AMSs can access the macro hotzone ABS

    - In general, femto ABSs are established under indoor condition or distance from road

  - Service provider establishes

    - Femto ABS can be established by individual user

- Definition of Macro Hot zone BS

  - An ABS with smaller transmission power than macro ABS (bigger than femto ABS) and possibly overlaid under the coverage of another macro ABS. Macro hotzone ABS is deployed by service provider. It can be deployed in outdoor environment.

# Proposed Text

Adopt the following remedies 1, 2, 3 and 4.

----------------------------------------------------- Start of Proposed Text -----------------------------------------------------

{Remedy 1: Modify the following sentence in line 27, page 48.}

15.2.3.11 AAI_NBR-ADV

AAI_NBR-ADV message may sort neighbor ABSs (RSs) according to their deployment types, which is categorized by the following parameters:

1) ABS type (macro, micro, macro hotzone, femto, relay, TBD)

2) carrier frequency

3) MAC version

4) TDD/FDD and related definitions (expected to be the same given carrier frequency)

5) BW, CP info: may not be required if carried in P/S-SCH channel

6) Multicarrier capability and configuration

{Remedy 2: Modify the following sentences in line 63, page 120.}

15.2.6.1.1 Network topology advertisement

An ABS shall periodically broadcast the system information of the neighboring ABSs using an AAI_NBR-ADV message. A broadcast AAI_NBR-ADV message should not include Open Subscriber Group femtos and/or macro hotzone ABSs, but shall not include information of neighbor Closed Subscriber Group (CSG) femtos. Special handling of neighbor information of femto is described in section 15.2.x 15.4.8.1.1.

A serving ABS may unicast the AAI_NBR-ADV message to an AMS upon reception of the AAI_NBR-REQ message or in unsolicited manner. The AAI_NBR-ADV message may include parameters required for cell selection e.g., cell load and cell type. The ABS may broadcast different fragment of AAI_NBR-ADV independently.

{Remedy 3: Modify the following sentences in line 14, page 549.}

15.4.8.1.1 Network Topology Advertisement

Femto ABS shall broadcast a list of neighboring macro ABSs, and should not broadcast OSG femto ABSs and/or macro hotzone ABSs via AAI_NBR-ADV.

{Remedy 4: Modify the following sentences in line 42, page 4, subclause “3. Definitions”.}

3.122 Macro hotzone ABS: An ABS with smaller transmission power than macro ABS’s (larger than femto ABS’s) and possibly overlaid under the coverage of another macro ABS. Macro hotzone ABS is deployed by service provider. It can be deployed in outdoor environment.

------------------------------------------------------ End of Proposed Text ------------------------------------------------------