| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text for AAI_LBS-ADV messages for location based service (15.8.2.4.2) |  |
| Date Submitted | 2009-11-12 |  |
| Source(s) | Lei Zhou, Fangmin Xu, Xufeng Zheng, Chuan Zhong Beijing Samsung Telecom R&D Center Sangheon Kim, Brian Shim, Naehyun Lim, Hokyu Choi, Heewon Kang Samsung Electronics Co., Ltd. | l.zhou@samsung.com +86-10-59253333 |
| Re: | Change Request on 802.16m Draft Amendment, IEEE P802.16m/D2 |  |
| Abstract | The contribution proposes to clarify TBD value in AAI_LBS-ADV and move it to MAC message section. |  |
| Purpose | To be discussed and adopted by TGm for 802.16m amendment working document. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed Text for a set of messages for location based service             in connect state

Lei Zhou, Fangmin Xu, Xufeng Zheng,Chuan Zhong

Beijing Samsung Telecom R&D Center

Sangheon Kim, Brian Shim, Naehyun Lim,Hokyu Choi, Heewon Kang

Samsung Electronics Co., Ltd.

- Text to change in AWD

-------------------------------  Text Start  ---------------------------------------------------

[Deleting the following text started from line# 3 Page 580 in P802.16m/D2]

15.8.2.4.1 AAI_LBS-ADV Message

An ABS that supports LBS shall use the AAI_LBS-ADV message to broadcast the LBS related configuration information. The message may be broadcast periodically without solicitation by the AMS. The periodicity of this transmission is TBD.

Table 926—AAI_LBS-ADV message format

| Name | Value | Usage |
| --- | --- | --- |
| Number of BS | TBD | Number of neighbor BSs included in this message that are identified using the BSID |
| BSID | TBD | The Base Station ID parameter in the A-MAP message of the Serving BS or Neighbor BS |
| Number of BS Index | TBD | Number of neighbor ABSs included in this message that are identified using an index to their position in the NBR-ADV message |
| Configuration change count for AAI_NBR-ADV | TBD | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to com¬press neighbor BSID |
| Neighbor BS Index | TBD | Index that corresponds to the position of the ABS in the NBR-ADV message. |
| Absolute Position (Long Format) | TBD | The transmitting ABS's coordinates. This Absolute Position (Long Format) shall only be used for the transmitting ABS and if the Absolute Position (Short Format) is not used. |
| Absolute Position (Short Format) | TBD | The transmitting ABS's coordinates. This Absolute Position (Short Format) shall only be used for the transmitting ABS and if the Absolute Position (Long Format) is not used. |
| Relative Position | TBD | This Relative Position is used to provide the absolute position of an ABS. A Neighbor ABS’s coordinates. When this Relative Position is included it provides the position of a neighbor ABS relatively to the transmitting ABS. |
| GPS Time | TBD | Information about GPS time and time accuracy. |
| Frequency Accuracy | TBD | nformation about the frequency accuracy |

[Adopting the following modified text started from line# 33 Page 93 in P802.16m/D2]

15.2.3.41 AAI_LBS-ADV Message

An ABS that supports LBS shall use the AAI_LBS-ADV message to broadcast the LBS related configuration information. The message may be broadcast periodically without solicitation by the AMS.

Table NNN—AAI_LBS-ADV message format

| Name | Value (bit) | Usage |
| --- | --- | --- |
| Number of BS | 8 | Number of neighbor BSs included in this message that are identified using the BSID |
| BSID | 16 | 16-bit LSB of unique 48-bit identifier of the Base Station ID parameter of the Serving BS or Neighbor BS. |
| Number of BS Index | 8 | Number of neighbor ABSs included in this message that are identified using an index to their position in the NBR-ADV message |
| Configuration change count for AAI_NBR-ADV | 8 | The value of Configuration Change Count in AAI_NBR-ADV message referred in order to compress neighbor BSID |
| Neighbor BS Index | 8 | Index that corresponds to the position of the ABS in the NBR-ADV message. |
| Absolute Position (Long Format) | 120 | The transmitting ABS's coordinates. This Absolute Position (Long Format) shall only be used for the transmitting ABS and if the Absolute Position (Short Format) is not used. Including parameter (Longitude, Latitude, Altitude) |
| Absolute Position (Short Format) | 64 | The transmitting ABS's coordinates. This Absolute Position (Short Format) shall only be used for the transmitting ABS and if the Absolute Position (Long Format) is not used. Including parameter (Longitude, Latitude, Altitude) |
| Relative Position | 48 | This Relative Position is used to provide the absolute position of an ABS. A Neighbor ABS’s coordinates. When this Relative Position is included it provides the position of a neighbor ABS relatively to the transmitting ABS. Including parameter (Longitude, Latitude, Altitude) |
| GPS Time | 40 | Information about GPS time and time accuracy. Including parameter(GPS time, GPS frame transmission offset, GPS time accuracy) |
| Frequency Accuracy | 8 | Information about the frequency accuracy |

-----------------------------------------  Text End ---------------------------------------------------------------