| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed text for Comment # 781 (D2 – 15.2.23) |  |
| Date Submitted | 2009-11-13 |  |
| Source(s) | Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, Jin Sam Kwak LG Electronics | Voice : +82-31-450-7828 E-mail: hj77cho@lge.com |
| Re: | IEEE 802.16-09/0057, IEEE 802.16 Working Group Letter Ballot #30a |  |
| Abstract | The contribution provides proposed text for comment #781. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m DRAFT amendment. |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposed text for Comment # 781 (D2 – 15.2.23)

Heejeong Cho, Youngsoo Yuk, Kiseon Ryu, Ronny (Yongho) Kim, JinSam Kwak

LG Electronics

- Introduction

This contribution includes the proposed text for the comment number 781 in the IEEE 802.16m DRAFT Amendment [1].

- References

[1] IEEE P802.16m/D2, “DRAFT Amendment to IEEE Standard for Local and metropolitan area networks”

- Text proposal for the 802.16m DRAFT amendment

-----------------------------------------------------------Start of the Text---------------------------------------------------------

[On the IEEE P802.16m/D2, page 259, line 28, add the following text to subsection 15.2.23:]

15.2.23 Update of S-SFH IEs

AMS shall compare values of each SCD change count in the last received P-SFH IE and the last stored P-SFH IE whenever it receives P-SFH IE.

a) If there is no difference of two SCD change counts, AMS may not decode S-SFH IE in the superframe.

b) Else, AMS shall check that change bits are toggled as the value of the difference.

1) If the number of toggled bits in the last received S-SFH SP change bitmap is the same as the difference, AMS shall update the S-SFH SP IE(s) whose bit of the S-SFH SP change bitmap is toggled. Additionally, AMS shall update SCD change count and S-SFH SP change bitmap.

2) Else, AMS shall update all S-SFH SP IEs, SCD change count and S-SFH SP change bitmap.

Updated S-SFH SP IEs shall start to apply from the superframe indicated by "Start superframe offset where SPn(n=1,2,3) information is used" in the corresponding S-SFH SP.

[On the IEEE P802.16m/D2, page 342, line 26, modify the following text to subsection 15.3.6.5.1.2:]

Table 791—S-SFH SP1 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP1 IE format () { |  |  |
| Start superframe offset where new SP1 information is used | 2 1 | 0b0 : S-SFH SP1 is applied in the current superframe. 0b1 : S-SFH SP1 is applied in the next superframe. |
| MSB of superframe number | 8 | Remaining bit of SFN except LSB of SFN in P-SFH |

[On the IEEE P802.16m/D2, page 343, line 36, modify the following text to subsection 15.3.6.5.1.2:]

Table 792—S-SFH SP2 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP2 IE format () { |  |  |
| Start superframe offset where new SP2 information is used | 2 1 | 0b0 : S-SFH SP2 is applied in the current superframe. 0b1 : S-SFH SP2 is applied in the next superframe. |
| Frame configuration index | 6 | The mapping between value of this index and frame configuration is listed in Table X, X+1,and X+2 |

[On the IEEE P802.16m/D2, page 344, line 28, modify the following text to subsection 15.3.6.5.1.2:]

Table 793—S-SFH SP3 IE format

| Syntax | Size (bit) | Notes |
| --- | --- | --- |
| S-SFH SP3 IE format () { |  |  |
| Start superframe offset where new SP3 information is used | 2 1 | 0b0 : S-SFH SP3 is applied in the current superframe. 0b1 : S-SFH SP3 is applied in the next superframe. |
| Rate of change of SP (1-3) info | 4 |  |

-----------------------------------------------------------End of the Text---------------------------------------------------------