| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposal to modify the usage of downlink pilots for measurements (15.3.5.4) |  |
| Date Submitted | 2009-11-13 |  |
| Source(s) | Isamu YOSHII Takaaki KISHIGAMI, Panasonic Corporation | Voice: + 81-46-840-5123 E-mail: yoshii.isamu@jp.panasonic.com kishigami.takaaki@jp.panasonic.com |
| Re: | P802.16m/D2 LB30a |  |
| Abstract | The contribution proposes text to modify the usage of pilots for downlink measurements and burst demodulation. |  |
| Purpose | To be discussed and adopted by TGm for the 802.16m Advanced Air Interface (Draft 2) |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

Proposal to modify the usage of downlink pilots for measurements (15.3.5.4)

Isamu YOSHII, Takaaki KISHIGAMI

Panasonic Corporation

- Introduction

We agree to add new section for the usage of downlink pilots described in C80216m-09_2569. In addition, we propose/clarify the pilot usage for MU-MIMO case in C802.16m-09_2396. We would like to propose modify the text in C80216m-09_2569 as following.

- References

[1] P802.16m DRAFT Amendment to IEEE Standard for Local and metropolitan area networks, P80216m/D2, Oct 2009.

- Text proposal for inclusion in the 802.16m AWD

< Insert the new section shown below on page 312 line 31, after the section on MIMO midamble >

Remove text from 09/2569

- Add new text to 09/2569

-------------------------------  Start of text proposal #1 --------------------------------------------

15.3.5.4.3 Usage of Downlink Pilots

The demodulation pilots in a given PRU on a given pilot stream shall be precoded the same way as the data transmitted on the same stream in that PRU. In DLRU the data transmitted in a given PRU on a given stream may be sent to several AMSs but in different tones using the same precoder. In CLRU the data transmitted in in a given PRU on a given stream shall be sent to only one AMS.

Two pilot streams shall always be transmitted across all the PRUs in the DLRUs, whether inside or outside the open-loop region type 0, and whether or not data is being transmitted by the ABS in all DLRUs.

If no data is transmitted by the ABS on all or some CLRUs in the open-loop region type 1 or type 2, the MaxMt pilots shall still be transmitted across all CLRUs in that open-loop region. If no data is transmitted by the ABS on all or some CLRUs outside any open-loop region, the pilots mayshall be omitted on the CLRUs where no data is sent.

The precoder may be adaptive (user-specific) or non-adaptive (non user-specific) depending on the DL MIMO mode. Non-adaptive precoders are determined according to the DL MIMO mode, the number of streams, the type of LRU, operation inside or outside the open-loop region, and the physical index of the subband or miniband where the precoder is applied.

In MU-MIMO transmissions in CLRU each pilot stream is dedicated to one AMS. The AMS shouldmay use its dedicated pilot stream for channel estimation within each PRU in the allocation. Other pilot streams may be used for intracell interference estimation. The total number of streams in the transmission and the index of the dedicated pilot stream are indicated in the DL Basic Assignment A-MAP IE and DL Persistent A-MAP IE. In persistent allocation using MU-MIMO operation, the pilots of deallocated AMS's stream shall be omitted on the CLRUs outside the OL region when PA deallocation timing is different between co-scheduled AMSs.

Channel estimation for demodulation of data burst at AMS should be performed as follows:

- In DLRU: the 2 streams non-adaptively precoded common pilots across the DLRU should be used for channel estimation by all AMSs allocated a burst in the DLRU. All pilots are shared by all AMSs for demodulation in DLRU.

- In CLRU: The AMS should use its dedicated pilot streams for channel estimation within each PRU in the allocation. Pilots are not shared by AMSs for demodulation in CLRU, whether they are non-adaptively or adaptively precoded.

CQI measurements at the AMS should be performed as follows:

- For CQI reports requested with a MIMO feedback mode for operation in an open-loop region, measurements should be taken on the MaxMt-streams non-adaptively precoded pilots in that open-loop region. All pilots are shared by all AMSs for CQI measurements in each open-loop region.

- For CQI reports requested with a MIMO feedback mode for operation outside the open-loop region, measurements shall be taken on the downlink MIMO midamble.

  - For reports requested with a MIMO feedback mode for open-loop MIMO operation, the AMS should adjust the non-precoded MIMO channel estimated from the midamble with the non-adaptive precoder according to the MFM, the subband index and assumption on STC rate.

  - For reports requested with a MIMO feedback mode for closed-loop MIMO operation, the AMS should adjust the non-precoded MIMO channel estimated from the midamble by making an assumption on the adaptive precoder.

-------------------------------  End of text proposal #1 --------------------------------------------