| Project | IEEE 802.16 Broadband Wireless Access Working Group <http://ieee802.org/16> |  |
| --- | --- | --- |
| Title | Proposed Text on Power Control Section for the IEEE 802.16m/D2 Amendment (15.3.9.4) |  |
| Date Submitted | 2009-11-18 |  |
| Source(s) | Rongzhen Yang, Hujun Yin, Yang-seok Choi, Apostolos Papathanassiou, Amir Rubin Intel Corp. Wilson Tim Motorola Inc. Kiran Kuchi CeWiT Yan-Xiu Zheng ITRI Yih-Shen Chen MediaTek Huiying Fang ZTE Corporation | Email : rongzhen.yang@intel.com E-mail: Tim.Wilson@motorola.com E-mail: kkuchi@cewit.org.in E-mail: zhengyanxiu@itri.org.tw E-mail: yihshen.chen@mediatek.com E-mail: fang.huiying@zte.com.cn |
| Re: | Comments on P802.16m/D2 |  |
| Abstract | The contribution proposes the text on Power Control |  |
| Purpose | To be discussed and adopted into P802.16m/D2 by WG LB |  |
| Notice | This document does not represent the agreed views of the IEEE 802.16 Working Group or any of its subgroups. It represents only the views of the participants listed in the “Source(s)” field above. It is offered as a basis for discussion. It is not binding on the contributor(s), who reserve(s) the right to add, amend or withdraw material contained herein. |  |
| Release | The contributor grants a free, irrevocable license to the IEEE to incorporate material contained in this contribution, and any modifications thereof, in the creation of an IEEE Standards publication; to copyright in the IEEE’s name any IEEE Standards publication even though it may include portions of this contribution; and at the IEEE’s sole discretion to permit others to reproduce in whole or in part the resulting IEEE Standards publication. The contributor also acknowledges and accepts that this contribution may be made public by IEEE 802.16. |  |
| Patent Policy | The contributor is familiar with the IEEE-SA Patent Policy and Procedures: <http://standards.ieee.org/guides/bylaws/sect6-7.html#6> and <http://standards.ieee.org/guides/opman/sect6.html#6.3>. Further information is located at <http://standards.ieee.org/board/pat/pat-material.html> and <http://standards.ieee.org/board/pat>. |  |

# Proposed Text on Power Control Section for the IEEE 802.16mD2 Amendment

Rongzhen Yang, Hujun Yin, Yang-seok Choi, Apostolos Papathanassiou

Intel Corp.

Wilson Tim

Motorola Inc.

Kiran Kuchi

CeWiT

Yan-Xiu Zheng

ITRI

Yih-Shen Chen

MediaTek

Huiying Fang

ZTE Corporation

# Introduction

This contribution includes the proposed text on the power control section for the IEEE 802.16m/D2 Amendment.

# References

[1] IEEE P802.16m/D2, “Draft Amendment to IEEE Standard for Local and metropolitan area network: Air Interface for Fixed and Mobile Broadband Wireless Access Systems”, Oct. 2009.

# Text proposal for the 802.16m amendment

Instruction to editor;

Black text: existing D2 text

Blue and strike out text: for deletion

Red and underline text: for addition

============================== Start of Proposed Text =================================

# 15.3.9.4 Uplink Power Control

Uplink power control is supported for both an initial calibration and periodic adjustment on transmit power without loss of data. The uplink power control algorithm determines the transmission power of an OFDM symbol to compensate for the pathloss, shadowing and fast fading. Uplink power control shall intend to control inter-cell interference level.

A transmitting AMS shall maintain the same transmitted power density, unless the maximum power level is reached. In other words, when the number of active LRU allocated to a user is reduced, the total transmitted power shall be reduced proportionally by the AMS as long as there is no additional change of parameters for power control. When the number of LRU is increased, the total transmitted power shall also be increased proportionally. However, the transmitted power level shall not exceed the maximum levels dictated by signal integrity considerations and regulatory requirements. In closed-loop power control, the AMS shall interpret PC-A-MAP IE as the required changes to the transmitted power density.

For interference level control, current interference level of each ABS may be shared among ABSs.

## 15.3.9.1 UL Open-Loop Power Control

When the open-loop power control is used, the The power per subcarrier and per transmission antenna stream shall be maintained for the UL transmission as indicated in Equation (272).

(272)

P(dBm)=L+SINRTarget+NI+Offset					(272)

Where:

is the target uplink SINR received by the ABS. The mode used to calculate this value is signaled through a power control message.

P is the TX power level (dBm) per stream and per subcarrier for the current transmission.

L is the estimated average current UDL propagation loss calculated by AMS. It shall include AMS’s Tx antenna gain and path loss.

NI is the estimated average power level (dBm) of the noise and interference per subcarrier at the ABS, not including ABS’s Rx antenna gain, which is transmitted via AAI_ULPC_NI message

is a correction term for AMS-specific power offset. It is controlled by the AMS. Its initial value is zero. (TBD)

Offset is a correction term for AMS-specific power offset. It is controlled by the ABS through power control messages.

The estimated average current UDL propagation loss, L, shall be calculated based on the total power received on the active subcarriers of the frame preamble.

15.3.9.4. 1 UL Open-Loop Power Control

15.3.9.4.1.1 UL Open-Loop Power Control for Data Channel

The Offset_ABSperAMS can be updated by ABS. Once AMS recognized this parameter, AMS should increase and decrease the Offset_ABSperAMS according to this value sent by ABS.

When AMS connects to network, it can negotiate the OLPC mode and AMS receives the broadcasted parameters used in Equation (273):

SINRTarget=10log10 max10(SINRMIN(dB)10), γIoT×SINRDL−α  −β×10log10(TNS), OLPC Mode 1CN, OLPC Mode 2

SINRTarget=10log10max10(SINRMIN(dB)10), γIoT×SIRDL−α−β×10log10(TNS) 				(273)

Where

C/N is the normalized C/N of the modulation/FEC rate for the current transmission, as appearing in Table 787.

OLPC Mode 1 Equation(273) is the target SINR value for IoT control and tradeoff between overall system throughput and cell edge performance, decided by the control parameter  and . Each parameter used in OLPC mode1equation(273) is explained as follows:

Where

is the SINR requirement for the minimum rate expected by ABS which is set by a unicast power control AAI_SCD message. has 4 bits to represent the value in dB among{,-3, -2.5, -2, -1.5, -1, -0.5, 0, 0.5, 1, 1.5, 2, 2.5, 3, 3.5}

is the fairness and IoT control factor, broadcast by the ABS. It has 4 bits to represent the value among {0, 0.1, 0.2, 0.3, 0.4, 0.5, 0.6, 0.7, 0.8, 0.9, 1.0, 1.1, 1.2, 1.3, 1.4, 1.5}. It can also be different for each frequency partition. Details on signaling of  is found in section 15.2.3.2815.2.x.2.

SIRDLis the linear ratio of the downlink signal to interference power, measured by the AMS.

is the factor according to the number of receive antennas at the ABS. It is signaled from MAC power control mode signaling AAI_SCD message by 3 bits to express {1, 1/2, 1/4, 1/8, 1/16, 0, reserved, reserved}.

is set to be zero or one by one bit of MAC power control mode signaling AAI_SCD message.

TNS is the Total Number of Streams in the LRU indicated by UL A-MAP IE. In case of SU-MIMO, this value shall be set to Mt where Mt is the number of streams for AMS. In case of CSM, TNS is the aggregated number of streams. In case of control channel transmission, this value shall be set to one.

For a data channel transmission, the Offset in Equation 272 shall be set to the value OffsetData conveyed in an AAI UL_POWER_ADJUST message[TBD]. The value used shall be taken from the most recent message which preceeded the first uplink subframe of the frame by more than Tproc.

15.3.9.4.1.2 UL Open-Loop Power Control for Control Channel

In case of control channel, except for initial ranging and sounding, transmission, SINRtarget is same as values in Table 896

Table 896—Normalized C/N SINRtarget per modulation

| Type | Modulation/FEC rate | Required C/NSINRtarget |
| --- | --- | --- |
| Control Channel | ACK/NAK |  |
|  | CQI |  |
|  | Ranging code |  |
|  | P-FBCH |  |
|  | S-FBCH |  |
|  | Analog Feedback |  |
|  | Bandwidth Request |  |
|  | Sounding |  |
| Rank 1 MCS (Index) | ‘0000’ |  |
|  | ‘0001’ |  |
|  | ‘0010’ |  |
|  | ‘0011’ |  |
|  | ‘0100’ |  |
|  | ‘0101’ |  |
|  | ‘0110’ |  |
|  | ‘0111’ |  |
|  | ‘1000’ |  |
|  | ‘1001’ |  |
|  | ‘1010’ |  |
|  | ‘1011’ |  |
|  | ‘1100’ |  |
|  | ‘1101’ |  |
|  | ‘1110’ |  |
|  | ‘1111’ |  |
| Rank 2 MCS (Index) | ‘0000’ |  |
|  | ‘0001’ |  |
|  | ‘0010’ |  |
|  | ‘0011’ |  |
|  | ‘0100’ |  |
|  | ‘0101’ |  |
|  | ‘0110’ |  |
|  | ‘0111’ |  |
|  | ‘1000’ |  |
|  | ‘1001’ |  |
|  | ‘1010’ |  |
|  | ‘1011’ |  |
|  | ‘1100’ |  |
|  | ‘1101’ |  |
|  | ‘1110’ |  |
|  | ‘1111’ |  |
| (TBD) | (TBD) | (TBD) |

For a control channel transmission, the Offset in Equation 272 shall be set to the value OffsetControl conveyed in an AAI UL_POWER_ADJUST message[TBD]. The value used shall be taken from the most recent message which preceeded the first uplink subframe of the frame by more than Tproc.

15.3.9.4.2 UL Closed-Loop Power Control

In FDD system, tTo maintain at the ABS a power density consistent with the modulation and FEC rate used by each AMS, the ABS may change the AMS’s TX power through direct power adjustment signaling such as by PC-A-MAP. Closed loop power control is defined in Equation (274).

Offset = Offset(last) + ΔPowerAdjust                                                (274)

Where,

Offset is a vector includes 2 elements: OffsetData and OffsetControl, which will be applied to Equation (272) for AMS TX power calculation.

Offset(last) is the latest  Offsetvector value before applying the power correction values indicated by ABS through PC-A-MAP.

ΔPowerAdjust is the power correction value indicated by ABS through PC-A-MAP.

The  Offsetvector will be reset by AAI_UL_POWER_ADJUST message[TBD].  Offsetvector reset by AAI_UL_POWER_ADJUST message or updated by equation (275) is done at the start point of uplink frames that is longer than the data burst processing time Tproc. In here, Tproc is 3 subframes.

Ptx=Plast+ΔSINR+ΔpowerAdjust										(274)

Where

Ptx is transmit powe level per subcarrier

Plast is the latest transmitted maximum power level among different uplink physical channels transmitted concurrently.

ΔSINR is the difference of the desired SINRs between the previous and new MCS levels for a uplink physical channel. Desired SINR for each MCS level is TBD.

ΔpowerAdjust is the value indicated by PC-A-MAP.

Power correction values are defined in Table 897.

Table 897– Power correction offset values

| Power Correction Value | Offset (dB) |
| --- | --- |
| 0b00 | -0.5 |
| 0b01 | 0.0 |
| 0b10 | 0.5 |
| 0b11 | 1.0 |

============================== End of Proposed Text =================================